3GPP TSG-SA1 Meeting #94e	S1-211164r1

Electronic Meeting, 10 - 20 May  2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0522 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KPI for 5G system with HAPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | HAPS |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As for ground operators, HAPS deployment is a good and easy way to extend the terrestrial 5G coverage, especially in the disaster relief and high data rate coverage in remote area scenarios. RAN is working on the solutions to support HAPS and UE devices in FR1 in Rel-16. However,the KPI performance of HAPS is lacked in 3GPP.It is proposed to add the KPI requirements in TS 22.261 to support HAPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add KPI requirements support for 5G systems with HAPS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3GPP does not consider KPI requirements for 5G systems with HAPS scenario yet. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, new 7.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************************the first change**************************************

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G enhanced positioning area: a subset of the 5G positioning service area that is assumed to be provided with additional infrastructure or deploy a particular set of positioning technologies to enhance positioning services.

NOTE 1: 	The enhanced positioning service area represents for example a factory plant, a dense urban area, an area along a road or railway track, a tunnel and covers both indoor and outdoor environments.

5G LAN-type service: a service over the 5G system offering private communication using IP and/or non-, i.e. UEs that are members of the same 5G LAN-VN IP type communications.

5G LAN-virtual network: a virtual network capable of supporting 5G LAN-type service.

5G satellite access network: 5G access network using at least one satellite.

5G positioning service area: a service area where positioning services would solely rely on infrastructures and positioning technologies that can be assumed to be present anywhere where 5G is present (e.g. a country-wide operator-supplied 5G network, GNSS, position/motion sensors).

NOTE 2: 	This includes both indoor and any outdoor environments.

active communication: a UE is in active communication when it has one or more connections established. A UE may have any combination of PS connections (e.g. PDP contexts, active PDN connections).

activity factor: percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network.

area traffic capacity: total traffic throughput served per geographic area.

communication service availability: percentage value of the amount of time the end-to-end communication service is delivered according to an agreed QoS, divided by the amount of time the system is expected to deliver the end-to-end service according to the specification in a specific area.

NOTE 3: The end point in "end-to-end" is assumed to be the communication service interface.

NOTE 4: The communication service is considered unavailable if it does not meet the pertinent QoS requirements. If availability is one of these requirements, the following rule applies: the system is considered unavailable if an expected message is not received within a specified time, which, at minimum, is the sum of maximum allowed end-to-end latency and survival time.

direct device connection: the connection between two UEs without any network entity in the middle.

direct network connection: one mode of network connection, where there is no relay UE between a UE and the 5G network.

Disaster Condition: This is the condition that a government decides when to initiate and terminate, e.g. a natural disaster. When this condition applies, users may have the opportunity to mitigate service interruptions and failures.

Disaster Inbound Roamer: A user that (a) cannot get service from the PLMN it would normally be served by, due to failure of service during a Disaster Condition, and (b) is able to register with other PLMNs.

Disaster Roaming: This is the special roaming policy that applies during a Disaster Condition.

end-to-end latency: the time that takes to transfer a given piece of information from a source to a destination, measured at the communication interface, from the moment it is transmitted by the source to the moment it is successfully received at the destination.

Hosted Service: a service containing the operator's own application(s) and/or trusted third-party application(s) in the Service Hosting Environment, which can be accessed by the user.

indirect network connection: one mode of network connection, where there is a relay UE between a UE and the 5G network.

IoT device: a type of UE which is dedicated for a set of specific use cases or services and which is allowed to make use of certain features restricted to this type of UEs.

NOTE 5: An IoT device may be optimized for the specific needs of services and application being executed (e.g. smart home/city, smart utilities, e-Health and smart wearables). Some IoT devices are not intended for human type communications.

network slice: a set of network functions and corresponding resources necessary to provide the required telecommunication services and network capabilities.

NG-RAN: a radio access network connecting to the 5G core network which uses NR, E-UTRA, or both.

non-public network: a network that is intended for non-public use.

NR: the new 5G radio access technology.

positioning service availability: percentage value of the amount of time the positioning service is delivering the required position-related data within the performance requirements, divided by the amount of time the system is expected to deliver the positioning service according to the specification in the targeted service area.

positioning service latency: time elapsed between the event that triggers the determination of the position-related data and the availability of the position-related data at the system interface.

priority service: a service that requires priority treatment based on regional/national or operator policies.

private communication: a communication between two or more UEs belonging to a restricted set of UEs.

private network: an isolated network deployment that does not interact with a public network.

private slice: a dedicated network slice deployment for the sole use by a specific third-party.

relative positioning: relative positioning is to estimate position relatively to other network elements or relatively to other UEs.

reliability: in the context of network layer packet transmissions, percentage value of the amount of sent network layer packets successfully delivered to a given system entity within the time constraint required by the targeted service, divided by the total number of sent network layer packets.

satellite: a space-borne vehicle embarking a bent pipe payload or a regenerative payload telecommunication transmitter, placed into Low-Earth Orbit (LEO) typically at an altitude between 300 km to 2 000 km, Medium-Earth Orbit (MEO) typically at an altitude between 8 000 to 20 000 k m, or Geostationary satellite Earth Orbit (GEO) at 35 786 km altitude.

HAPs: an airborne vehicle operating in altitudes typically between 20 and 50 km including High Altitude Platforms

satellite access: direct connectivity between the UE and the satellite.

satellite NG-RAN: a NG-RAN which uses NR in providing satellite access to UEs.

NOTE 6: The service area can be indoors.

NOTE 7: For some deployments, e.g. in process industry, the vertical dimension of the service area can be considerable.

service continuity: the uninterrupted user experience of a service that is using an active communication when a UE undergoes an access change without, as far as possible, the user noticing the change.

NOTE 8: 	In particular service continuity encompasses the possibility that after a change the user experience is maintained by a different telecommunication service (e.g. tele- or bearer service) than before the change.

NOTE 9: 	Examples of access changes include the following. For EPS: CS/PS domain change. For EPS and 5G: radio access change, switching between a direct network connection and an indirect network connection.

Service Hosting Environment: the environment, located inside of 5G network and fully controlled by the operator, where Hosted Services are offered from.

survival time: the time that an application consuming a communication service may continue without an anticipated message.

Time to First Fix (TTFF): time elapsed between the event triggering for the first time the determination of the position-related data and the availability of the position-related data at the positioning system interface.

User Equipment: An equipment that allows a user access to network services via 3GPP and/or non-3GPP accesses.

user experienced data rate: the minimum data rate required to achieve a sufficient quality experience, with the exception of scenario for broadcast like services where the given value is the maximum that is needed.

wireless backhaul: a link which provides an interconnection between 5G network nodes and/or transport network using 5G radio access technology.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3D	Three Dimensional

5G	Fifth Generation

5G LAN-VN	5G LAN-Virtual Network

AR	Augmented Reality

A/S	Actuator/Sensor

E2E	End to End

eFMSS	Enhancement to Flexible Mobile Service Steering

eV2X	Enhanced V2X

FMSS	Flexible Mobile Service Steering

GEO	Geostationary satellite Earth Orbit

HAPS	High Attitude Platforms

ICP	Internet Content Provider

ID	Identification

IMU	Inertial Measurement Unit

IOPS	Isolated E-UTRAN Operation for Public Safety

IoT	Internet of Things

KPI	Key Performance Indicator

LCS	Location Services

LEO	Low-Earth Orbit

MBB	Mobile Broadband

MCS	Mission Critical Services

MCX	Mission Critical X, with X = PTT or X = Video or X = Data

MEO	Medium-Earth Orbit

MIoT	Massive Internet of Things

MMTEL	Multimedia Telephony

MNO	Mobile Network Operator

MPS	Multimedia Priority Service

MSGin5G	Message Service Within the 5G System

MVNO	Mobile Virtual Network Operator

NGMN	Next Generation Mobile Networks

NPN	Non-Public Network

QoE	Quality of Experience

RSTP	Rapid Spanning Tree Protocol

SEES	Service Exposure and Enablement S

URLLC	Ultra Reliable Low Latency Communication Support

SST	Slice/Service Type

TTFF	Time To First Fix

UAV	Unmanned Aerial Vehicle

UHD	Ultra High Definition

VR	Virtual Reality

**************************************end of the first change*************************************

****************************************the second change***************************************

7.X	KPIs for a 5G system with HAPS

7.X.1	Description

5G system can use infrastructures integrated on a minimum of gNB or relay placed in HAPS.

HAPS typically operates at the altitude of 20km and the maximal coverage distance of the HAPS is 229km at 5° elevation angle. For the UE to the HAPS path, the maximal one-way propagation delay associated with HAPS is 1.53ms. The differential delay between nadir and Edge of Coverage is 0.7ms.

7.X.2	Requirements

The 5G system with HAPS shall support high uplink and downlink data rates and for 5G UEs and IoT services.

Table 7.X.2-1: Performance requirements for HAPS

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pedestrian for wide area (note 2) | 1 Mbit/s | 100Kbit/s | [20M]bit/s/km2 | [2M]bit/s/km2 | [100]/km2 | 20 % | Pedestrian | Smart phones |
| remote area (note 3) | 50 Mbit/s | 25 Mbit/s | 100Mbit/s/km2 | 50Mbit/s/km2 | 10/km2 | 20% | Pedestrian | Smart phones |
| Vehicular connectivity (note 4) | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | 50 % | Up to 250 km/h | Smart phones |
|  |  |  |  |  |  |  |  | Vehicle mounted |
| Stationary (note 5) | 100 Mbit/s | 50 Mbit/s | TBD | TBD | TBD | N/A | Stationary | Building mounted |
| High Speed train (note 6) | 750Mbit/s | 375Mbit/s | - | - | 2/train | N/A | up to 500 km/h | Train mounted |
| Wide area IoT connectivity （note 7） | - | <1Mbps | TBD | TBD | 10/km2 to 1000/km2 | TBD | TBD | IoT UE |
| Note 1: Area capacity is averaged over a HAPS beam. Note 2: Data rates based on Extreme long-range coverage target values in clause 6.17.2. User density and activity based on rural area in Table 7.1-1. Note 3: Data rates based on rural area in Table 7.1-1. User density based on the assumption of a typical remote area Note 4: Based on Table 7.1-1. Note 5: Based on an assumption of 10 users per building 50/25 Mbit/s data rate and 20 % activity factor per user Note 6: Based on an assumption of 1000 users per train 15/7.5 Mbit/s data rate and 10 % activity factor per user Note 7: Based on Table 7.5.2-1. Note 8: All the values in this table are targeted values and not strict requirements. Note 9: Performance requirements for all the values in this table should be analysed independently for each scenario. |  |  |  |  |  |  |  |  |

**************************************end of the second change*************************************

****************************************the third change***************************************

Annex X (informative):

The delay performance of HAPS

Based on the assumption of 20 km altitude for HAPS, the maximal one-way propagation delay associated with HAPS is 1.53ms at 5° elevation angle. The differential delay between nadir and Edge of Coverage is 0.7ms.

Table X. 1-1: UE to HAPS delay

| elevation | UE to HAPS propagation Delay [ms] | One-way propagation Delay |
| --- | --- | --- |
| 90° | 0.067 | 0.134 |
| 5°(note 1) | 0.763 | 1.526 |

Note 1: At 5° elevation angle the distance to the aerial vehicle is 229 km, and the Gateway is supposed to be at same distance.

**************************************end of the third change*************************************