3GPP TSG-SA WG1 Meeting #94e	  S1-211170r1

Electronic Meeting, 10 May – 20 May 2021                     (revision of S1-20xxxx)

Title:		Update to the use case of Protection of DER and grid interconnection

Agenda Item:	7.9.1 (FS_5GSEI)

Source:	Huawei, Orange, China Telecom, CEPRI, EDF, Novamint

Contact:	Shuang ZHANG (zhang.shuang2@huawei.com)

Abstract: EN is removed and details are added to use case in clause 5.16 of TR 22.867.

Acknowledgment: This contribution was endorsed by EU DSO Stedin and EU TSO TenneT.

Discussion

In response to the comments received from the SA1 #93e meeting, the service flow is updated to remove the EN accompanying [PR 5.16.6-1]. To improve the readability, this requirement is broken down into three, namely [PR 5.16.6-1], [PR 5.16.6-2], [PR 5.16.6-3].

Introduction and motivation of changes to this updated use case follow in the remainder of the discussion section.

Proposal

The following changes are proposed for clause 5.16 of TR 22.867.

Begin First Change

### 5.16.3	Service Flows

Company U operates a large national grid. Thanks to U’s existing IP/MPLS infrastructure, PMUs deployed in substations belonging to transmission grid are connected to this IP/MPLS network. The number of PMUs can range from a few hundred to tens of thousands. Each individual PMU continually streams measurement data to a number of PDCs located in control centers at different levels for monitoring and analysis performed by different teams (or analysis by IEDs with eg. protection functions). The network engineer Arjen configures in each IP/MPLS router, so that a dedicated VPN is used for transmitting R-SV datagrams. Then, Arjen assigns IP addresses to PMUs from the IP/MPLS network. Due to the high number of PMUs, Arjen configures the IP/MPLS network to efficiently transfer the PMU data to destination PDCs. In the meantime, to improve security, Arjen uses the source specific multicast (SSM) feature to ensure the IP/MPLS network only forwards datagrams to receiving PDCs from only the source PMUs to which the receiving PDCs have explicitly joined. The multicast trees are static once the PMU application is up and running. Worth mentioning is that for each PMU, its corresponding multicast tree is built based on the WAN smart grid protection application logics. In other words, the multicast tree reflects where in the smart grid the R-SV measurement data from a particular PMU is needed.

With the renewables booming, the distribution grid of company U is connected to an increasing amount of PV parks and wind parks. At and around the interconnections, gradually more new PMUs are deployed. It is impractical to extend the IP/MPLS network physically to all these locations. Therefore, company U decides to make use of the 5G service from an operator KTT.

Now, for U’s DERs and micro grids, Arjen again refers to the WAN smart grid protection application logics to configure the newly installed PMUs. The purpose is that these PMUs will virtually take part in the existing multicast trees in U’s IP/MPLS network. Thanks to KTT’s 5G_LAN service, these PMUs and control centre IP gateways are added to the VPN that Arjen previously configured in the IP/MPLS network. See as the extension of U’s IP/MPLS network footprint, 5G delivers the PMUs’ datagrams conforming the source-specific multicast a priori.

As a result, 5G network provides communication among the PMUs with PDCs in the control centres. A PMU produces a single source of R-SV measurements, and the 5G network delivers the measurements efficiently to the intended group of recipients.

Suddenly an outage of a distributed generator at a given site causes generation load imbalance (site 1). Similar conditions happen to another distributed generators connected nearby (site 2).

PMUs measure synchrophasors, frequency and ROCOF associated with the fundamental/primary components at various interconnections with DERs.

PMUs multicast measurement data to the subscribers, which could be other PMUs and the WAMPAC.

At abnormal site 1, based on the PMU’s locally measured ROCOF, the protection function decides the problem to be a weak load imbalance. So the protection will not be triggered since the frequency is supposed to restore after some time. At other abnormal sites 2, similar decisions are made by the local protection functions.

However, the WAMPAC receives measurement from all of these PMUs. Based on its global view, it detects that area with prevalence of under frequency conditions spread among a number of DERs could potentially cause severe impact on the overall distribution grid. The WAMPAC decides to trip the generators in some of these problematic sites (treating the under voltage condition as severe imbalance), so as to prevent system collapse.

Begin Second Change

### 5.16.6	Potential New Requirements needed to support the use case

[PR 5.16.6-1] The 5G system shall be able to deliver data originated by a UE to a group of recipient UEs distributed over a large geographical area

[PR 5.16.6-2] The 5G system shall allow the originating UE to make use of several groups at the same time

[PR 5.16.6-3] The 5G system shall enable recipient UEs to indicate their interest in receiving data from several originating UE groups at the same time, with a specific source indicated per UE group.

End Second Change