3GPP TSG-SA WG1 Meeting #94e 	S1-211187r3

Electronic Meeting, 10 – 20 May 2021	(revision of S1-21xxxx)

Title:	FS_TACMM: Haptic feedback for a personal exclusion zone

Agenda Item:	7.16.1

Source:	InterDigital

Contact:	Atle Monrad <atle dot monrad at interdigital dot com>

Discussion

This document proposes a new use case for the study item FS_TACMM.

Proposal

It is proposed to include the below new use case in 3GPP TR 22.847 v0.1.0.

* * * *   First Change   * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-T, “Technology Watch Report: The Tactile Internet”, August 2014.

[3]	O. Holland et al., “The IEEE 1918.1 “Tactile Internet” Standards Working Group and its Standards,” Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[4]	3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications".

[5]	3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications".

[6] 	3GPP TS 22.261: " Service requirements for the 5G system ".

[7] 	Kwang Soon Kim, et al., " Ultrareliable and Low-Latency Communication Techniques for Tactile Internet Services", PROCEEDINGS OF THE IEEE, Vol. 107, No. 2, February 2019

[8]	"SAE J3186: Application Protocol and Requirements for Maneuver Sharing and Coordinating Service", version 0.24, developing document, Feb. 2021.

[9] 	SAE J3224, "V2X Sensor-Sharing for Cooperative & Automated Driving" version 0.05, developing document, Feb. 2021.

[10] 	M. During and K. Lemmer, “Cooperative maneuver planning for cooperative driving,” IEEE Intell. Transp. Syst. Mag., vol. 8, no. 3, pp. 8–22, Jul. 2016.

[11]	D. Soldani, Y. Guo, B. Barani, P. Mogensen, I. Chih-Lin, S. Das, "5G for ultra-reliable low-latency communications". IEEE Network. 2018 Apr 2;32(2):6-7.

[12]	O. Dubuisson and P. Fouquart, "ASN.1: Communication between Heterogeneous Systems", Morgan Kaufmann Publishers Inc., San Francisco, CA, USA, 2001.

* * * *   Next Change   * * * *

## 5.X	Haptic feedback for a personal exclusion zone in dangerous remote environments

### 5.X.1	Description

With the assistance of 5G networks, many industries including mining, operate unmanned and automated. In mining scenarios, drilling safety and precise control for automated rigs and sending effective alarms when needed to the onsite crew is vital for their safety. When crew move heavy lifting equipment wearing personal protection equipment (PPE) for their safety or work in a noisy/poor visibility environment, audio/light alarm systems coverage may not be detected for immediate reaction therefore use of wearables (belts, shoe sole, arm/shoulder tactile equipment) can improve the reliability of alarm system.

Furthermore, the nature of human brain response time to light and audio makes the use of haptic feedback to alert faster and more reliable. Human brain response to the sense of touch in range of 1 ms where the response to audio and video is in 100s of milliseconds. Therefore, the alarm system can be enriched with additional haptic information and multimodality session can be relayed to the on-site crew over to accelerate human response time and improve the system reliability.

While haptic alarm can be actuated on an extended PPE haptic device (e.g., belt) to improve alerting the on-site crew, the audio siren and light evacuation signals (where possible) need to be sent, as an alert to ambient actuators requiring a multimodal information transfer via separate service data flows, to the proximity devices during an emergency for directing them to a safe location.

### 5.X.2	Pre-conditions

In a large mining scenario, a hazard scenario is detected by the remote control unit to notif and navigate on-site workers by a multi modal alarm system to avoid exclusion zones. The multi modal alarm system monitors the environment using surveillance cameras fixed in the local site, on drones, or on workers helmets as well as haptic information relayed by the workers. Ambient sensory information (smoke, temperature, audio/video) can be relayed to a remote central monitoring and control unit to predict/detect hazard scenarios.

The personal exclusion zone needs to be defined to prohibit the entry of the on-site workers which can change over time. The detected exclusion zone information from remote control unit will actuate devices (e.g., siren and light) fixed in the local site or in drones as well PPE haptic belt actuator to navigate the workers.

Multi-modal device gateways interconnect both remote and local sites and the sensors and actuators can be connected to the 5G network, run by a single operator, through one or more linked subscriptions, possible through a common PDU Session obviating the need for multiple PDU Sessions. Devices can be connected to the 5G network either directly or through a site gateway. The gateway at the local site may provide connectivity to PPE/actuators/UEs of the onsite crew (either over same 3GPP and non-3GPPP Access Network or multiple Access Networks), e.g., alarm/surveillance camera devices through wired connectivity and haptic alert belt/glove wearable devices through wireless connectivity. At the remote site, a gateway may connect to monitoring equipment as well as the control and navigation units.

The multimodal information delivered over one common session to different UEs includes different types of information. A multicast approach can be used to transmit all types of data to all terminals by the multimodal gateway, (i.e., when relaying data common to all terminals). When transmitting specific data to specific terminals (depending on terminal capability) a unicast approach can be used, e.g., the haptic information to be relayed to the haptic glove or belt to navigate the worker and audio to be relayed to the headset worn by the same user.

A haptic alert sensed by the workers in the dangerous environment can navigate them over a trained haptic language instruction (single buzz on the right side: move right, single buzz on the left side: move left, etc.) to safely leave the hazard areas.

Figure 5.x.2-1. Example of a personal exclusion zone

### 5.X.3	Service Flows

1.	If a hazard situation is detected, on-site workers need to be alerted immediately and defined personal exclusion zone(s) should be communicated with the on-site miners according to their proximity.

2.	The haptic wearable (e.g., belt) can navigate the on-site workers to move away from exclusion zone.

NOTE:	As the multimodal information delivered over an associated distributed session to different UEs carry different type of information, the alert system information varies depending on the location of the on-site workers and the environment logistics and availability of the devices.

### 5.X.4	Post-conditions

Remote monitoring and control can assess the hazard situation and the mobility pattern of the on-site workers based on the navigation instructions received regarding the exclusion zones.

### 5.X.5	Existing features partly or fully covering the use case functionality

In 3GPPTS 22.263 [5], there are requirements for supporting video, imaging, and audio for professional applications.

### 5.X.6	Potential New Requirements needed to support the use case

[PR. 5.X.6-1] The 5G network shall be able to provide a mechanism to a distributed execution of a multimodal single session over different terminals belonging to one user or different ambient terminals in the proximity of the user and in the trusted 3rd party. See key performance requirements in Table 5.x.6-1. The provided requirements apply to both UL and DL.

Table 5.x.6-1: Potential Key performance requirements for a personal exclusion zone in dangerous remote environments.

| Traffic Type | Packet Size | Reliability (%) | Latency (ms) | Average Data rate |
| --- | --- | --- | --- | --- |
| Video | 1.5 kB | 99.999 | <400 [11] | 1-100 Mbps |
| Audio | 50 B | 99.9 | <150 [11] | 5-512 kbps |
| VR | MTU | 99.9 | <300 | 600 Mbps |
| Haptic | Tactile sigs. 1 DoF: 2-8 B 10 DoFs: 20-80 B 100 DoFs: 200-800 B | 99.999 | 0-50 [11] [12] | 1-4k pkts/s (w/o compression) 100-500 pkts/s, (w/ compression.) |

Editor’s Note:	KPIs and Performance requirements for personal exclusion zone in dangerous remote environments may need further discussion and verification.

* * * *   End of Change   * * * *