3GPP TSG-SA WG1 Meeting #94e 	S1-211187r5

Electronic Meeting, 10 – 20 May 2021	(revision of S1-21xxxx)

Title:	FS_TACMM: Haptic feedback for a personal exclusion zone

Agenda Item:	7.16.1

Source:	InterDigital

Contact:	Atle Monrad <atle dot monrad at interdigital dot com>

Discussion

This document proposes a new use case for the study item FS_TACMM.

Proposal

It is proposed to include the below new use case in 3GPP TR 22.847 v0.1.0.

* * * *   First Change   * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-T, “Technology Watch Report: The Tactile Internet”, August 2014.

[3]	O. Holland et al., “The IEEE 1918.1 “Tactile Internet” Standards Working Group and its Standards,” Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[4]	3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications".

[5]	3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications".

[6] 	3GPP TS 22.261: " Service requirements for the 5G system ".

[7] 	Kwang Soon Kim, et al., " Ultrareliable and Low-Latency Communication Techniques for Tactile Internet Services", PROCEEDINGS OF THE IEEE, Vol. 107, No. 2, February 2019

[8]	"SAE J3186: Application Protocol and Requirements for Maneuver Sharing and Coordinating Service", version 0.24, developing document, Feb. 2021.

[9] 	SAE J3224, "V2X Sensor-Sharing for Cooperative & Automated Driving" version 0.05, developing document, Feb. 2021.

[10] 	M. During and K. Lemmer, “Cooperative maneuver planning for cooperative driving,” IEEE Intell. Transp. Syst. Mag., vol. 8, no. 3, pp. 8–22, Jul. 2016.

[11]	D. Soldani, Y. Guo, B. Barani, P. Mogensen, I. Chih-Lin, S. Das, "5G for ultra-reliable low-latency communications". IEEE Network. 2018 Apr 2;32(2):6-7.

[12]	O. Dubuisson and P. Fouquart, "ASN.1: Communication between Heterogeneous Systems", Morgan Kaufmann Publishers Inc., San Francisco, CA, USA, 2001.

* * * *   Next Change   * * * *

## 5.X	Haptic feedback for a personal exclusion zone in dangerous remote environments

### 5.X.1	Description

With the assistance of 5G networks, many industries including mining, operate unmanned and automated. In mining scenarios, drilling safety and precise control for automated rigs and sending effective alarms when needed to the onsite crew is vital for their safety. When crew move heavy lifting equipment wearing personal protection equipment (PPE) for their safety or work in a noisy/poor visibility environment, audio/light alarm systems coverage may not be detected for immediate reaction therefore use of wearables (belts, shoe sole, arm/shoulder tactile equipment) can improve the reliability of alarm system.

Furthermore, the nature of human brain response time to light and audio makes the use of haptic feedback to alert faster and more reliable. Human brain response to the sense of touch in range of 1 ms where the response to audio and video is in 100s of milliseconds. Therefore, the alarm system can be enriched with additional haptic information and multimodality session can be relayed to the on-site crew over to accelerate human response time and improve the system reliability.

While haptic alarm can be actuated on an extended PPE haptic device (e.g., belt) to improve alerting the on-site crew, the audio siren and light evacuation signals (where possible) need to be sent, as an alert to ambient actuators requiring a multimodal information transfer via separate service data flows, to the proximity devices during an emergency for directing them to a safe location.

As an exmple, in a large mining environment, a hazard scenario is detected by the remote control unit to notif and navigate on-site workers by a multi modal alarm system to avoid exclusion zones. The multi modal alarm system monitors the environment using surveillance cameras fixed in the local site, on drones, or on workers helmets as well as haptic information relayed by the workers. Ambient sensory information (smoke, temperature, audio/video) can be relayed to a remote central monitoring and control unit to predict/detect hazard scenarios.

The personal exclusion zone needs to be defined to prohibit the entry of the on-site workers which can change over time. The detected exclusion zone information from remote control unit will actuate devices (e.g., siren and light) fixed in the local site or in drones as well PPE haptic belt actuator to navigate the workers.

### 5.X.2	Pre-conditions

Multi-modal UEs, with sensor, haptic and audio/visual capabilities, are interconnected, through the 5G Network provided by the MNO, both locally, or remotly over the MNO 5G Network.

The MNO provides service data flows for multimodal information delivered over one common session to different UEs. E.g., a single user may receive haptic information be sent to her/his haptic glove and belt and audio to be sent to her/his headset to help navigate the worker through the exclusion zone.

Figure 5.x.2-1. Example of a personal exclusion zone

### 5.X.3	Service Flows

1.	A Worker using PPE connected to a monitoring and control unit walks through a mining site where multiple exclusion zones are defined to protect her/him from hazardous occurrences. As the worker gets closer to the exclusion zone, her/his PPE establishes a session to enable transmission of alarms to the monitoring and control unit, e.g., noise levels, visibility levels and user location

- If a hazard situation is detected, on-site workers need to be alerted immediately and defined personal exclusion zone(s) should be communicated with the on-site miners according to their proximity to the exclusion zone. Thus, the monitoring system determines that the user must be alerted, through haptic and audio alerts, since the monitoring and control unit determined that those are the modal types the user can utilized safely in the dangerous environment

- The monitoring and control system, navigates workers using both a trained haptic language instruction (single buzz on the right side: move right, single buzz on the left side: move left, etc.) set and audio/visual mechanism to enable the user to safely leave the hazard area

### 5.X.4	Post-conditions

The site worker is safely navigated away from the dangerous exclusion zone as the 5G network enables the PPE to alert the monitoring and control unit of dangerous situations which triggers the monitor and control unit to use the 5G network to transmit multi modal commands to steer away site workers from dangerous exclusion zones.

### 5.X.5	Existing features partly or fully covering the use case functionality

In 3GPP TS 22.263 [5], there are requirements for supporting video, imaging, and audio for professional applications.

In 3GPP TS 22.261 [6] there are requirements to support the following:

Clause 6.3 Multiple Access Technologies

“Based on operator policy, the 5G system shall be able to provide simultaneous data transmission via different access technologies (e.g. NR, E-UTRA, non-3GPP), to access one or more 3GPP services”

“The 5G system shall support a set of identities for a single user in order to provide a consistent set of policies and a single set of services across 3GPP and non-3GPP access type”

Clause 6.26.2.5 Traffic Types

“The 5G system shall support traffic scenarios typically found in a home setting (from sensors to video streaming, relatively low amount of UEs per group, many devices are used only occasionally) for 5G LAN-type service”

### 5.X.6	Potential New Requirements needed to support the use case

[PR. 5.X.6-1] The 5G network shall be able to provide a mechanism to route specific traffic types from multimodal applications over a single session for a user using two or more UEs with different multimodal capabilities .See key performance requirements in Table 5.x.6-1. Haptic (DL) information is communicated only from Remote Site to Local Site.

Table 5.x.6-1: Potential Key performance requirements for a personal exclusion zone in dangerous remote environments.

| Traffic Direction | Traffic Type | Packet Size | Reliability (%) | Latency (ms) | Average Data rate |
| --- | --- | --- | --- | --- | --- |
| UL/DL | Video | 1.5 kB | 99.999 | <400 [11] | 1-100 Mbps |
| UL/DL | Audio | 50 B | 99.9 | <150 [11] | 5-512 kbps |
| UL/DL | VR | MTU | 99.9 | <300 | 600 Mbps |
| DL | Haptic | Tactile sigs. 1 DoF: 2-8 B 10 DoFs: 20-80 B 100 DoFs: 200-800 B | 99.999 | 0-50 [11] [12] | 1-4k pkts/s (w/o compression) 100-500 pkts/s, (w/ compression.) |

Editor’s Note:	KPIs and Performance requirements for personal exclusion zone in dangerous remote environments may need further discussion and verification.

Editor’s Note:	The use case must be reviewed to consider aspects on time synchronization.

* * * *   End of Change   * * * *