3GPP TSG-SA WG1 Meeting #94e 	S1-211226r2

Electronic Meeting, 10 – 20 May 2021	(revision of S1-21xxxx)

Title:	Resident architecture pictures

Agenda Item:	7.11

Source:	KPN

Contact:	Toon .Norp @tno. nl

Abstract: This contribution proposes an annex to 22.858 with architecture descriptions. Normally, it is SA2 that is responsible for architecture definition. However, the nature of Resident (where new services are tied to new pieces of hardware) makes it relevant to describe architecture in the SA1 report for Resident.

++++++++++++++++++++++++Proposed changes (all new text)++++++++++++++++++++++++

# Annex X: Resident networks overview

## A.1	Introduction

The following provides an overview of the entities defined in this Technical Report and their relations.

## A.2	Customer Premises Network overview

Figure A.2-1: Customer Premises Network connected to 5GC

The Resident networks overview contains the following entities.

-	evolved Residential Gateway (eRG).

-	Premises Radio Access System (PRAS).

-	Customer Premises Network (CPN)

-	User Equipment (UE).

-	Non-3GPP Device.

## A.3	Customer Premises Network entities

### 4.3.1	evolved Residential Gateway (eRG)

The evolved Residential Gateway is defined in clause 3.1.

Connectivity between the eRG and the UE, non-3GPP Device, or PRAS can use any suitable non-3GPP technology (e.g. Ethernet, optical, WiFi)

### 4.3.2	Premises Radio Access System (PRAS)

The Premises Radio Access System is defined in clause 3.1.

A PRAS can use licensed spectrum, but also can use unlicensed spectrum.

Connectivity between the PRAS and the UE can use NR radio access (Uu) or non-3GPP radio access.

The PRAS only provides non-3GPP radio access to UEs with 3GPP credentials (similar to other non-3GPP access to the 5GC). The PRAS is not e.g. a WiFi AP that provides non-3GPP access to devices with no 3GPP credentials.

The PRAS is part of the Customer Premises Network and therefore connected to the eRG.

The PRAS may also provide E-UTRA radio access. However, this may require clarification of the relation between PRAS and HeNB.

### 4.3.3	Customer Premises Network (CPN)

The Customer Premises Network is defined in clause 3.1.

### 4.3.4	User Equipment (UE)

The User Equipment is defined in [1].

### 4.3.3	Non-3GPP device

The Non-3GPP device is defined in clause 3.1.

+++++++++++++++++++Next change++++++++++++++++++++++++++++++++++++

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Authorised Administrator: a user or other entity authorised to partially configure and manage a network node in a CPN (e.g. a PRAS, or eRG).

Customer Premises Network: a network located within a premises (e.g. a residence, office or shop), which is owned, installed and/or (at least partially) configured by the customer of a public network operator.

Evolved Residential Gateway: a gateway between the public operator network (fixed/mobile/cable) and a customer premises network within a residence, office or shop.

Non-3GPP device: a non-3GPP device is a device that uses non-3GPP access technology and does not have 3GPP credentials

Premises Radio Access Station: a base station installed at a customer premises network primarily for use within a residence, office or shop.