3GPP TSG-SA WG1 Meeting #94e 	S1-211236r1

Electronic Meeting, 10 – 20 May 2021

Title:	[DRAFT] Reply LS on selecting a PLMN not allowed in the country where a UE is physically located

Response to:	LS (C1-212600/S1-21xxxx) on selecting a PLMN not allowed in the country where a UE is physically located from CT1

Release:	Release-17

Work Item:	5GSAT_ARCH-CT

Source:	SA1

To:	CT1

Cc:	SA3LI, SA2

Contact Person:	Toon Norp

toon .norp @tno .nl

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

SA1 thanks CT1 for the LS asking the following questions (taken from C1-212600):

1.	Should a UE, with or without a USIM, attempt to obtain emergency services from a PLMN not allowed to operate in the country of the UE’s location?

2.	Should a UE, with or without a USIM, always expect to obtain emergency services from a PLMN allowed to operate in the country in the UE location?

3.	If the answer to any of the above questions is no, could SA1 provide the necessary guidance for selecting a PLMN that can provide emergency services to the UE over satellite access?

SA1 has investigated those questions and would like to provide the following answers:

SA1 response to CT1 question 1:

This depends on how the UE knows the PLMN is not allowed to operate at the UE’s location.

If a PLMN has indicated to the UE that it is not allowed to operate in the country of the UE’s location, then this is also an indication that the PLMN is unable to provide support the various requirements related to emergency services, notably routing to a suitable PSAP. Note that it cannot be assumed that any PLMN will be able to support emergency calls for any country.

However, in border areas, PLMNs and/or PSAPs may provide some support for emergency calls from across a border. This also currently happens in terrestrial networks. It should be up to the PLMN to determine whether and to what extend cross-border support is available and emergency calls can be allowed. This implies that, in case the UE itself has determined that it is in a country the PLMN is not allowed to operate in, the UE may attempt to obtain emergency services.

SA1 response to CT1 question 2:

Yes.

SA1 response to CT1 question 3:

The UE should first determine a most suitable PLMN based on its own physical location determination. As there may be border cases where the UE cannot determine sufficiently accurate or sufficiently precise in which country it is, the UE may also still attempt to obtain (emergency) services from a PLMN that according to the UE is not allowed to operate in the country of the UE’s location. If the PLMN indicates it is not allowed to operate in the country of the UE’s location, the UE should (re)-attempt emergency calls.

Furthermore, SA1 would like to draw CT1 attention to:

It may not be possible for a PLMN to determine the location of a UE without SIM. Moreover, some PLMNs do not support emergency calls for UEs without SIM.

TS 22.101 contains a requiremen for PSAP call-back: Subject to local/regional regulations the network shall support a call-back from a PSAP. SA1 understands this requirement makes it practically impossible for the 5G system by its discretion to release an emergency service session, once it’s established to PSAP.

SA1 assumes a solution to be selected could have an impact on call setup time and considers that the call setup time is to be comparable to that of a usual IMS emergency call.

2. Actions:

To CT1

ACTION: SA1 kindly asks CT1 to take the answers above into account.

3. Date of Next TSG SA WG1 Meetings:

SA1#95e	23 Aug – 2 Sep 2021 	Electronic Meeting

SA1#96*	15 –19 Nov 2021		Sophia-Antipolis, France

*or 9 – 19 Nov 2021 if e-meeting