3GPP TSG-SA WG1 Meeting #94e 	S1-211242

Electronic Meeting, 10 – 20 May 2021	(revision of S1-21xxxx)

Title:	Update of Use case 5.17, on relay sharing

Agenda Item:	7.14.1

Source:	Qualcomm

Contact:	Francesco Pica
fpica<at>qti.qualcomm.com

Abstract: This document proposes updates to the use case 5.17, on relays sharing, captured in TR 22.839 (v.020). Main changes are to address ENs and comments on clarifying some of the terminology and sharing scenarios.

## ============ P-CR (with track changes) ===============

## 5.17	Vehicle Relay sharing

### 5.17.1	Description

A nationwide company, 5G-travels, provides long-distance bus transport service and has recently installed 5G mobile BS relays on their bus fleet, for enhanced 5G connectivity to their passengers. Two national cellular MNOs, PLMN-A and PLMN-B, already using RAN sharing in several areas of the country, make commercial arrangements with the bus company and among themselves, for sharing the use of the bus relays, such that their customers can have access to their respective HPLMN, without roaming.

Figure 5.17.1: Multi-PLMN sharing of donor RAN and vehicle relays

### 5.17.2	Pre-conditions

The following aspects and options are assumed in this use case:

- Each PLMN operator has his own 5G core network; both core networks are connected to a certain number of shared donor RAN nodes, deployed along the bus routes, which provide wireless connectivity to the vehicle relays on the buses.

- Spectrum and RAN resources used for UEs and wireless relays access could be managed by either PLMN A or B, or

by a hosting operator different from operator A or B.

- The vehicle relays will broadcast both PLMN-IDs of the sharing operators, appearing to their users/UEs like their home PLMN network.

- Based on user subscription/UE identity, the shared RAN can route traffic of PLMN-A and PLMN-B customers to/from their corresponding PLMN core network.

- It is assumed that other passengers, without subscription or roaming agreement with either PLMN-A nor B, will not get access via the bus relays. If roaming agreement is in place, in-bound roamers would also be allowed to access the vehicle relay.

### 5.17.3	Service Flows

Two users, from South-ville, are taking an inter-regional bus, for visiting their parents living up in North-ville, over the weekend:

- Ms Red, who is a PLMN-A subscriber

- Mr. Blue, who is a PLMN-B subscriber

Before entering the bus, their UEs are connected to the fixed macro-RAN (which may or may not be shared between PLMN-A and PLMN-B), and their UEs will display “PLMN-A” and “PLMN-B”, respectively.

Upon entering the bus, their UE’s display shows “PLMN-A_BUS” and “PLMN-B_BUS”, respectively, and both users receive a short message announcing the new “enhanced 5G-bus service” offered by their respective HPLMN operator, which they can use without additional charge.

During the few hours trip, both Ms Red and Mr Blue use their smartphones to keep entertained, make few calls, check emails, social media etc.

### 5.17.4	Post-conditions

Mr Blue and Ms Red enjoy the very good 5G connectivity and service quality during the whole trip, much better than last month’s trip, when the enhance 5G-Bus service was not available.

At the arrival, when leaving the bus, their UEs will re-select and/or connect from the relay to the macro-RAN (which may or may not be shared between PLMN-A and PLMN-B)

### 5.17.5	Existing features partly or fully covering the use case functionality

Basic support of RAN sharing is defined for 5G NG-RAN (since Rel-15), including service requirements (e.g. in 22.101) and stage-2/3 capabilities (e.g. 23.501, 38.331); nevertheless, they do not cover or assume specific consideration for the case of shared wireless access via mobile base stations.

### 5.17.6	Potential New Requirements needed to support the use case

[PR-5.17-1] The 5G system shall be able to support RAN sharing between multiple PLMNs for UEs connected to 5G network via mobile base station relays (e.g. mounted on vehicles).

NOTE 1: the above requirement assumes that the mobile base station relays broadcast all PLMN-IDs of the sharing PLMN operators.

NOTE 2: the above requirement assumes both relay and donor RAN resources, including UE access link and relay backhaul link, are shared among operators.

[PR-5.17-2] The 3GPP System shall support end-to-end service continuity for a UE active connection to RAN via a mobile base station relay (e.g. mounted on a vehicle), when there is a change between a shared RAN (e.g. inside the vehicle) and a non-shared RAN (e.g. outside a vehicle), or when RAN sharing changes (for the same mobile relay) between different sharing PLMNs.