3GPP TSG-SA1 Meeting #95e	S1-213176r3

Electronic Meeting, 23 August –2 September 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.262 | CR | 0001 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MSGin5G store-and-forward modifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MSGin5G |  |  |  |  |  | Date: |  |  | 2021-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA6 sent SA1 an LS (S1-213197/S6-211831) to clarify the following MSGin5G requirement: [R-5.1.2-005] The MSGin5G Service shall support storage of a message if a UE is unavailable (disconnected or power off) for future delivery once the UE becomes available. SA6 asks for clarification about the requirement as it pertains to: - Can the sender provide per-message information that can be leveraged for store-and-forwarding? If yes, what information should the sender provide to control the message handling (e.g. an indication of whether the message can be buffered, how long the message is valid, or a priority of the message)? - Can the receiver pre-configure information (e.g. preference to opt in/out) about store-and-forward? |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a service requirement for MSGin5G Service to specify the sender may provide per-message information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA6 does not complete the Rel-17 5GMARCH exception work in time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the Change---

### 5.1.2	Requirements

[R-5.1.2-001] The MSGin5G Service shall support UE sending and receiving a text or data message with end-to-end latency less than [500] ms.

NOTE 1:	Initial connection activation latencies may be longer depending on receiving UE power saving states, paging, etc.

[R-5.1.2-002] The MSGin5G Service shall support variable size of payload of a text or data message with maximum [2048] bytes, and support segmented transmission if the content is large than the maximum payload length of a message.

[R-5.1.2-003] The MSGin5G Service shall support delivery of a message to a specific application in the terminated UE. This message contains the contents that can be handled by the specific application.

[R-5.1.2-004]	The MSGin5G Service shall support acknowledgement of delivery status (success, failure) of a message and indication of reason if the delivery is failed.

[R-5.1.2-005]	The MSGin5G Service shall support storage of a message if a UE is unavailable (disconnected or power off) for future delivery once the UE becomes available.

[R-5.1.2-006]	The MSGin5G Service shall support a server in the network triggering the UE to perform an action (e.g. wake up and establish a PDN connection).

[R-5.1.2-007]	The MSGin5G Service shall support a UE sending and receiving messages via a MSGin5G Gateway

NOTE 2:	The connection between the UE and the MSGin5G Gateway can be 3GPP or non-3GPP access (e.g. WLAN.)

[R-5.1.2-008] The MSGin5G Service shall support the mobility of a UE (i.e. the UE can still send/receive messages when it changes the location of network access).

## [R-5.1.2-009] The MSGin5G Service shall support per-message information for store and forward, e.g. whether the message can be buffered or how long the message is valid.

## ---End of the Change---