3GPP TSG SA WG 1 Meeting 95-e		 S1-213010

Electronic Meeting, 23 August –2 September 2021

Title:	Reply LS on emergency services in an SNPN deployed in an area which does not belong to any country

Response to:	LS C1-213960 from CT1

Release:	Release 17

Source:	SA1

To:	CT1

Cc:	SA2

Contact person:	Toon Norp

Toon .norp @ tno .nl

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA1 thanks CT1 for their LS on emergency services in an SNPN deployed in an area which does not belong to any country. Please consider the SA1 answers to the CT1 questions.

In answering the CT1 questions, SA1 made the following assumptions:

-	It is assumed that an SNPN may be implemented on a vessel (ship, plane, oil platform, etc). Emergency support in that case is considered for UEs (belonging to crew members or passengers) that are connected to the SNPN, not for a ‘UE dedicated to a vessel’ as specified in 3GPP TS 22.119 “Maritime Communication Services over 3GPP system; Stage 1”, clause 7.2.2.

-	3GPP TS 22.119, clause 7.2.2 contains a requirement “A UE dedicated to a vessel shall support the capability of sending maritime emergency requests including the vessel-dedicated UE’s actual position (e.g. current position, heading and speed of the vessel and scheduled navigational routes).”. This requirement relates to maritime emergency services such as the Global Maritime Distress and Safety System. There is no SA1 requirement that SNPNs on a vessel should provide such support to UEs connected to the SNPN.

-	There may be different ways in which a country may have jurisdiction over a SNPN. E.g. it may have assigned frequencies, it may be the numbering authority for a PLMN ID used by the SNPN, or it may have jurisdiction for the vessel the SNPN was implemented on (e.g. country of registration for a ship). In many cases, it will be possible to find a country that has some form of jurisdiction over the SNPN.

-	When the SNPN is deployed within a country, national regulations will stipulate whether the SNPN is required to support emergency services. Many SNPN deployments will not be required to provide emergency services.

The CT1 questions relate to the following stage-1 requirement in TS 22.101 subclause 10.1:

The serving network may download emergency call numbers to the UE in order to ensure that local emergency call numbers are known to the UE. The UE shall regard these emergency numbers as valid in that country only (as identified by the MCC) and shall discard them when a new country is entered.

CT1 Question 1: When the serving network is an SNPN, whether or not the above stage-1 requirement is applicable?

SA1 answer: Yes the above stage-1 requirement is applicable when the serving network is an SNPN.

CT1 Question 2: Are emergency services supported in an SNPN deployed in an area which does not belong to any country (e.g. international waters)?

SA1 answer: Emergency services may be supported. Even in international waters, there may be a country that has jurisdiction for the SNPN or the vessel the SNPN is deployed on. In that case, regulations apply from that country, e.g. whether emergency calls need to be supported on SNPNs, and/or whether emergency calls should be supported in international waters. If there is no country that can claim jurisdiction over the SNPN, there will also be no country that can handle the emergency services response and therefore emergency services cannot be supported.

CT1 Question 3: If the answer to Question 2 is Yes, can an SNPN deployed in an area which does not belong to any country (e.g. international waters) provide emergency numbers to the UE?

SA1 answer: The SNPN may provide a list of emergency numbers in accordance with a country that has jurisdiction over the SNPN even when the SNPN is deployed in international waters. If there is no country that has jurisdiction, or if the regulations for the country that has jurisdiction stipulate that no emergency services are supported, then the SNPN should not download emergency numbers to the UE.

CT1 Question 4: If the answers to Question 2 and Question 3 are Yes, is it correct that those emergency numbers provided by the SNPN deployed in an area which does not belong to any country (e.g. international waters) are valid only in the SNPN from which they were received?

SA1 answer: A UE may be able to determine a country for the SNPN based on the MCC in the PLMN ID. In that case the requirement in TS 22.101 subclause 10.1 applies. If the UE cannot determine a country (e.g. MCC=999 is used), then the UE shall disregard emergency numbers it may have received from another network that uses MCC=999.

CT1 Question 5: If the answer to Question 2 is No, how should the UE handle the case when the user dials an emergency number stored in the ME or the USIM (e.g 112 or 911)? Should a normal call be initiated?

SA1 answer: If emergency services are not supported in an SNPN (note this can also happen when the SNPN is not in international waters but local regulations do not require emergency support for the SNPN), then the UE should not initiate a normal call to the SNPN. An SNPN that does not provide emergency support will have no way to route an emergency call to a PSAP.

# 2	Actions

To CT1

ACTION: 	Please take the above into account

# 3	Dates of next TSG SA WG 1 meetings

SA1#96e	8-18 Nov 2021			Electronic Meeting

SA1#97e	21-25 Feb 2021			Poland