3GPP TSG-SA WG Meeting #95-e	S1-213095
Electronic Meeting, Aug 23-Sept 2, 2021                                   	(revision of S1-21xxxx)

Title:           		Discussion for proposal on consolidated requirements of direct communications for FS_PIN

Agenda Item:	7.8.2

Source:	Xiaomi

Contact:	Ni Rui (nirui1@xiaomi.com)

# Abstract: This document proposes a discussion details on consolidated requirements of direct communications for FS_PIN (TR 22.859).

# Discussion

This discussion paper provides the details how PRs consolidated into CPRs, includes a proposal for clause 7.5 Direct Communications consolidated requirements, in addition to those already captured in TR 22.859 v 18.0.1.

| PR No. | CPR No. | Comments |
| --- | --- | --- |
| [PR 5.2.6-3] The 5G system shall enable PIN direct connections between PIN Elements in a PIN to use licensed spectrum (under the control of a MNO) or between PIN Elements to use unlicensed spectrum (may be under the control of the MNO, or not). | [CPR 001] The 5G system shall enable PIN direct connections between PIN Elements in a PIN to use licensed spectrum (under the control of a MNO) or between PIN Elements to use unlicensed spectrum (may be under the control of the MNO, or not). | Both for the definition of direct communication supported by 5G system. |
| [PR 5.7.6-5] The 5G system shall be able to support a PIN Element shall be able to concurrently use both operator managed and non-operator managed PIN direct connectivity with another PIN Element. |  |  |
| [PR 5.3.6-1] For intra-PIN communications, a PIN Element shall be able to transmit media to one or more PIN Element at the same time. | [CPR 002]The 5G system shall enable a PIN Element directly communicates to more than one PIN Elements at the same time. | A common statement |
| [PR 5.4.6-3] When PIN UE Element uses unlicensed spectrum for direct device connections for intra PIN UE device communications the 5G System may, subject to local/regional regulations and user consent, collect statistics data, including if 3GPP authentication was used. | [CPR 003]The 5GS may collect statistics data while PIN Elements direct communications with unlicensed spectrum, subject to local/regional regulations and user consent or 3GPP authentication if used. | A common statement |
| [PR 5.11.6-2] 5G system shall be able to allow authorized PIN Elements automatically or manually to discover other PIN element(s) for communication. | [CPR 004]The 5G system shall be able to allow authorized PIN Elements automatically or manually to discover and identify other PIN element(s) for communication. | The latter PR’s statement is contained in the former one. |
| [PR 5.13.6-1] The 5G system shall enable PIN elements to discover and identify each other. |  |  |

# Proposal

To agree the CR S1-213093 for the consolidated potential requirements for clause 7.5 direct communications in 22.859 v18.0.1, which are listed as follows.

[CPR 001] The 5G system shall enable PIN direct connections between PIN Elements in a PIN to use licensed spectrum (under the control of a MNO) or between PIN Elements to use unlicensed spectrum (may be under the control of the MNO, or not).

[CPR 002]The 5G system shall enable a PIN Element directly communicates to more than one PIN Elements at the same time.

[CPR 003]The 5GS may collect statistics data while PIN Elements direct communications with unlicensed spectrum, subject to local/regional regulations and user consent or 3GPP authentication if used.

[CPR 004]The 5G system shall be able to allow authorized PIN Elements automatically or manually to discover and identify other PIN element(s) for communication.