3GPP TSG-SA WG1 Meeting #95-e	S1-213098

Electronic Meeting, Aug 23 – Sept 2, 2021	(revision of S1-212006)

Title:	FS_TACMM: Resolving the Editor’s Notes on the use case Haptic feedback for a personal exclusion zone

Agenda Item:	7.12.1

Source:	InterDigital

Contact:	Atle Monrad <atle dot monrad at interdigital dot com>

Discussion

This document proposes resolving the Editor’s Notes on the use case “Haptic feedback for a personal exclusion zone”. Synchronization is covered by clause 5.7.

Proposal

It is proposed to modify the below use case to remove the Editor’s Notes in clause 5.5 in 3GPP TR 22.847 v0.3.0.

* * * *   First Change   * * * *

### 5.5.6	Potential New Requirements needed to support the use case

[PR. 5.5.6-1] The 5G network shall be able to support a mechanism for a 3rd party application to indicate coordination of the communication for two or more UEs and their sessions. See key performance requirements in Table 5.5.6-1. Haptic (DL) information is communicated only from Remote Site to Local Site.

Table 5.5.6-1: Potential Key performance requirements for a personal exclusion zone in dangerous remote environments.

| Traffic Direction | Traffic Type | Packet Size | Reliability (%) | Latency (ms) | Average Data rate |
| --- | --- | --- | --- | --- | --- |
| UL/DL | Video | 1.5 kByte | [99.999] | <400 [11] | 1-100 Mbit/s |
| UL/DL | Audio | 50 Byte | [99.9] | <150 [11] | 5-512 kbit/s |
| UL/DL | VR | 1.5 kByte | [99.9] | <300 | 600 Mbit/s |
| DL | Haptic | Tactile sigs. 1 DoF: 2-8 Byte 10 DoFs: 20-80 Byte 100 DoFs: 200-800 Byte | [99.999] | 0-50 [11] [12] | 1-4k pkts/s (w/o compression) 100-500 pkts/s, (w/ compression.) |

* * * *   End of Change   * * * *