3GPP SA WG1 Meeting #95e 	S1-21xxxx

Electronic Meeting, 23 Aug –2 Sep 2021	(revision of xx-yyxxxx)

# Source:	China Mobile

# Title:	New SID on supporting Computing Aware Network

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on supporting Computing Aware Network

Acronym: FS_CAN

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x |  | x |  |
| No |  |  |  |  |  |
| Don't know | x |  | x |  | x |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

In the critical period of the current 5G network development and construction, technologies such as edge computing and NFV require the coordinated development of networks and computing. In ITU-T, work regarding computing aware network has already been discussed. In the definition of Draft new Recommendation ITU-T Y.CAN-reqts: “Functional requirements of computing-aware networking”, computing aware network is the networking to support cloud computing including distributed cloud (edge cloud)/edge computing, which is not a specific cloud application or cloud service. It focuses on networking aspect and helps to solve the problem that service quality cannot be guaranteed and the problem of lack of coordination of computing resources and network resources.

The collaboration of network and computing was driven by the following aspects:

- Digital transformation requires ubiquitous connectivity and computing power. The exponential growth of data size poses a huge challenge to traditional networks and cloud computing, and drives computing down to the edge to form scattered computing resources in the network. Therefore, while the network realizes the interconnection of decentralized nodes, it also needs to coordinate the scheduling of computing power.

- Edge computing drives computing power from the centre to the edge. The computing power resources of a single site are limited, and it is impossible to guarantee that all services will be provided with the required SLA guarantee at a specific point in time. In order to ensure the user experience of delay, bandwidth, computing and other services, the computing task is scheduled to the optimal edge node for processing. On-site edge computing realizes flexible business access, providing users with intelligent access and real-time data processing

- Network evolution promotes mutual awareness of computing power networks. Computing power and network resources will co-exist. IT and CT systems need to have the ability to perceive each other, to achieve joint optimization and scheduling of network and computing power across the entire network, and to provide end-to-end ICT system SLA experience guarantee.

4	Objective

This study is aiming at identifying use cases, providing gap analysis and defining potential requirements in the following aspects regarding computing aware network.

The objectives include:

- Definition of computing aware network.

- Study use cases related to 5GS support of computing-aware network and potential requirements, e.g.

  - Measurement of computing resource and computing requirements.

  - Authentication and registration of 3rd party computing resource

  - Discovery and utilization of computing capability for service

  - Scheduling management aspect of computing resource

  - Potential enhancement on QoS of computing involved communication service

  - Interactions between computing capability provider (e.g. computing nodes) and 5GS for more efficient path routing scheduling and computing capability provider selecting

- Identify KPIs regarding computing capability including latency, reliability, etc.

- Other aspects include security, charging and privacy.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR |  | Study on supporting Computing Aware Network |  |  | Xiaonan Shi |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

Xiaonan Shi, CMCC, shixiaonan@chinamobile.com

7	Work item leadership

SA1

8	Aspects that involve other WGs

None.

9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

China Mobile

CATT

ZTE