3GPP SA WG1 Meeting #95e 	S1-213270

Electronic Meeting, 23 Aug –2 Sep 2021	(revision of S1-21329r1)

# Source:	Samsung, FirstNet, AT&T, Ericsson, Kontron, Motorola Solutions, Nokia, Nokia Shanghai Bell, SynchTechno, UIC, UK Home Office

# Title:	New WID on supporting Ad Hoc Group Communication in Mission Critical Services

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Ad hoc Group Communication support in Mission Critical Services

Acronym:	AHGC

Unique identifier:	??????

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x |  | x | Application (MCX) |
| No | x |  | x |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

{Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN}

| x | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 700028 | Mission Critical Services Common Requirements | This feature consolidated all mission critical group communication requirements into one specification, TS 22.280. |

Dependency on non-3GPP (draft) specification:

None

# 3	Justification

SA1 has already studied use cases related to MCX services and identified the requirements. Normative work is ongoing to fulfill the requirements. During the development of normative work particularly related to the user regrouping a need to support Ad hoc Group Communication has been identified. Ad hoc group communication enables authorized users to combine a random set of MCX Service Users into a group and enable communication. Ad hoc group is used for single communication and it does not persist when the communication is terminated. This non-persistent characteristic of an ad hoc group makes it different from the existing user regroup functionality. Ad hoc group communication feature is commonly found in commercial PTT systems especially the ones based on OMA-PoC, including public safety mission critical systems. This feature is not standardized by 3GPP. This could lead to non-interworking implementations. Many public safety organizations and Railway organizations have expressed interest in having a standards-based version of this feature to enable interoperability between vendors.

To achieve these objectives, this feature identifies the requirements for Ad hoc Group communication and includes them in TS 22.280. These requirements for ad-hoc communications provide distinct requirements to those defined in the requirements in TS 22.280 clause 6.6.4.1.1 on the formation of groups.

# 4	Objective

The objective of this work item will be to specify requirements to support Ad hoc Group communication feature as part of MCX Services and thereby available for MCPTT, MCVideo and MCData. Ad hoc Group communication will be treated as a type of MCX service Group communication and it supports the mechanism of call processing and call termination identical to MCX service Group communication. This work item will identify and specify service requirements for:

  - enabling users to choose an arbitrary set of users and establish group communication without having to create group ahead of communication

  - enabling administrators to configure parameters related to the ad hoc group communication (e.g., authorization aspects, maximum call duration, default priority, maximum number of participants etc.)

  - security aspects

  - affiliation aspects pertaining to the participants of the ad hoc group communication

  - off-network support

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {e.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {e.g. "TSG#89"} | {Free text} |
| 22.280 | CR to add requirements | TSG 93 |  |

# 6	Work item Rapporteur(s)

Erik.Guttman@samsung.com

# 7	Work item leadership

SA1

# 8	Aspects that involve other WGs

There are security aspects to MCX ad hoc service group creation, authorized membership and communication security.

# 9	Supporting Individual Members

Supporting IM name

AT&T

Ericsson

FirstNet

Kontron Transportation France

Motorola Solutions

Nokia

Nokia Shanghai Bell

Samsung

SynchTechno

UIC

UK Home Office