3GPP TSG-SA1 Meeting #95e	S1-213040r1

Electronic Meeting, 23 August - 2 September 2021	(revision of S1-213zzz)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.104 | CR | 0080 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex for smart grid |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom, CEPRI, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEI |  |  |  |  |  | Date: |  |  | 2021-08-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When Smart Grid related communication KPIs are planed to add into TS22.104, it will helpful to add brief background information for them especially the new use cases which deduce these new KPIs. And considering the new use cases descriptions are not only for “Electric-power distribution” but also including distributed energy storage, advanced metering, Micro-Grids, etc, it is also suggested to rename the title of A.4 to “Smart Grid”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rename the title of A.4 to “Smart Grid”. The new smart grid use cases and requirements are added to TS22.104 Annex A.4 to share reference and background of smart energy performance requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The background information for Smart Energy Infrastructure performance requirements will not be included. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4, A.4.4, adding new clauses: A.4.6,A.4.7,A.4.8,A.4.9, A.4.10,A.4.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 22.104 CR 0078, |  |  |
|  |  |  |  |  |  |  |  | TS 22.104 CR 0079 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************** Begin 1st Change *******************

# A.4	Smart Grid

***************** End of 1st  Change *******************

***************** Begin 2nd  Change *******************

## A.4.4	Distributed automated switching for isolation and service restoration

A power distribution grid fault is a stressful situation. There are self-healing solutions for automated switching, fault isolation and, service restoration. Furthermore, these solutions are ideally suited to handle outages that affect critical power consumers, such as industrial plants or data centres. Supply interruptions must be fixed within less than a second for critical power consumers. Automated solutions are able to restore power supply within a few hundred milliseconds.

Figure A.4.4-1: Depiction of a distribution ring and a failure (flash of lighting)

The FLISR (Fault Location, Isolation & Service Restoration) solution consists of switch controller devices which are especially designed for feeder automation applications that support the self-healing of power distribution grids with overhead lines. They serve as control units for reclosers and disconnectors in overhead line distribution grids.

The system is designed for using fully distributed, independent automated devices. The logic resides in each individual feeder automation controller located at the poles in the feeder level. Each feeder section has a controller device. Using peer-to-peer communication among the controller devices, the system operates autonomously without the need of a regional controller or control centre. However, all self-healing steps carried out will be reported immediately to the control centre to keep the grid status up-to-date. The controllers conduct self-healing of the distribution line in typically 500 ms by isolating the faults.

Peer-to-peer communication via IEC 61850 GOOSE (Generic Object Oriented Substation Event) messages provides data as fast as possible (Layer 2 multicast message). They are sent periodically (in steady state, with changing interval time in fault case) by each controller to several or all other controllers of the same feeder and are not acknowledged.

The data rate per controller is low in steady state, but GOOSE bursts with high data rate do occur, especially during fault situations. GOOSE messages are sent by several or all controller units of the feeder nearly at the same point in time during the fault location, isolation and service restoration procedure with a low end-to-end latency.

If the control of electrical power distribution components is performed from a central system entity, the controlled entities can be operated in a way that a controlled service restoration is possible without the use of GOOSE. Though this is not as effective as the communication has less strict requirements, this form of Distribution Automation is nevertheless effective, compliant with IEC standards and widely deployed in energy systems. The associated KPI is provided in Table A.4.4-2: KPIs for Distributed Automation.

Another use case is about the distributed feeder automation. As illustrated in the Figure A.4.4-2, the distributed feeder automation system is mainly composed of distributed master station, distribution monitoring terminal and the communication system. The distribution master station is mainly used for information gathering and human-computer interaction, and the distributed terminal is used for feeder status information collection, judgment, fault location, isolation and power supply restorationThe implementation result will be reported to the distribution master station.The communication system is to provide the communication link among the distribution terminals.

The distribution master station is usually connected with the 5G system via wired or LAN which is out of 3GPP scope. Distribution master station manages multiple distributed terminals.

Each distributed terminal here is served by 5G UE to exchange the collected data with other distributed terminals. And from application aspect, the communication between distributed terminals is peer-to-peer.The connection between the distributed terminal and the 5G UE is out of 3GPP scope.

The 5G communication hereshould have high reliability. Therefore at least two communication links are usually deployed for hot standby or transmitting data synchronously between two distributed monitoring terminals. And it has the same condition between one distributed terminal and the distributed master station.

Figure A.4.4-2 Example of distributed feeder automation

During the feeder system normal working phase, the heartbeat packet is periodic transmitted with 1s. When a fault occurs, it performs incremental periodic transmission with 2ms, 2ms, 4ms or 8ms time interval.. After there is no sudden change in collected data, the heartbeat packet transmission of 1s is restored.

In general, the feeder distributed terminals can be deployed along the overhead line or integrated in one power distribution cabinet per one square kilometre. These two topologies will require different communication density.

The third related use case is about the high speed current differential protection for smart energy distribution systems is capable of sub-millisecond fault detection. The approach utilises the natural characteristics of differential current measurements to significantly reduce fault detection times. The protection relays exchanges the current samples via the 5G system. Each relay then compares the sent and received samples to determine if a fault has occurred in a protected area in order to isolate the fault. The sampling rate varies dependent on the algorithms designed by the manufacturers. A protection relay collects the current samples (with the typical message size of up to 245 bytes) at a frequency in the range of [600, 1200, 1600, 3000] Hz. The exchange of measurement samples is done in a strictly cyclic and deterministic manner. With the sampling rate of 600 Hz (6 times per 10 ms), the transfer interval would be 1,667 ms with the required bandwidth of 1.18 Mbit/s; for 1200 Hz (12 times per 10 ms), the transfer interval would be 0,833 ms with the required bandwidth of 2.36 Mbit/s. As to the latency, as pointed out in clause 5.4 the maximum allowed end-to-end delay between two protection relays would be between 5ms and 10ms depending on the voltage levels as specified in IEC 61850-90-1. For some legacy systems the latency usually is set to 15ms.

Table A.4.4-1: Service performance requirements for distributed automated switching for isolation and service restoration

| Use case # | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value [%] | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area (note 1) |
| 1 (note 2) | 99.999 9 | – | < 5 ms | 1 kbit/s (steady state) 1.5 Mbit/s (fault case) | < 1,500 | < 60 s (steady state) ≥ 1 ms (fault case) | transfer interval (one frame loss) | stationary | 20 | 30 km x 20 km |
| NOTE 1: Length x width NOTE 2: UE to UE communication (two wireless links) |  |  |  |  |  |  |  |  |  |  |

Table A.4.4-2: KPIs for Distributed Automation

| Use case # | Bandwidth (kbit/s) | Latency | Availability (%) | Density #UE/km2 | Coverage | Power supply backup (note1) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 (note2) | 9.6-100 | 100 ms – 2 s | 99.999 | Concentrated rural 70.79562 Dispersed rural Semi-urban 7.63437 Mandatory rural support 0.04765 Urban 11.02120 | TBD | 24h-72 h |
| NOTE 1: The Power supply backup KPI is provided for background information and a deployment issue. NOTE 2: DA referred to in this table uses a centralized architecture. |  |  |  |  |  |  |

Table A.4.4-3 aperiodic deterministic communication KPI for Feeder Automation

| Use case # | Communication service availability |  |  | Communication service reliability: mean time between failures |  |  |  | Max Allowed End-to-end latency (note 1) |  | Service bit rate: user-experienced data rate |  | Message size [byte] |  | UE speed | Survival time | # of UEs | Service Area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | > 99,999 % |  |  |  |  |  | 20 ms |  |  |  | < 100 |  | Stationary |  |  | ≤ 100/km2 | several km2 |  |
|  |  |  |  |  |  | NOTE 1: UE to UE communication. |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.4-4 synchronicity KPI for Feeder Automation

| Use case # | Number of devices in one Communication group for clock synchronisation | 5GS synchronicity budget requirement | Service area |
| --- | --- | --- | --- |
| 1 | ≤ 100/km2 | ≤ 10 μs | several km2 |

Use case #1:

GOOSE (a)periodic deterministic communication service supporting bursty message exchange for fault location, isolation, and service restoration.

Table A.4.4-4 KPI for Distributed Feeder Automation (note1)

| Use case # | User experienced Data rate (bps) | IoT device to 5G network Latency (ms)(note2) | Latency jitter (us) (note3) | Synchronicity budget requirement (us) | Reliability % | Connection density | coverage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 2M-10M | # Normal:1s; Fault:2ms | <50 | <10 | 99.999 | 54/km2 (note4) 78/km2 (note5) |  |
| NOTE 1: The KPI values are sourced from [ee] NOTE 2: It is the one way delay from a distributed terminal to 5G network. NOTE 3: The latency jitter is required for the switch off between the active and standby communication links NOTE 4: When the distributed terminals are deployed along overhead line, about 54 terminals will be distributed along overhead lines in one square kilometre with the power load density is 20MW/km2. NOTE 5: When the distributed terminals are deployed in power distribution cabinets, and considering the power load density is 20MW/km2, there are about 78 terminals in one square kilometre. |  |  |  |  |  |  |  |

Use cases #2:

Distributed feeder automation

Table A.4.4-5 KPI for high speed current differential protection

| Use case # | Communication service availability | End-to-end latency: maximum (note 1) | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | > 99,999 % | 15 ms | 2,5 Mbit/s | < 245 | ≤ 1 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
|  | > 99,999 % | 15 ms | 1,2 Mbit/s | < 245 | ≤ 2 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
|  | > 99,999 % | 10 ms | 2,5 Mbit/s | < 245 | ≤ 1 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
|  | > 99,999 % | 10 ms | 1,2 Mbit/s | < 245 | ≤ 2 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
|  | > 99,999 % | 5 ms | 2,5 Mbit/s | < 245 | ≤ 1 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
|  | > 99,999 % | 5 ms | 1,2 Mbit/s | < 245 | ≤ 2 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
| NOTE 1: UE to UE communication. |  |  |  |  |  |  |  |  |  |

Use cases #3:

High speed current differential protection

***************** End of 2nd  Change *******************

***************** Begin 3rd  Change *******************

## A.4.6   Distributed Energy Storage

Distributed energy includes various forms such as solar energy, wind energy, fuel cell and gas combined. It is generally distributed in the user site, or near locations to realize energy generation, storage and supply. Distributed energy system has the characteristics of flexible location and decentralization which bring new technical problems and challenges to the power grid operation. When the distributed energy is connected to the large power grid, the energy flow on the distributed power grid becomes more complicated for that the user is becoming both the electricity user and the generator, and the current presents two-way flow and real-time dynamic changes.

The information exchanging in the distributed energy grid is utilized not only to collect energy related data, but also to coordinate working flows of the distributed energy storage equipment, to change the equivalent load characteristics, and to realize flexible energy grid through load interaction etc.

Figure A.4.6-1 Example of Distributed Energy Storage grid architecture

Seeing Figure A.4.6-1, it is one example of Distributed Energy storage grid architecture.The distributed power grid which is comprised of residential, commercial and light storage users, requires to exchange information among the Distributed Energy Storage Management Platform (DESMP) and the diverse Distributed Energy Devices (DED). The information exchanged in the distributed energy grid is not only to collect energy related data, but also to coordinate working flows of the distributed energy storage equipment, to change the equivalent load characteristics, and to realize flexible energy grid through load interaction etc.

The DED is plug-and-play and periodically collects the operating information, such as battery energy, charge and discharge energy, alarm information, etc., and transfers them to DESMP. The DESMP regularly maintains connections with the DED and determines the online status and issue instructions to the DED to control the switch of the device and set the energy etc. Further, it obtains the electricity, energy, load and other information of the grid-connected users in the area from the DED to support the decision-making of flexible interaction between the distributed energy storage and the large power grid, or to analyze the user's electricity habits to guide the operation of distributed energy storage.

Table A.4.6-1 periodic communication service performance requirements --- data for distributed energy storage

| scenario | use case | transfer interval target value (ms) | message size (byte) | data rate per storage location (bps) (note1) | communication latency (ms) | reliability | storage node density # /km2 (note2) | active factor/ km2 (note3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban | Virtual energy storage monitoring | UL: 10 | UL:>20k | UL: >16M DL: >100k | DL:10 UL:10 | DL: >99.90% | >1k | 10% |
|  | Virtual energy storage : other data collection | UL: 1000 | UL: 16k DL: >100k | UL: >128k DL: >100k | DL:<10 UL:<1000 | DL: >99.90% | >1k | 10% |
| Rural | Power storage station monitoring | UL: 10 | UL: 16k*50 | UL: > 640M DL: > 100k | DL:<10 UL:<10 | DL: >99.90% | >100 | 10% |
|  | Energy storage station operation data collection: other data | UL: 1000 | UL: 26k*50 DL: >100k | UL: > 10.4M DL: > 100k | DL:<10 UL:<1000 | DL: >99.90% | >100 | 10% |
| NOTE 1: this KPI is to require data rate in one Energy storage station which may via one or more 5G connections and via one or more 3GPP UE(s) at the same time. NOTE 2: It is used to deduce data volume in an area which has multiple energy storage stations. The data volume can be deduced through follow formula: (Current + other data) data rate per storage station * (Storage node density /km2) * (Active factor/km2) + video data rate per storage station * (Storage node density /km2) NOTE 3: active factor means the proportion that the number of active DED which are delivering its collected data during one second time window compared with whole number of DED in the area which has multiple energy storage station |  |  |  |  |  |  |  |  |

Table A.4.6-2 Aperiodic communication service performance requirements -- - video for distributed energy storage

| scenario | use case | data rate per storage station (bps) (note1) | communication latency (ms) | reliability | storage node density # / km2 |
| --- | --- | --- | --- | --- | --- |
| Rural | Energy storage station: video | UL: >5G DL: >100k | DL:<10 UL:<1000 | DL>99.90% | >100 |
| NOTE1: the data rate is to require data rate in one Energy storage station which may via one or more 5G connections and one or more 3GPP UE(s) at the same time. It can be calculated with following formula: 12.5M(bytes)/s * 50(containers) * 8 = 5Gbps |  |  |  |  |  |

Use case:

Distributed energy storage

## A.4.7   Advanced metering

Instead of recording and sending the metering data from a traditional wired electricity meter unit, electricity metering collecting can be executed by an UE integrated smart meter unit Smart meter units can send real-time metering data to the server in the Power Enterprise through mobile networks. In this way, the Power Enterprise based on the analysis of the user's power consumption behavior give users more scientific and reasonable power consumption suggestions, to develop users' power consumption and energy-saving habits.

The electric smart meters monitor relevant user energy status and deliver these data through concentrator to measurement data management system (MDMS). The MDMS sends control commands according to its policy and status data collected. The remote commands from the MDMS include: tripping, closing permission, alarm, alarm release, power protection, power protection release. Accurate Fee control is one of the basic service of advanced metering, when the user owes the fee or there is an emergency, it should be able to cut off or restore power supply in time, as the operation involves the safety& QoE of power users, a real-time response is required. Due to massive electricity meter boxes and household appliances keep online at the same time, the amount of connections the mobile network maintains simultaneously is so enormous that challenging to the mobile network capacity. It is estimated that the increased connections will up to 50-100 times.

Table A.4.7-1  Communication KPI for advanced metering (note1)

| User experienced data rate (bit/s) | Latency (ms) | Reliability % | Connection density | Coverage |
| --- | --- | --- | --- | --- |
| UL:<2M DL:<1M | Accuracy fee control: < 100 (note2); General information data collection: <3000 | >99.99 | <10000/km2 (note3) | N/A |
| NOTE 1: The KPI values refer [ee]. NOTE 2: The accuracy fee control latency here is for communication one way latency from 5G IoT device to backend system while the distance between them is no more than 40 km i.e. city range. The command implementation needs 100 ms. NOTE 3: It is the typical connection density in today city environment. With the evolution from meter centralization collection to sockets in home directly collection, the connection density is expected to increase 5-10 times. |  |  |  |  |

Use case:

Advanced metering

## A.4.8   Smart Distribution Transformer Terminal

The Smart Distribution Transformer Terminal is usually deployed in the distribution transformer area. It could support multiple energy applications simultaneously. Multiple kinds of energy data is collected firstly by the terminal and then delivered to related energy application platform. Some kind of data could be analysed, or even the terminal itself can make decision to perform real-time action. Figure 5.8.1-1 illustrates a work flow example of Smart Distribution Transformer Terminal.

Figure A.4.8-1: Example of Smart Distribution Transformer Terminal work flow

In general, the connections between the Smart Distribution Transformer Terminal and application platform are provided by 5G system, while the connections between energy end equipment and Smart Distribution Transformer Terminal may be provided by 5G system either. When they are 5G connections, about 300 ~ 500 energy end equipment are connected to a Smart Distribution Transformer Terminal. The average application layer packet data rate for one kind of smart grid application between the Smart Distribution Transformer Terminal and energy end equipment is more than 2 Mbit/s in uplink e.g. smart metering, while the communication range is from 100m to 500m.

Table A.4.8-1: Key Performance for Smart Distribution Transformer Terminal

| Average data rate (UL) (note 1) | End-to-end latency (note 2) | Area user density | Range |
| --- | --- | --- | --- |
| >2 Mbit/s | 10 ms, 100 ms, 3 s | 500 devices /distribution area (note 3) | 100 m ~ 500 m, outdoor, indoor / deep indoor |
| NOTE 1: It is the smart metering application data rate between the Smart Distribution Transformer Terminal and energy end equipment. Once there are multiple smart grid applications, it is required more data rate. NOTE 2: It depends on different applications supported by the Smart Distribution Transformer Terminal. The less the latency is, the more applications can be supported. NOTE 3: The distribution area can be calculated as 3.14*Range2 and in general is between 0.031km2~0.785km2. |  |  |  |

Use case:

Smart Distribution Transformer Terminal

## A.4.9   Distributed Energy Resources and Micro-Grids

Distributed energy resources (DER) become more and more important because of its close proximity to the user side and the convenience of complementary energy forms. The potentially large number of DERs have a direct impact on the operation and power trading of the power system, affecting the security, stability and operation efficiency of the system. The concept of micro-grid was introduced to aggregate and optimise DER thus reduce the negative impact on power grids.

The integration of DERs into the energy system cause many challenges into the communication field. The operational conditions of a micro-grid may vary rapidly due to DER contribution with low inertia of nonrotating elements and rapid changes in weather conditions (wind and solar radiation) . To incorporate more renewable and alternative energy sources, the communication infrastructure must have the ability to easily handle an increasing amount of data traffic or service requests and must provide a real‐time monitoring and control operation of all these nodes. A reliable communication between the system elements is crucial. In fact, any type of communication dependent electrical protection scheme requires robustness, a virtually full-time availability, and strictly bounded latency.

Table A.4.9-1: Key Performance for Distributed energy resources (DER)

| Characteristic parameter |  |  |  | Influence quantity |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability: target value | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bit rate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed |  |
| 99,9999 % | – | < 3 ms | 4,5 Mbit/s | 140 | ≤ 1 ms | transfer interval | stationary | 50Hz power system |
| 99,9999 % | – | < 3 ms | 5,4 Mbit/s | 140 | ≤ 1 ms | transfer interval | stationary | 60Hz power system |
| NOTE: UE to UE communication is assumed. |  |  |  |  |  |  |  |  |

Table A.4.9-2: Key Performance for DER

| Communication service availability | Max Allowed End-to-end latency | Message size [byte] | UE speed | # of UEs | Service Area |
| --- | --- | --- | --- | --- | --- |
| > 99,9999 % | < 3 ms | 160 | Stationary | - | - |
| NOTE: UE to UE communication is assumed. |  |  |  |  |  |

Use case:

Distributed energy resources and micro-Grids

## A.4.10   Ensuring uninterrupted MTC service availability during emergencies

During emergencies, public mobile land networks (PLMNs) may restrict network access, which may lead to a lack of service reliability for machine-type communication (MTC) in critical applications, such as power systems and in particular in microgrids. Microgrids are separate parts of a power grid, which can be controlled and operated individually in a so-called island mode or together as a whole. Existing features of a mobile network can be used to differentiate MTC of devices in a microgrid from other MTC or human-to-human (H2H) communication and ensure that these microgrid devices have service during emergencies, which enables use of mobile communication to co-ordinate the use of Distributed Energy Resources (DER) in microgrids, so that they can autarkically perform blackout recovery of an islanded microgrid. The idea is to ensure reliable communications for selected MTC devices during emergency conditions, without giving these devices additional priority over other network users during normal conditions or adversely affecting the service to prioritised users during emergencies. It was shown that this method allows the blackout recovery 100 times faster than with a conventional black start.

Blackout recovery using autarkic microgrids:

-	Identify which nodes in the MAS are working

-	Identify the current microgrid topology and DER capacity

-	Re-energise the microgrid and the loads.

Table A.4.10-1: Key Performance for uninterrupted MTC service availability

| Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability | Communication service reliability: mean time between failures | Max Allowed End-to-end latency (note 1) (note 2) | Service bit rate: user-experienced data rate (note 2) | Message size [byte] | Survival time | UE speed | # of UEs | Service Area |
| 99.999 9 % |  | 100 ms | < 1 Kbit/s per DER |  |  | Stationary |  |  |
| NOTE 1: Unless otherwise specified, all communication includes 1 wireless link (UE to network node or network node to UE) rather than two wireless links (UE to UE). NOTE 2: It applies to both UL and DL unless stated otherwise. |  |  |  |  |  |  |  |  |

Use case:

Ensuring uninterrupted MTC service availability during emergencies

## A.4.11   Applications Using IEC 61850-9-2 Sampled Values

As electric grids are expanding, there is an increasing need for fault detection and location as well as maintaining system stability in real time which requires precision timing. There is also a raising interest to have other clock synchronization sources in addition to GNSS.

IEC 61850-9-2 standard [49] specifies the protocol for transmitting measurement information in a power system using Ethernet. Standard also recommends the use of PTP (Precision Time Protocol) for clock synchronization and IEC 61850-9-3 standard [7] shall be followed whenever PTP is used. PTP accuracy is affected by network structure and devices in the network. Since different types of clock devices produce different time errors and jitters, overall time inaccuracy must be verified to make sure that application time accuracy requirements are fulfilled. Requirements in IEC 61850-9-3 [7] aim at achieving a network time inaccuracy better than 1μs after crossing 15 transparent clocks or 3 boundary clocks. PTP profile for power utility automation defines the inaccuracy as follows:

Table A.4.11-1. Allowed inaccuracy defined by IEC 61850-9-3 [7]

| Clock type | Inaccuracy |
| --- | --- |
| Transparent clock (TC) | < 50ns |
| Boundary clock (BC) | < 200ns |
| Grandmaster clock | < 250ns |

Grandmaster clocks in substations today use a GNSS receiver to achieve accurate clock synchronization with holdover time even up to 24h (corresponding to <1μs accuracy). However, since the time source (GNSS) may become unavailable due to failure (e.g. broken antenna, satellite availability or interference), 5G is a candidate resiliency solution (see [51] clause 5.2). In order to reach the same level of measurement accuracy, 5G capable grandmaster clock should fulfil the requirement of maximum 250ns inaccuracy when its timing reference is used in the same location as of the Grandmaster clock that is used to synchronize the smart grid devices over Ethernet.

Synchronization requirements are related to various aspects. In particular, IEC/IEEE 60255-118-1:2018 [50] defines a Phasor Measurement Unit (PMU) that must maintain less than a 1% total vector error (TVE). Such error includes time, phasor angle, and phasor magnitude estimation errors. Standard also recommends that a time source that reliably provides time, should be at least 10 times better than values corresponding to 1 % TVE. Therefore IEC 61850-9-3 [7] should be followed whenever substation has PMUs either as standalone devices or PMU capable protection relays.

By using PTP within a substation the system can have multiple grandmaster capable clocks which, in case of failure of the current grandmaster, can be used to maintain accurate time between devices and processing sampled values. However, when sampled values are processed outside the substation, time inaccuracy limits defined by IEC 61850-9-3 [7] must be followed since the devices are not connected to the same grandmaster clock and therefore measurement timestamps may have too much inaccuracy. Application outside substation may be but not limited to Wide Area Protection.

Table A.4.11-2: Clock synchronization service performance requirements

| User-specific clock synchronicity accuracy level [ee] | Number of devices in one Communication group for clock synchronisation | Clock synchronicity requirement (see note) | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 4 | Up to 100 UEs | <250ns - 1 µs [ee] | < 20 km2 | Smart Grid: Synchronicity between sync master and PMUs This range covers the extreme cases where the PTP clock in the end device uses 5G sync modem as direct time-source (1 µs) The 5G sync modem acts as PTP GM or 5G sync modem provides PPS output to PTP GM at the top of the Ethernet based synchronization chain with up to 15 transparent clocks or 3 boundary clocks (250 ns). |
| 4a | Up to 100 UEs | <10-20 µs [ee] | < 20 km2 | Smart Grid: Power system protection in digital substation with merging units, line differential protection and synchronization |
| 4b | Up to 100 UEs | <1 ms [ee] | < 20 km2 | Smart Grid: Event reporting and Disturbance recording use-cases |
| NOTE: The clock synchronicity requirement refers to the clock synchronicity budget for the 5G system |  |  |  |  |

Use case:

Applications Using IEC 61850-9-2 Sampled Values

***************** End of 3rd Change *******************