3GPP TSG-SA1 Meeting #95e	S1-213040r2

Electronic Meeting, 23 August - 2 September 2021	(revision of S1-213zzz)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.104 | CR | 0080 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex for smart grid |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom, CEPRI, Samsung, Siemens? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEI |  |  |  |  |  | Date: |  |  | 2021-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When Smart Grid related communication KPIs are planed to add into TS22.104, it will helpful to add brief background information for them especially the new use cases which deduce these new KPIs. And considering the new use cases descriptions are not only for “Electric-power distribution” but also including distributed energy storage, advanced metering, Micro-Grids, etc, it is also suggested to rename the title of A.4 to “Smart Grid”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add references support the Annex information. Rename the title of A.4 to “Smart Grid”. The new smart grid use cases and requirements are added to TS22.104 Annex A.4 to share reference and background of smart energy performance requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The background information for Smart Energy Infrastructure performance requirements will not be included. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, A.4, A.4.4, adding new clauses: A.4.6,A.4.7,A.4.8,A.4.9, A.4.10,A.4.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 22.104 CR 0078, |  |  |
|  |  |  |  |  |  |  |  | TS 22.104 CR 0079 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************** Begin 1st Change *******************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	IEC 61784-3: "Industrial communication networks – profiles – part 3: functional fieldbuses – general rules and profile definitions".

[4]	BZKI, "Aspects of dependability assessment in ZDKI", June 2017.

[5] 	BZKI, "Requirement Profiles in ZDKI", 2017.

[6]	IEC 61158: "Industrial communication networks – fieldbus specification", 2014.

[7]	IEC 61907, "Communication network dependability engineering".

[8]	Richard C. Dorf and Robert H. Bishop, "Modern Control Systems", Pearson, Harlow, 13th Edition, 2017.

[9]	Ernie Hayden, Michael Assante, and Tim Conway, "An Abbreviated History of Automation & Industrial Controls Systems and Cybersecurity", SANS Institute, https://ics.sans.org/media/An-Abbreviated-History-of-Automation-and-ICS-Cybersecurity.pdf {accessed: 2017-05-23}, 2014.

[10]	IEC 61512 "Batch control - Part 1: Models and terminology".

[11]	RESERVE project, Deliverable D1.3, ICT Requirements, 
http://www.re-serve.eu/files/reserve/Content/Deliverables/D1.3.pdf, September 2017.

[12]	RESERVE project, Deliverable D1.2, Energy System Requirements 
http://www.re-serve.eu/files/reserve/Content/Deliverables/D1.2.pdf, September 2017.

[13]	G. Garner, "Designing Last Mile Communications Infrastructures for Intelligent Utility Networks (Smart Grids)", IBM Australia Limited, 2010.

[14]	B. Al-Omar, B., A. R. Al-Ali, R. Ahmed, and T. Landolsi, "Role of Information and Communication Technologies in the Smart Grid", Journal of Emerging Trends in Computing and Information Sciences, Vol. 3, pp. 707-716, 2015.

[15]	H. Kagermann, W. Wahlster, and J. Helbig, "Recommendations for implementing the strategic initiative INDUSTRIE 4.0", Final report of the Industrie 4.0 working group, acatech – National Academy of Science and Engineering, Munich, April 2013.

[16]	IEC 62443-3-2: "Security for industrial automation and control systems - Part 3-2: Security risk assessment and system design", in progress.

[17]	IEC 62657-2: "Industrial communication networks - Wireless communication networks - Part 2: Coexistence management", 2017.

[18]	IEC 62657-1: "Industrial communication networks – Wireless communication networks – Part 1: Wireless communication requirements and spectrum considerations".

[19]	IEEE Std 802.1Q "Media Access Control (MAC) Bridges and Virtual Bridge Local Area Networks".

NOTE: IEEE Std 802.1Qbv-2015 "Enhancements for Scheduled Traffic" has been included into IEEE Std 802.1Q-2018.

[20] 	IEEE, Use Cases IEC/IEEE 60802, 2018.

[21] 	"IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications in Bridged Local Area Networks--Corrigendum 1: Technical and Editorial Corrections," IEEE Std 802.1AS-2011/Cor 1-2013 (Corrigendum to IEEE Std 802.1AS-2011), pp. 1-128, Sept 2013.

[22] 	"IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications," IEEE Std 802.1AS-Rev/D7.3, pp. 1-502, August 2018.

[23]	3GPP TS 22.289: "Mobile Communication System for Railways".

[24]	IEEE P802.1CS: “IEEE Draft Standard for Local and metropolitan area networks - Link-local Registration Protocol”

[25]	IEEE P802.1Qdd: “IEEE Draft Standard for Local and Metropolitan Area Networks--Bridges and Bridged Networks -- Amendment: Resource Allocation Protocol (RAP)”

[26]	IEC/IEEE 60802: "Time-Sensitive Networking Profile for Industrial Automation".

[27]	3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)".

[aa] 	 IEC TR 61850-90-1:2010, Communication Networks and Systems for Power Utility automation – Part 90-1: Use of IEC61850 for the communication between substations.

[bb]	5G DNA White Paper: "5GDN@Smart Grid White Paper: Requirements, Technologies, and Practices" https://www.5gdna.org/

[cc]	IEC 61850-9-2:2011 Communication networks and systems for power utility automation - Part 9-2: Specific communication service mapping (SCSM) - Sampled values over ISO/IEC 8802-3

[dd]	61850-90-3-2016 – IEC/IEEE International Standard - Communication Networks and Systems for Power Utility automation – Part 90-3: Precision time protocol profile for power utility

[ee]	IEC/IEEE 60255-118-1-2018 - IEEE/IEC International Standard - Measuring relays and protection equipment - Part 118-1: Synchrophasor for power systems – Measurements

[ff]	IEEE Guide for Application of Digital Line Current Differential Relays Using Digital Communication.

***************** End of 1st  Change *******************

***************** Begin 2nd  Change *******************

A.4	Electric-power distribution and Smart Grid

***************** End of 2nd  Change *******************

***************** Begin 3rd  Change *******************

A.4.4	Distributed energy automation

A.4.4.1	Distributed automated switching for isolation and service restoration

A power distribution grid fault is a stressful situation. There are self-healing solutions for automated switching, fault isolation and, service restoration. Furthermore, these solutions are ideally suited to handle outages that affect critical power consumers, such as industrial plants or data centres. Supply interruptions must be fixed within less than a second for critical power consumers. Automated solutions are able to restore power supply within a few hundred milliseconds.

Figure A.4.4.1-1: Depiction of a distribution ring and a failure (flash of lighting)

The FLISR (Fault Location, Isolation & Service Restoration) solution consists of switch controller devices which are especially designed for feeder automation applications that support the self-healing of power distribution grids with overhead lines. They serve as control units for reclosers and disconnectors in overhead line distribution grids.

The system is designed for using fully distributed, independent automated devices. The logic resides in each individual feeder automation controller located at the poles in the feeder level. Each feeder section has a controller device. Using peer-to-peer communication among the controller devices, the system operates autonomously without the need of a regional controller or control centre. However, all self-healing steps carried out will be reported immediately to the control centre to keep the grid status up-to-date. The controllers conduct self-healing of the distribution line in typically 500 ms by isolating the faults.

Peer-to-peer communication via IEC 61850 GOOSE (Generic Object Oriented Substation Event) messages provides data as fast as possible (Layer 2 multicast message). They are sent periodically (in steady state, with changing interval time in fault case) by each controller to several or all other controllers of the same feeder and are not acknowledged.

The service bit rate per controller is low in steady state, but GOOSE bursts with high service bit rate do occur, especially during fault situations. GOOSE messages are sent by several or all controller units of the feeder nearly at the same point in time during the fault location, isolation and service restoration procedure with a low end-to-end latency.

The associated periodic communication KPI is provided in Table A4.4-1. The fault detection and isolation is typically event driven requires aperiodic communication while the KPI is provided in Table A4.4.1-2. Its time synchronicity requirement is provided in Table A4.4.1-3.

Table A.4.4.1-1: Service performance requirements for distributed automated switching for isolation and service restoration

| Use case # | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value [%] | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area (note 1) |
| 1 (note 2) | 99.999 9 | – | < 5 ms | 1 kbit/s (steady state) 1.5 Mbit/s (fault case) | < 1,500 | < 60 s (steady state) ≥ 1 ms (fault case) | transfer interval (one frame loss) | stationary | 20 | 30 km x 20 km |
| NOTE 1: Length x width NOTE 2: UE to UE communication (two wireless links) |  |  |  |  |  |  |  |  |  |  |

Table A.4.4.1-2 Aperiodic deterministic communication KPI for fault detection and isolation

| Use case # | Communication service availability |  | Max Allowed End-to-end latency (note 1) |  | Message size [byte] |  | UE speed | # of UEs | Service Area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | > 99.999 % | 20 ms |  | < 100 |  | Stationary |  | ≤ 100/km2 | several km2 |  |
| NOTE 1: UE to UE communication. |  |  |  |  |  |  |  |  |  |  |

Table A.4.4.1-3 Synchronicity KPI for fault detection and isolation

| Use case # | Number of devices in one Communication group for clock synchronisation | 5GS synchronicity budget requirement | Service area |
| --- | --- | --- | --- |
| 1 | ≤ 100/km2 | ≤ 10 μs | several km2 |

Use case one

GOOSE (a)periodic deterministic communication service supporting bursty message exchange for fault location, isolation, and service restoration.

.

A.4.4.2	Distributed Automation without use of GOOSE

If the control of electrical power distribution components is performed from a central system entity, the controlled entities can be operated in a way that a controlled service restoration is possible without the use of GOOSE. Though this is not as effective as the communication has less strict requirements, this form of Distribution Automation is nevertheless effective, compliant with IEC standards and widely deployed in energy systems. The associated KPI is provided in Table A.4.4.2-1: KPIs for Distributed Automation.

Table A.4.4.2-1: KPIs for Distributed Automation without use of GOOSE

| Use case # | Bandwidth (kbit/s) | End-to-end latency: maximum | Communication service availability: target value (%) | Density #UE/km2 | Coverage | Power supply backup (note1) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 9.6 to100 | 100 ms to 2 s | 99.999 | Concentrated rural 70.8 Dispersed rural Semi-urban 7.6 Rural support 0.048 Urban 11.0 | TBD | 24 h to 72 h |
| NOTE 1: The Power supply backup KPI is provided for background information and a deployment issue. |  |  |  |  |  |  |

Use case one

Distributed automation without use of GOOSE using a centralized architecture.

A.4.4.3	Intelligent distributed feeder automation

Intelligent distributed feeder automation system which supported by 5G connections among the intelligent power distribution terminals and distributed master station is designed to realize intelligent judgment, analysis, fault location, fault isolation and non-fault area power supply restoration operations. As illustrated in the Figure A.4.4.3-1, the distributed feeder automation system is mainly composed of distributed master station, distribution terminal and the communication system (UEs in the substations, 5G network, plus the data network). The distribution master station is mainly used for information gathering and human-computer interaction, and the distributed terminals are used for feeder status information collection, judgment, fault location, isolation, and power supply restoration. The implementation result will be reported to the distribution master station. The 5G communication system is to provide the communication link among the distribution terminals. The distribution master station is usually connected to the 5G system via a data network, which is out of 3GPP scope. The distribution master station manages multiple distributed terminals. Each distributed terminal is served by a 5G UE to exchange the collected data with other distributed terminals. From an application perspective, the communication between distributed terminals is peer-to-peer. The 5G communication service availability needs to be very high. Therefore, at least two communication links are usually deployed for hot standby or transmitting data synchronously between two distributed terminals. The associated KPIs are provided in Table A.4.4.3-1.

Figure A.4.4.3-1 Example of intelligent distributed feeder automation

Table A.4.4.3-1 KPI for intelligent distributed feeder automation

| Use case # | User experienced data rate (bit/s) (note 1) | IoT device to 5G network Latency (note 2) | Synchronicity budget (μs) | Reliability % | Connection density | coverage |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 M to10 M | # Normal:1 s; Fault:2 ms | <10 | 99.999 | 54/km2 (note 4) 78/km2 (note 5) |  |
| NOTE 1: The KPI values are sourced from [bb] NOTE 2: It is the one way delay from a distributed terminal to 5G network. NOTE 3: When the communication link switches off between the active and standby links, the switch off time between the two links is less than 50 µs which will be considered in the E2E latency, NOTE 4: When the distributed terminals are deployed along overhead line, about 54 terminals will be distributed along overhead lines in one square kilometre. The resulting power load density is 20 MW/km2. NOTE 5: When the distributed terminals are deployed in power distribution cabinets, there are about 78 terminals in one square kilometre. The resulting power load density is 20 MW/km2. |  |  |  |  |  |  |

Use cases #1: Intelligent Distributed feeder automation

A.4.4.4	High speed current differential protection

High-speed current differential protection which is required sub-millisecond fault detection is another typical use case of power distribution automation. The approach utilises the natural characteristics of differential current measurements to significantly reduce fault detection time. The protection relays exchange the current samples via the 5G system. Each relay then compares the sent and received samples to determine if a fault has occurred in a protected area. This is done in order to identify and isolate a fault in the grid. The sampling rate varies dependent on the algorithms designed by the manufacturers. A protection relay collects the current samples (with the typical message size of up to 245 bytes) at a frequency in the range of 600Hz, 1200Hz, 1600Hz, 3000Hz. The exchange of measurement samples is done in a strictly cyclic and deterministic manner. With the sampling rate of 600 Hz, the transfer interval is 1.7 ms and the required bandwidth 1.2 Mbit/s; for 1200 Hz , the transfer interval is 0.83 ms and the required bandwidth 2.4 Mbit/s. As to the maximum allowed end-to-end delay between two protection relays would be between 5 ms and 10 ms, depending on the voltage (see  IEC 61850-90-1 for more details [aa]). For some legacy systems, the latency usually is set to 15 ms. The associated KPIs are provided in Table A.4.4.4-1.

Table A.4.4.4-1 KPI for high speed current differential protection

| Use case # | Communication service availability | End-to-end latency: maximum (note 1) | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | > 99.999 % | 15 ms | 2.5 Mbit/s | < 245 | ≤ 1 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
| 2 | > 99.999 % | 15 ms | 1.2 Mbit/s | < 245 | ≤ 2 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
| 3 | > 99.999 % | 10 ms | 2.5 Mbit/s | < 245 | ≤ 1 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
| 4 | > 99.999 % | 10 ms | 1.2 Mbit/s | < 245 | ≤ 2 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
| 5 | > 99.999 % | 5 ms | 2.5 Mbit/s | < 245 | ≤ 1 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
| 6 | > 99.999 % | 5 ms | 1.2 Mbit/s | < 245 | ≤ 2 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
| NOTE 1: UE-to-UE communication. |  |  |  |  |  |  |  |  |  |

Use case #1: High speed current differential protection with sampling rate of 1200 Hz for legacy system.

Use case #2: High speed current differential protection with sampling rate of 600 Hz for legacy system.

Use case #3: High speed current differential protection with sampling rate of 1200 Hz under voltage condition1 (see  IEC 61850-90-1 for more details [aa]).

Use case #4: High speed current differential protection with sampling rate of 600 Hz under voltage condition1 (see  IEC 61850-90-1 for more details [aa]).

Use case #5: High speed current differential protection with sampling rate of 1200 Hz under voltage condition2 (see  IEC 61850-90-1 for more details [aa]).

Use case #6: High speed current differential protection with sampling rate of 600 Hz under voltage condition2 (see  IEC 61850-90-1 for more details [aa]).

***************** End of 3rd  Change *******************

***************** Begin 4th  Change *******************

A.4.6   Distributed energy storage

Distributed power generation includes various power sources such as solar, wind, fuel cells, and gas. Distributed power generation typically comes with a low power density and entails thus typicall a decentralised deployment. Decentralisation causes technical problems and challenges Smart Grid operators. When distributed power generation is connected to the elctrical grid, the energy flow becomes more complicated as the user is both a electricity consumer and producer (a so-called “prosumer”). Therefore, the current in the electricity grid can change direction at different locations of the grid and at different times of the day.

The information exchanging in the distributed energy grid is not only including power-generation-related data, but also the control order for the distributed energy storage equipment, e.g. change the load characteristics to realise a flexible electricity grid etc.

Figure A.4.6-1 Example of Distributed Energy Storage grid architecture

Figure A.4.6-1 shows an example architecture of Distributed Energy Storage Grid.The Distributed Energy Storage Grid, which is comprised of residential, commercial and light storage users, requires the exchange of information among the Distributed Energy Storage Management Platform (DESMP) and the Distributed Energy Devices (DED).

The DED is one kind of plug-and-play device and periodically collects its energy information, such as battery energy, charge and discharge status, energy alarm information, etc., and transfers them via 5G UE to DESMP. The DESMP regularly manages the DEDs e.g. monitors their working status, controls their working modes or configures their energy parameters etc. The associated KPIs are provided in Table A.4.6-1 and Table A.4.6-2.

Table A.4.6-1 periodic communication service performance requirements ‒ data for distributed energy storage

| scenario | use case | transfer interval target value (ms) | message size (byte) | User experienced data rate per storage location (bit/s) (note1) | communication latency (ms) | reliability | storage node density # /km2 (note2) | activity factor/ km2 (note3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban | Virtual energy storage monitoring | UL: 10 | UL: > 20 k | UL: > 16 M DL: > 100 k | DL: 10 UL: 10 | DL: > 99.90% | > 1 k | 10 % |
|  | Virtual energy storage : other data collection | UL: 1000 | UL: 16 k DL: > 100 k | UL: > 128 k DL: > 100 k | DL: < 10 UL: < 1000 | DL: > 99.90% | > 1 k | 10 % |
| Rural | Power storage station monitoring | UL: 10 | UL: 16k x 50 | UL: > 640 M DL: > 100 k | DL: < 10 UL: < 10 | DL: > 99.90% | > 100 | 10 % |
|  | Energy storage station operation data collection: other data | UL: 1000 | UL: 26 k x 50 DL: >100 k | UL: > 10.4 M DL: > 100 k | DL: < 10 UL: < 1000 | DL: > 99.90% | > 100 | 10 % |
| NOTE 1: this KPI is to require service bit rate in one Energy storage station which may via one or more 5G connections and via one or more 3GPP UE(s) at the same time. NOTE 2: It is used to deduce data volume in an area which has multiple energy storage stations. The data volume can be deduced through follow formula: (Current + other data) service bit rate per storage station x (storage node density per km2) x (activity factor per km2) + (video service bit rate per storage station) x (Storage node density per km2) NOTE 3: Activitiy factor means the proportion that the number of active DED which are delivering its collected data during one second time window compared with whole number of DED in the area which has multiple energy storage station |  |  |  |  |  |  |  |  |

Table A.4.6-2 Aperiodic communication service performance requirements -- - video for distributed energy storage

| Scenario | Use case | User experienced data rate per storage station (bit/s) (note1) | Communication latency (ms) | Reliability | Storage node density (#/ km2) |
| --- | --- | --- | --- | --- | --- |
| Rural | Energy storage station: video | UL: > 5 G DL: > 100 k | DL: < 10 UL: < 1000 | DL > 99.9% | > 100 |
| NOTE1: It can be calculated with following formula: 12.5 Mbytes/s x 50(containers) x 8 = 5 Gbit/s |  |  |  |  |  |

A.4.7   Advanced metering

Instead of recording and sending metering data from a wired electricity meter unit, electricity metering collecting can be executed by an UE-integrated smart meter unit. Smart meter units can send real-time metering data to the server in the power enterprise through mobile networks. In this way, the Power Enterprise―based on the analysis of the user's power consumption behavior―-gives users power consumption suggestions, which fosters the users' power consumption and energy saving habits.

The electric smart meters monitor relevant user energy status and deliver the status data to measurement data management system (MDMS). The MDMS sends control commands according to its policy and status data collected from the smart meters. The MDMS commands include tripping, closing permission, alarm, alarm release, power protection, and power protection release. Accurate-fee control is one of the basic services of advanced metering. When the electric power user doesn’t pay his electric fee on time, the MDMS can cut off power supply in time. And when there is a need for temporary power supply for this user, the MDMS can recover the power supply either. This operation requires real-time interaction between the electric smart meter and MDMS. Due to massive electricity meters utilized, it is estimated that in the near future, the amount of this kind of communication connection will be increased up to 5-10 times. The associated KPIs are provided in Table A.4.7-1.

Table A.4.7-1  Communication KPI for advanced metering (note1)

| User experienced data rate (bit/s) | Latency (ms) | Reliability % | Connection density |
| --- | --- | --- | --- |
| UL:<2M DL:<1M | Accuracy fee control: < 100 (note2); General information data collection: <3000 | >99.99 | <10000/km2 (note3) |
| NOTE 1: The KPI values refer [ee]. NOTE 2: It is one way end-to-end latency from 5G IoT device to backend system. The distance between the two is below 40 km (city range). NOTE 3: It is the typical connection density in today city environment. With the evolution from centralised meters to socket meters in the home, the connection density is expected to increase 5 to 10 times. |  |  |  |

A.4.8   Smart Distribution Transformer Terminal

The Smart Distribution Transformer Terminal is usually deployed in the distribution transformer area. It can support multiple energy applications simultaneously. Multiple kinds of energy data are collected firstly by the terminal and then delivered to related energy application platform. The collected energy data also can be analysed in the terminal locally and then infer the decision to perform real-time action. Figure A.4.8.1-1 illustrates a work flow example of Smart Distribution Transformer Terminal.

Energy end equipment

Energy application platform

Figure A.4.8-1: Example of Smart Distribution Transformer Terminal work flow

In general, the connections between the Smart Distribution Transformer Terminal and the Energy application platform are provided by the 5G system. The connections between energy end equipment and Smart Distribution Transformer Terminal may be or not provided by 5G system. If it is provided by 5G system, about 300 to 500 energy end devices are connected to one Smart Distribution Transformer Terminal. The average service bit rate between the Smart Distribution Transformer Terminal and an energy end device is more than 2 Mbit/s in uplink for each application. The related communication distance is between 100m to 500m. The associated KPIs are provided in Table A.4.8-1.

Table A.4.8-1: Key Performance for Smart Distribution Transformer Terminal

| Average service bit rate (UL) (note 1) | End-to-end latency (note 2) | User density | Coverage |
| --- | --- | --- | --- |
| > 2 Mbit/s | 10 ms, 100 ms, 3 s | 500 devices/distribution area (note 3) | 100 m ~ 500 m, outdoor, indoor / deep indoor |
| NOTE 1: It is the smart metering application data rate between the Smart Distribution Transformer Terminal and energy end equipment. Once there are multiple smart grid applications, it is required more data rate. NOTE 2: It depends on different applications supported by the Smart Distribution Transformer Terminal. The less the latency is, the more applications can be supported. NOTE 3: The distribution area can be calculated as 3.14 x range2 and in general is between 0.031 km2 and 0.79 km2. |  |  |  |

A.4.9   Distributed energy resources and micro-Grids

Distributed energy resources (DER) become increasingly important. The potentially large number of DERs will have an impact on security, stability, and operation efficiency of the whole energy grid. The concept of micro-grid was introduced to DER and thus reduce the negative impact on energy grids.

The integration of DERs into the energy grid poses many challenges for the involved communication system. To incorporate more renewable and alternative energy sources, the communication infrastructure must have the ability to easily handle an increasing amount of data traffic or service requests and must provide a real-time monitoring and control operation for these distributed energy resources. A reliable communication between them is crucial.

When it comes to communications architecture, IEC61850 is a widely accepted standard for automation and equipment of power utilities and DER, specifically for defining protocols for IEDs (Intelligent electronic devices) at electrical substations. IEC 61850 standard specifies the timing constraints for messages typically used in substations (table 5.15.1-1). GOOSE (Generic Object Oriented Substation Events) and SV (Sampled Values) messages are assumed as time critical messages, having the tightest deadlines (maximum allowed transfer time) among all IEC 61850 messages, corresponding to 3 ms. While GOOSE is typically used to transfer information closely related to monitoring and control functions (circuit breaker status etc.), SV is used to transfer numerical samples of current and voltage signals. The SV protocol works on a periodic information transmission model, regularly sending messages at a fixed rate. For protection purposes, the default rate is 4000 or 4800 messages per second for, respectively, 50 or 60 Hz power systems. On the other hand, the GOOSE protocol operates in a sporadic information transmission model, where a continuous flow of data is maintained to increase communication reliability. The typical sizes of GOOSE and SV messages are, respectively, 160 and 140 bytes. GOOSE messages are transmitted at two different modes: 1) Safe operation: 1 message / second (with the bitrate of 1.28 kbps); 2) Emergency operation: 32 message / second (with the bitrate of 40.96 kbps). SV messages are transmitted at much high rate 4800 message / second (with the bitrate of 5.376 Mbps). The associated KPIs are provided in Table A.4.9-1 and Table A.4.9-2.

Table A.4.9-1: Key Performance for Distributed energy resources (DER): using SV (Sampled Values) message

| Characteristic parameter |  |  |  | Influence quantity |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability: target value | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bit rate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed |  |
| 99,9999 % | – | < 3 ms | 4,5 Mbit/s | 140 | ≤ 1 ms | transfer interval | stationary | 50 Hz power system |
| 99,9999 % | – | < 3 ms | 5,4 Mbit/s | 140 | ≤ 1 ms | transfer interval | stationary | 60 Hz power system |
| NOTE: UE to UE communication is assumed. |  |  |  |  |  |  |  |  |

Table A.4.9-2: Key Performance for DER: using GOOSE message

| Communication service availability | Max Allowed End-to-end latency | Message size [byte] | UE speed | # of UEs | Service Area |
| --- | --- | --- | --- | --- | --- |
| > 99.9999 % | < 3 ms | 160 | Stationary | - | - |
| NOTE: UE to UE communication is assumed. |  |  |  |  |  |

A.4.10   Ensuring uninterrupted communication service availability during emergencies

During emergencies, public mobile land networks (PLMNs) may restrict network access, which may lead to a prohibitevly low communication service availability for machine-type communication (MTC) in critical condition. Example is communication for microgrids. Microgrids are separate parts of a power grid, which can be controlled and operated individually in a so-called island mode or together with other part of a power grid as a whole. Existing features of a mobile network can be used to differentiate MTC of devices in a microgrid from other kind of MTC devices or human-to-human communication. It ensures that these microgrid devices have communication service during emergencies. Thus it enables co-ordinate the use of DERs in micro-grids, then the DERs can autarkically perform blackout recovery of an islanded microgrid. The idea is to ensure reliable and available communication for selected devices during emergency conditions, without giving these devices additional priority or adversely affecting to other prioritised users during emergencies.

The associated KPIs are provided in Table A.4.10-1.

Table A.4.10-1: Key Performance for uninterrupted MTC service availability

| Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability | Communication service reliability: mean time between failures | Max Allowed End-to-end latency (note 1) (note 2) | Service bit rate: user-experienced data rate (note 2) | Message size [byte] | Survival time | UE speed | # of UEs | Service Area |
| 99.999 9 % |  | 100 ms | < 1 kbit/s per DER |  |  | Stationary |  |  |
| NOTE 1: Unless otherwise specified, all communication includes 1 wireless link (UE to network node or network node to UE) rather than two wireless links (UE to UE). NOTE 2: It applies to both UL and DL unless stated otherwise. |  |  |  |  |  |  |  |  |

A.4.11   Applications using sampled values according to IEC 61850-9-2

As electric grids are expanding, there is an increasing need for fault detection and fault location. For maintaining system stability in real time, it requires precision timing. There is also a raising interest to have other clock synchronization sources in addition to GNSS.

IEC 61850-9-2 [cc] specifies a protocol for transmitting measurement information in an energy grid using Ethernet. This standard also recommends the use of the Precision Time Protocol (PTP) for clock synchronization and the IEC 61850-9-3 standard [dd] shall be followed whenever PTP is used. Since different types of clocks produce different time errors and jitter, overall time inaccuracy must be evaluated to make sure that application time accuracy requirements are fulfilled. In IEC 61850-9-3 [dd], the inaccuracy of a network time is specified, i.e. it should be better than 1μs after crossing 15 transparent clocks or 3 boundary clocks. PTP inaccuracies allowed for power utility automation are listed in Table A.4.11-1:

Table A.4.11-1. Allowed inaccuracy defined by IEC 61850-9-3 [dd]

| Clock type | Inaccuracy |
| --- | --- |
| Transparent clock (TC) | < 50 ns |
| Boundary clock (BC) | < 200 ns |
| Grandmaster clock | < 250 ns |

Grandmaster clocks in substations today use a GNSS receiver to achieve clock synchronization with holdover time even up to 24h (corresponding to <1μs accuracy). However, since the time source (GNSS) may become unavailable due to failure (e.g., broken antenna, unavailability of the GNSS satellites, or interference of the satellite signal), 5G is a candidate resiliency solution. In order to reach the same level of measurement accuracy as GNSS, 5G capable grandmaster clock must have an inaccuracy of of 250ns or less when its timing reference is used in the same location as the Grandmaster clock that is usually used to synchronise Smart-Grid devices over Ethernet.

Synchronization requirements are related to various scenarios. In particular, IEC/IEEE 60255-118 [ee] defines a Phasor Measurement Unit (PMU) that must maintain less than a 1% total vector error (TVE). Such error includes time, phasor angle and phasor magnitude estimation errors. It also recommends that a time source that reliably provides time, should be at least 10 times better than values corresponding to 1 % TVE. IEC 61850-9-3 [dd] should be followed whenever substation has PMUs either as standalone devices or PMU-capable protection relays.

By using PTP within a substation, the system can have multiple grandmaster-capable clocks, which―in case of failure of the current grandmaster―can be used to maintain accurate time between devices. However, when sampled values are processed outside the substation, time inaccuracy limits defined by IEC 61850-9-3 [dd] must be followed, since the devices are not connected to the same grandmaster clock and therefore measurement timestamps may be too inaccurate.

The associated KPIs are provided in Table A.4.11-2.

Table A.4.11-2: Clock synchronization service performance requirements

| User-specific clock synchronicity accuracy level [ff] | Number of devices in one Communication group for clock synchronisation | Clock synchronicity requirement (see note) | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 4 | Up to 100 UEs | < 250 ns - 1 µs [dd] | < 20 km2 | Smart Grid: Synchronicity between sync master and PMUs This range covers the extreme cases where the PTP clock in the end device uses 5G sync modem as direct time-source (1 µs) The 5G sync modem acts as PTP GM or 5G sync modem provides PPS output to PTP GM at the top of the Ethernet based synchronization chain with up to 15 transparent clocks or 3 boundary clocks (250 ns). |
| 4a | Up to 100 UEs | < 10 µs to 20 µs [dd] | < 20 km2 | Smart Grid: Power system protection in digital substation with merging units, line differential protection and synchronization |
| 4b | Up to 100 UEs | < 1 ms [dd] | < 20 km2 | Smart Grid: Event reporting and Disturbance recording use-cases |
| NOTE: The clock synchronicity requirement refers to the clock synchronicity budget for the 5G system |  |  |  |  |

***************** End of 4th Change *******************

***************** End of Changes *********************