3GPP TSG-SA1 Meeting #95e	S1-213139r1

Electronic Meeting, 23 August – 2 September 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0568 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of VMR requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Firstnet, SyncTechno Inc., Philips B.V., Lenovo, Motorola Mobility, Bosch, AT&T, InterDigital, Verizon UK Ltd, vivo Mobile Communications Ltd, Telstra, DENSO Corporation, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2021-08-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the outcome of the SA1 Rel-18 study on FS_VMR, and the corresponding normative WID, new service requirements are needed in TS 22.261. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new section capturing service requirements on mobile base station relays, based on the latest consolidated potential requirements in TR 22.839. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of normative service requirements for mobie base station relays. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.xx (new section, after 6.37) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============================  Start of Changes =======================================

## 6.xx	Mobile base station relays

### 6.xx.1	Introduction

The requirements below refer to a “mobile base station relay” (or mobile BS relay), which is a mobile base station acting as a relay between a UE and the 5G network, i.e. providing a NR access link to UEs and connected wirelessly (using NR) through a donor NG-RAN to the 5G Core. Such mobile BS relay is assumed to be mounted on a moving vehicle and serve UEs that can be located inside or outside the vehicle (or entering/leaving the vehicle).

NOTE: 	The radio link used between a mobile BS relay and served UEs, as well as between mobile BS relay and donor RAN, is assumed to be NR-Uu; in that regard, it should be clear that a mobile BS relay is different than a UE relay (which uses instead a PC5-based link to provide indirect connection to remote UEs).

Few main underlying assumptions are:

- requirements cover single-hop relay scenarios as baseline (multi-hop is not precluded);

- legacy UEs are supported;

- other stage-1 requirements (e.g. on wireless self-backhaul), as well as existing stage-2/3 functionalities and architecture options (e.g. IAB) do not assume or address full relay mobility (e.g. BS relays on board of moving vehicles), thus may not cover the requirements below, which are intended to be specific to mobile BS relays;

- the identified requirements do not intend to imply or exclude specific network/relay architectures and topology solutions (e.g. could be IAB based, or others).

### 6.xx.2	Requirements

The 5G system shall support efficient operation of mobile base station relays.

The 5G system shall be able to support means, for a mobile network operator, to configure, provision and control the operation of a mobile base station relay.

The 5G system shall be able to support provisioning and configuration mechanisms to control UEs’ selection and access to a mobile base station relay.

The 5G system shall be able to support RAN sharing between multiple PLMNs for UEs connected to the 5G network via mobile base station relays.

NOTE 1: 	the above requirement assumes both relay and (donor) RAN resources, including UE access link and relay backhaul link, are shared among operators.

The 5G system shall be able to configure and provision specific required QoS for traffic relayed via a mobile base station relay.

NOTE 2: 	QoS is end-to-end, i.e. from UE to 5GC.

The 5G system shall support means to expose monitoring information of a mobile base station relay (e.g. number of UEs connected, QoS) to an MNO’s authorized third-party, based on monitoring configuration requested by the third-party.

The 5G system shall be able to provide means to optimize network behaviour to efficiently deliver data based on the mobility information (e.g., itinerary), known or predicted, of mobile base station relays.

The 5G system shall be able to support communication from/to users of one MNO (MNO-A) via mobile base station relays, where the traffic between the relay and the MNO-A network is transported using 5G connectivity (RAN and 5GC) provided by a different MNO (MNO-B).

NOTE 3: The 5G connectivity provided to the MNO-A relays by the different MNO (MNO-B) assumes a generic wireless backhaul transport, independent from the mobile BS relay functionalities.

The 5G system shall be able to support UEs connectivity to RAN using simultaneously, a link without mobile BS relay and a link via a mobile BS relay, or simultaneous links via different mobile BS relays.

NOTE 4: 	The above requirements cover scenarios were the two links (to the RAN) could be connected to the same or different RAN node(s), and assuming both relay and RAN belong to the same PLMN.

The 5G system shall be able to provide means to support efficient UE cell selection and cell reselection (between mobile BS relays or between relays and RAN) in the presence of mobile base station relays.

The 5G system shall be able to ensure end-to-end service continuity, in the presence of mobile base station relays

NOTE 5: 	the above requirement intends to cover different scenarios of UE mobility (e.g. UE moving between two mobile BS relays, or between macro RAN and BS relay) and BS relay mobility (e.g. BS relay moving between different donor RAN nodes).

The 5G system shall be able to identify and differentiate UEs’ traffic carried via a mobile base station relay and collect charging information, including specific relay information (e.g. geographic location served by the relay).

The 5G System shall support means for a mobile base station relay to have a certain subscription with a HPLMN, used to get access and connectivity to the HPLMN network (via a donor RAN).

The 5G System shall support the ability of a base station relay to roam from its HPLMN into a VPLMN.

The 5G system shall support mechanisms, for the HPLMN controlling a mobile base station relay, to enable/disable mobile relay operation if the relay is roaming in a VPLMN.

The 5G system shall support mechanisms to disable mobile relay operation by a VPLMN where a mobile base station relay is roaming to.

The 5G system shall be able to fulfil necessary regulatory requirements (e.g. for support of emergency services) when UEs access the 3GPP network via a mobile base station relay.

The 5G system shall be able to support priority services (e.g. MPS) when UEs access the 3GPP network via a mobile base station relay.

The 5G system shall be able to support location services for the UEs accessing 5GS via a mobile base station relay.

The 5G system shall ensure that existing end-to-end 5G security between the UE and 3GPP network is unaffected when the UE accesses the 3GPP network via a mobile base station relay.

The 5G system shall be able to minimize radio interference possibly caused by mobile base station relays.

The 5G system shall minimize the impact of the presence of mobile base station relays on radio network management (e.g. through automatic neighbour cell list configuration).

==============================  End of Changes =======================================