3GPP SA WG1 Meeting #97e 	S1- 220034

Electronic Meeting, 14 - 24 Feb 2022

# Source:	China Telecom

# Title:	New SID on Support for Minimization of Service Interruption during Core Network Failure

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Support for Minimization of Service Interruption during Core Network Failure

Acronym:	FS_MINT_CN

Unique identifier:	XXXXXX

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No |  |  |  |  | X |
| Don't know | X | X |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 830018 | Study on Support for Minimization of Service Interruption | This SID studies the minimization of service interruption during RAN failure. |

3	Justification

In 5G system, ensuring the high availability of communication service is critical, because the number of users that are impacted by the CN is large. Once the 5G CN service is failed, the communication for millions of users would be interrupted and unpredictable loss would be caused. To minimize the service interruption, some related study has been done in SA1(e.g. MINT). However, the existing identified requirements only consider the service interruption caused by the failure of RAN, and the requirements of service interruption caused by the failure of 5G CN remain unclear.

Two typical use cases have been identified as follows:

- Use case 1: CN failure due to failed network control

Failed upgrade of NF/NE(s), computer virus, software bugs, incorrect network configurations or malfunctions may cause abnormal behavior of the CN network and put the entire network out of service.

- Use case 2: CN failure due to natural disasters

Natural disasters such as storms, floods and fires may lead to extreme and sudden damage to CN network NF/NE(s) in a particular region.

Such events have led to lots of network service disruptions in the past few years leading to huge social and economic loss. Therefore, current 3GPP system needs some improvements to minimize service interruption during CN failure.

In order to avoid the repeated construction of 5G network infrastructure and save the construction cost, Multi Operator Core Network (MOCN) network sharing scenario, where only the RAN is shared, has been introduced to 5G system. MOCN supports operator’s ability to use more than one PLMN ID. It can be considered as an example scenario to study minimizing service interruption during the failure of 5G CN. In MOCN scenario, when one operator’s 5G CN is failed, the service interruption may be minimized by temporarily using the service provided by the CN of the cooperative operator.

When an operator’s 5G CN cannot provide communication service to its user due to certain events, it’s important to minimize the time and the impact of service interruption. To achieve this, some aspects need to be considered from a stage 1 perspective during the study. For example, when the users of the failed CN use the service provided by the cooperative operator’s CN, it should not lead to congestion of the network and the service provided to its home users due to surge of connection. Other aspects include privacy protection, e.g. the privacy information of the failed CN cannot be exposed to the CN of the cooperative operator in the duration when the users of the failed CN access to the cooperative CN.

It is noted that in 22.261 and 22.011, service requirements for minimization of service interruption in Disaster Condition have been specified, where the Disaster Condition is defined as the condition that a government decides whether applicable or not based on, e.g. a natural disaster or a man-made disaster. However, existing requirements do not cover the cases when the PLMN service is interrupted due to conditions that are not the Disaster Condition defined by the government as mentioned in the use cases above. It is also noted that the Work Item MINT in R17 only deals with the 5G RAN failure, but does not cope with the 5G CN failure.

4	Objective

The aim of this study is to gather and analyse scenarios of 5G CN failure and derive potential service requirements out of these scenarios. Potential aspects to be addressed to derive relevant potential service requirements include e.g.:

- 	The scenarios, scales or causes of interruption of communication service;

- 	Allowing impacted users to be authorized to access some essential service (e.g. voice, SMS) of the functioning 5G CN, while not affecting the service provided to its home users;

- 	Other aspects, including security and privacy requirements;

- 	Gap analysis between the identified requirements for MINT_CN and what is already defined by existing 3GPP requirements.

This study considers both the MOCN network sharing and non-MOCN scenarios. The scope is limited to the case during 5G CN failure.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 22.XXX | Study on Support for Minimization of Service Interruption during Core Network Failure | TSG#98 (Dec 2022) | TSG#99 (Mar 2023) | Yinglin Chen, China Telecom, chenyl37@chinatelecom.cn |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| N/A | N/A | N/A | N/A |

6	Work item Rapporteur(s)

Yinglin Chen, China Telecom, chenyl37@chinatelecom.cn

7	Work item leadership

SA1

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Telecom