3GPP TSG-SA WG1 Meeting #97e 	S1-220084

Electronic Meeting, 14 – 24 February 2022	(revision of S1-22xxxx)

Source:	Samsung

Title:	Concerning Reply LS on MINT and Higher priority PLMN Selection

Document Type:	Discussion

Agenda item:	3 (LSs and related contributions)

Document for:	Discussion

Contact:	Erik Guttman <erik.guttman@samsung.com>

Abstract: This document considers the issues and proposed resolutions from incoming LS C1-220816 and the draft reply S1-220zzz.

1. Introduction

CT1 asks three questions related to support of Disaster Inbound Roamers. How should the PLMN search be done by the UE in this condition?

2. Discussion

Before considering the questions in the LS, I introduce the related functional requirements in 22.261.

In 22.261, 6.31.2.2 there are the following requirements:

The 3GPP system shall be able to provide efficient means for a network to inform Disaster Inbound roamers that a Disaster Condition is no longer applicable.

Subject to regulatory requirements or operator’s policy, the 3GPP system shall support a PLMN operator to be made aware of the failure or recovery of other PLMN(s) in the same country when the Disaster Condition is applies, or when the Disaster Condition is not applicable.

It is not specified in stage 1 how the UE knows whether the disaster condition applies. This is understandable since SA1 doesn’t specify ‘how’ only ‘that’ functions work.

In fact, the way that the UE can find out about the disaster condition is

-	The SIB2 (which lists which PLMNs have a disaster condition)

-	PLMN selection information (if it includes a non-forbidden PLMN)

I argue you need both forms of information for the following reason:

-      The Disaster condition must apply for the UE to do disaster roaming at all.

-      However, the fact that the disaster condition applies does not mean that the UE needs to do DIR. If there is a network that the UE could roam to without DIR, it must do that. (See the requirements below.)

In 22.011, 3.2.2.2. A):

In the case of a UE operating in UE operation mode A or B, an allowable PLMN is one which is not in the "Forbidden PLMN" data field in the SIM/USIM. This data field may be extended in the ME memory (see clause 3.2.2.4). In the case of a UE operating in UE operation mode C, an allowable PLMN is one which is not in the "Forbidden PLMN" data field in the SIM/USIM or in the list of "forbidden PLMNs for GPRS service" in the ME. When there is no available PLMN except for PLMNs in the "Forbidden PLMN" data field in the SIM/USIM, and the available PLMNs indicate that Disaster Condition applies, this PLMN shall be considered allowable for registration to the UE while the Disaster Condition is applicable.

And  22.261 6.31.2.3:

The 3GPP system shall be able to provide means to enable a UE to access PLMNs in a forbidden PLMN list if a Disaster condition applies and no other PLMN is available except for PLMNs in the forbidden PLMN list.

How can the UE know if there is an available PLMN if it doesn’t check regularly?

The 3GPP system shall provide means to enable that a Disaster Condition applies to UEs of a specific PLMN.

The SIB2 lists that some PLMNs have this condition.

Consider the following scenario to illustrate this situation.

The incoming LS C1-220816 asks three questions.

Question 1) Is the UE allowed or even mandated to scan for all available PLMNs in order to obtain service on an Answer to question 1) Yes, the UE is mandated to scan for all available PLMNs in order to obtain service on an allowable PLMN when registered for disaster roaming service.

Since the UE is required to determine this lack of availability of PLMNs that are not on the Forbidden PLMN list, the UE is required to scan for all available PLMNs when registered for disaster roaming service.

Question 2) Is the UE allowed or even mandated to attempt service on such other allowable PLMNs when registered for disaster roaming service or shall the UE continue to use disaster roaming service on the FPLMN?

Answer to question 2) Yes, the UE is mandated to scan for all available PLMNs in order to obtain service on an allowable PLMN when registered for disaster roaming service.

The UE is required to register on a PLMN that is not on the Forbidden PLMN list if it is possible to do so, rather than a PLMN on the Forbidden PLMN List, when registered for disaster roaming service.

Question 3) The interval of time between searches for a higher priority PLMN is configured by HPLMN. Can SA1 kindly provide the reasoning whether this interval of time needs be different/adjusted for the UE in disaster roaming condition?

Answer to question 3) Further details, and possible changes, on the interval of time needed for PLMN scanning should be better discussed in CT1.

4. Proposal

Please consider approving the draft reply LS in S1-220zzz.