3GPP TSG-SA WG1 Meeting #97e 	S1-220090

Electronic Meeting, 14 – 24 February 2022	(revision of S1-22xxxx)

Source:	Lenovo, Motorola Mobility

Title:	Discussion on support of disaster roaming by PNI-NPN with CAG

Agenda Item:	3

Document for:	Approval

Contact:	Hyung-Nam Choi (hchoi5@lenovo.com)

Abstract: This document proposes SA1 to respond to the RAN2 Reply LS on UAC enhancements and system information extensions for minimization of service interruption.

##### Discussion

CT1, SA2 and RAN2 are working on the Rel-17 stage 2 and stage 3 specifications for MINT. In this context RAN2 discussed whether disaster roaming can be supported by PNI-NPN and asked CT1 for clarification. In the reply LS [1] CT1 provided the following response to RAN2 question:

Question 3: RAN2 understood that disaster roaming is not supported for SNPNs, but is disaster roaming supported for PNI-NPNs? With respect to SNPN, CT1 agrees with RAN2. With respect to PNI-NPN, CT1 agreed that disaster roaming is not supported, i.e., a PNI-NPN does not accept a UE performing disaster roaming if the UE is not allowed to access the PNI-NPN, as per the guidance provided by SA1 in the attached LS C1-213553.

However, companies in RAN2 interpreted the response from CT1 differently. Therefore, RAN2 sent further reply LS [2] for asking CT1 on the correct interpretation, see below:

RAN2 did not reach consensus on how to interpret whether MINT is supported for PNI-NPN. RAN2 identified two interpretations of CT1's reply in C1-217156: - disaster roaming is not supported for PNI-NPN at all (i.e. with or without CAG), or - disaster roaming service can be provided by a PNI-NPN with CAG. And a disaster roaming UE is allowed to access the PNI-NPN as long as the UE is allowed to access the CAG based on the UE's Allowed CAG list. RAN2 asks CT1 to confirm which of interpretation 1 or 2 is correct. And if interpretation 2 is correct to provide potential clarifications whether there are any other applicable conditions for a disaster roaming UEs that attempt to access a PNI-NPN.

Since SA1 specified the service requirements for MINT and this issue was already discussed in SA1#94e (May 2021) in the email thread [LS S1-211265] we think the correct response should be interpretation 2 which is aligned with the response SA1 sent to CT1 [3], see below. However, considering the potential impacts to stage 3 specifications (e.g. specifying procedures for registering a CAG-capable disaster roaming UE to a PNI-NPN and configuring CAG information to that UE) and the tight schedule to complete the stage 3 MINT work by March 2022, the final decision whether to support disaster roaming by PNI-NPN with CAG can be left to CT1.

On following question from CT1: Question: When a CAG-supporting UE determines that Disaster Condition applies, and a PLMN can provide disaster roaming to the UE, is the UE without CAG configuration for the PLMN allowed to select and register on a CAG cell of the PLMN? SA1’s answer is: Answer: No. Current CAG restrictions apply also during disaster conditions and roaming. As such, a UE without CAG configuration for a PLMN (even if it can provide disaster roaming) is not allowed to select and register on a CAG cell of that PLMN.

Based on the discussion above, we propose SA1 to respond to RAN2 and CT1 with the following answer:

- From service requirements point of view disaster roaming can be supported by PNI-NPN with CAG. However, considering the potential impacts to stage 3 specifications SA1 want to leave the final decision to CT1.

##### 2. References

[1]	R2-2200061, Response to reply LS on UAC enhancements and system information extensions for minimization of service interruption (C1-217156; To: RAN2; contact: Nokia)

[2]	S1-220053, Reply LS on UAC enhancements and system information extensions for minimization of service interruption (R2-2201841, To: CT1; cc: SA1; contact: Ericsson)

[3]	S1-211323, Reply LS on disaster roaming and non-public network hosted by a PLMN (To: CT1; contact: LG Electronics)