3GPP TSG-SA WG1 Meeting #97e 	   S1-220103

Electronic Meeting, 14 – 24 Feb. 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.874 | CR | 0007 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of KPI in TR22.874 FS_AMMT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMMT |  |  |  |  |  | Date: |  |  | 2022-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In use case of 5.4 Split control for robotics, as described in 5.4.6, If everything is done on the edge part, the robot needs to send 592B sensing data per control cycle (every millisecond) and receive 200B per control cycle from the “remote controller”, which leads to a UL data rate of 4.7Mbit/s and a DL data rate of 1.6Mbit/s. However maximum round-trip latency is limited to 3ms, and the one-way latency for downloading “remote control part” needs to be limited to 1ms. If the splitting strategy is followed, downloading the “remote control part” from the cloud/edge control server requires downloading a 40kB data burst per control cycle (every millisecond) as more information is needed to ensure the local controller can take over in case of unexpected latencies, which leads to a user experienced DL data rate of 320Mbit/s. In that case, a maximum round-trip latency of 25ms can be tolerated between each control cycle, and the one-way latency for downloading “remote control part” can be relaxed to 12ms. there are two use cases (full control at edge, and split control): Use case-1 (full control at edge) For each 1ms, an UL data with size of 592B is sent by UE, and the corresponding DL data with size of 200B is sent back to UE. The round-trip latency (3ms) = application processing(1ms) + 5GS UL and DL transmission (2ms). Uss case-2 (split control) For each 1ms, an UL data with size of 592B is sent by UE, and the corresponding DL data with size of 40kB is sent back to UE. The round-trip latency (25ms) = application processing (12ms) + 5GS UL and DL transmission (13ms) Actually, instead of discussing oneway latency, the UE mostly cares about the round-trip latency (i.e. totall UL+DL latency for data transmission with certain UL and DL data size). Therefore, it is proposed to further update it to add the round-trip latency with certain UL and DL data size. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add round-trip latency with certain UL and DL payload size for use case “split control for robotics” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some requirements are not clarified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.6, 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 22.261 CR 0633 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** Start of Change ******************

5.4	Split control for robotics

5.4.1	Description

Mobile robots have been playing an increasingly important role in some scenarios, e.g. warehouse, disaster rescue and smart factories [5], thanks to their high mobility. The mobile robots need to work in an ever-changing environment, hence need to perform fast and reliable sensing, planning and controlling. If the corresponding computation is performed on board in the robot, it will require intensive computations which lead to increased requirements of computation capabilities and power consumption. However, a light-weight form factor is always a requirement to the mobile robots working in the real-world environment, which prevents the robots to be equipped with a large number of CPU/GPU units and large-capacity batteries. As the example provided in [17], an advanced commercially available quadruped robot, carries 3kg of batteries of about 650Wh energy, while the high-end GPU consumes more than 250W of power, significantly impacting battery life if such computational power was embedded on the robot.

Offloading computations from robots to the cloud has been studied in many references [16]. Meanwhile relying on either data or code from a network to support the robot’s operation, the designers of autonomous mobile robots have to consider the scenarios where the robots must include capacity of local processing for low-latency responses during periods when network access quality is varying worse.

The resulting system is different from the fully remote-controlled robot system described in [5], in which the planning and controlling are carried out by cloud computing, and the robots only reports the sensing data (incl. video), and receives the control commands. Since the complete cloud computing can hardly meet the latency requirement of the ms-level feedback control loop of some types of mobile robots, e.g. legged robots, the split control of mobile robots is an agreeable solution in this case.

[17] introduces a robot whole-body balance control split over 5G network. The AI inference for the controlling can be split between the robot and the cloud server: As shown in Figure 5.4.1-1, the part which is complex but less susceptible to delays is offloaded to the remote computation in the cloud or edge control server. The low-complexity part which contains the error feedback terms and is latency-critical can be efficiently done by the local computation in the robot. If the robot fails to receive the optimal control from “remote control part” from the cloud/edge control server due to communication delays or packet loss, it can approximate the “remote control part” using pre-computed feedback matrices received previously. And in certain duration, the approximation will still enable the robot to perform feedback control for the tasks approximately and ensure that the robot can still operate.

Figure 5.4.1-1. Split control of legged robot over 5G network

The results in [17] show that, in case the robot is completely controlled by a cloud server, the robot cannot finish the walking task if the round-trip latency is larger than 3ms (from sending sensing data to receiving control commands, including processing at cloud/edge). Due to delayed control commands, the robot would fall down (as shown in Figure 5.4.1-2 (a)). However, if the split control is employed, a worse-case 25ms round-trip latency can be sustained, and the robot can still perform the walking task (as shown in Figure 5.4.1-2 (b)).

- 5G remote control without local control   (b) 5G remote control with local control at robot

Figure 5.4.1-2. Simulated performance of robot whole-body balance control over 5G network with 25ms round-trip latency

5.4.2	Pre-conditions

The involved AI/ML endpoints (e.g. UE (robot), AI/ML cloud/edge server) run applications providing the capability of AI/ML model inference for robot control task, and support the split robot control operation.

The 5G system has the ability to provide 5G network related information to the AI/ML server.

5.4.3	Service Flows

- The UE (robot) is connected with the cloud/edge control server via 5G network.

- The split of the control operations on robot side and cloud/edge control server side for the robot control task is determined by the robot or the network.

- Under the determined split mode and split point, the robot performs the local control computation based on the collected sensing data, and sends the sensing data to the cloud/edge control server. The cloud/edge control server performs the remote control computation, and feeds the outputs back to the robot.

- The robot controls its motion jointly based on the outputs of local and remote control computations.

- Start with Step 3) with more control operations, until the robot control task is terminated.

5.4.4	Post-conditions

The robot receives the control from local and remote with required accuracy and latency, so to finish the moving tasks, e.g. balance task and walking task.

The robot control task can be completed under the available computation and energy resource of the robot. And the consumed the computation, communication and energy resources over the AI/ML endpoints are optimized.

5.4.5	Existing features partly or fully covering the use case functionality

This use case mainly requires high data rate together with low latency. The high data rate requirements to 5G system are listed in Clause 7.1 and 7.6 of TS22.261 [4]. As in Table 7.1-1 of [4], 300Mbps DL experienced data rate and 50Mbps UL experienced data rate are required in dense urban scenario, and 1Gbps DL experienced data rate and 500Mbps UL experienced data rate are required in indoor hotspot scenario. As in Table 7.6.1-1 of [4], cloud/edge/Split rendering-related data transmission requires up to 0.1Gbps data rate with [5-10]ms latency countrywide.

5.4.6	Potential New Requirements needed to support the use case

If everything is done on the edge part, the robot needs to send 592B sensing data per control cycle (every millisecond) and receive 200B per control cycle from the “remote controller”, which leads to a UL data rate of 4.7Mbit/s and a DL data rate of 1.6Mbit/s.  However maximum round-trip latency is limited to 3ms, and the one-way latency for downloading “remote control part” needs to be limited to 1ms. If the splitting strategy is followed, downloading the “remote control part” from the cloud/edge control server requires downloading a 40kB data burst per control cycle (every millisecond) as more information is needed to ensure the local controller can take over in case of unexpected latencies, which leads to a user experienced DL data rate of 320Mbit/s. In that case, a maximum round-trip latency of 25ms can be tolerated between each control cycle, and the one-way latency for downloading “remote control part” can be relaxed to 12ms.

This implies a trade-off between DL data rate and latency: Compared with the full control at edge, the split control mode requires a higher user experienced DL data rate, but relaxes the stringent latency requirement. Different from the traditional URLLC services requiring continuous coverage of 5G network which can only be provided with FR1 spectrum, for a 5G operator with FR2 spectrum, the split control for robotics can be offloaded to the 5G mmWave network with non-continuous coverage.

5.4.6.1	Potential KPI Requirements

The potential KPI requirements needed to support the use case include:

[P.R.5.4-001] The 5G system shall support “Remote control part” downloading for split control for robotics with a maximum latency as given in Table 5.4.6.1-1 (corresponding size of “Remote control part” is also listed in the table).

[P.R.5.4-002] The 5G system shall support “Remote control part” downloading for split control for robotics with a user experienced DL data rate as given in Table 5.4.6.1-1 (corresponding size of “Remote control part” is also listed in the table).

[P.R.5.4-003] The 5G system shall support “Remote control part” downloading for split control for robotics with communication service availability not lower than 99.999 %.

Table 5.4.6.1-1: Data rate and latency requirements for robotic control

| Control mode | User experienced UL data rate for sensing data uploading | Size of “Remote control part” : maximum | User experienced DL data rate for “Remote control part” downloading | Latency for “Remote control part” downloading: maximum | Total latency of UL+DL data transmission |
| --- | --- | --- | --- | --- | --- |
| Full control at edge | 4.7Mbit/s (NOTE 1) | UL: 592Bytes DL: 200Bytes | 1.6Mbit/s (NOTE 1) | 1ms | 2ms |
| Split control | 4.7Mbit/s (NOTE 2) | UL: 592Bytes DL:40kBytes | 320Mbit/s (NOTE 2) | 12ms | 13ms |
| NOTE 1: In the use case of Full control of edge, the UL and DL data rate may be changed as long as the data transfer is completed within the round-trip latency (13ms) with certain UL and DL payload size (UL: 592Bytes, DL: 200Bytes) NOTE 2: In use case of Split control, the UL and DL data rate may be changed as long as the data transfer is completed within the round-trip latency (13ms) with certain UL and DL payload size (UL: 592Bytes, DL: 40kBytes) |  |  |  |  |  |

*******************Second Change*****************

8	Consolidated potential requirements

8.1	KPI for AMMT services

Editor’s Note: 	This section summarizes the available KPI values from the various approved use cases based on v0.3.0. The KPI values in the tables may be corrected/modified aligned with the newly-added use-cases and the updates to the approved use-cases.

In table 8.1-1, 8.1-2 and 8.1-3, only the subset of experienced data rate values which are agreeable for the expected 5G enhancements are captured from the use cases.

- The agreeable upper bound for split AI/ML inference is [1.1Gbit/s] in DL and UL respectively.

- The agreeable upper bound for AI/ML model downloading is [1.1Gbit/s] in DL. The upper bound is [4Gbit/s] in DL if the downloading is only supported in hotspot coverage.

- The agreeable upper bound for Federated Learning is [1.1Gbit/s] in DL and UL respectively.

NOTE:	It might be possible in the selection of members for the federated/distributed learning to prioritize users in good coverage areas. In that case, higher data rates can be possible.

Table 8.1-1 KPI Table of split AI/ML inference between UE and Network Server/Application function

| # Uplink KPI |  |  |  |  | # Downlink KPI |  |  |  |  | # Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Max allowed UL end-to-end latency | # Experienced data rate | # Payload size | # Communication service availability | # Reliability | # Max allowed DL end-to-end latency | # Experienced data rate | # Payload size | # Reliability | # Round-trip latency |  |
| [CPR-001] 2ms | [CPR-002] 1.08Gbit/s (see note 1) | # 0.27 MByte | # [CPR-003] 99.999 % | # [CPR-037] 99.9% |  |  |  | # [CPR-038] 99.999% |  | # Split AI/ML image recognition |
| [CPR-004] [100ms] | [CPR-005] [1.5Mbit/s] |  |  |  | # [CPR-006] [100ms] | # [CPR-007] [150] Mbit/s | 1.5 MByte /frame |  |  | # Enhanced media recognition |
| # 1ms # (see NOTE3) | # 4.7Mbit/s (see NOTE3) | 592 Byte |  |  | [CPR-008] 12ms (see NOTE3) | [CPR-009] 320Mbit/s (see NOTE3) | 40kByte |  | 13ms (UL+DL) | Split control for robotics |
| NOTE 1: Only the values corresponding to AlexNet model is captured. NOTE 2: Communication service availability relates to the service interfaces, and reliability relates to a given system entity. One or more retransmissions of network layer packets may take place in order to satisfy the reliability requirement. NOTE 3: The UL and DL latency and data rate can be adjusted as long as the transfer of data payload (UL: 592Bytes, DL: 40kBytes) is completed within the round-trip latency (13ms). |  |  |  |  |  |  |  |  |  |  |

Table 8.1-2 KPI Table of AI/ML model downloading

| # Max allowed DL end-to-end latency | # Experienced data rate # (DL) | # Model size | # Communication service availability | # Reliability | # User density | # # of downloaded AI/ML models | # Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [CPR-010] 1s | [CPR-011] 1.1Gbit/s | 138MByte | [CPR-012] 99.999 % | [CPR-039] 99.9% for data transmission of model weight factors; 99.999% for data transmission of model topology |  |  | AI/ML model distribution for image recognition |
| [CPR-013] 1s | [CPR-014] 640Mbit/s | 80MByte | [CPR-015] 99.999 % |  |  |  | AI/ML model distribution for speech recognition |
| [CPR-016] 1s | [CPR-017] 512Mbit/s / [4Gbit/s] (see note 1) | < 64MByte / 500MByte |  |  |  | Parallel download of up to 50 AI/ML models | Real time media editing with on-board AI inference |
| [CPR-018] 1s |  | 536MByte |  |  | [CPR-019] up to 5000~ 10000/km2 in an urban area |  | AI model management as a Service |
| [CPR-020] [500ms] | [CPR-021] [100 Mbit/s] | [40MByte] | [CPR-022] 99.999 % |  |  |  | AI/ML based Automotive Networked Systems |
| [CPR-023] [1s] |  | [500]MByte |  |  |  |  | Shared AI/ML model monitoring |
| [CPR-024] 3s | [CPR-025] 450Mbit/s | [CPR-026] 170MByte |  |  |  |  | Media quality enhancement |
| NOTE 1: 512Mbit/s concerns AI/ML models having a size below 64 MB. 4Gbit/s concerns AI/ML models having a size below 500 MB where the model downloading is only supported in hotspot coverage. NOTE 2: Communication service availability relates to the service interfaces, and reliability relates to a given system entity. One or more retransmissions of network layer packets may take place in order to satisfy the reliability requirement. |  |  |  |  |  |  |  |

Table 8.1-3 KPI Table of Federated Learning between UE and Network Server/Application function

| # Max allowed DL or UL end-to-end latency | # DL experienced data rate | # UL experienced data rate | # DL packet size | # UL packet size | # Communication service availability | # Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| [CPR-027] [1]s | [CPR-028] 1.0Gbit/s | [CPR-029] 1.0Gbit/s | 132MByte | 132MByte |  | Uncompressed Federated Learning for image recognition |
| [CPR-030] [1s] | [CPR-031] 80.88Mbit/s | [CPR-032] 80.88Mbit/s | 10Mbyte | 10Mbyte | [CPR-033] [99.9%] | Compressed Federated Learning for image/video processing |
| [CPR-034] [1s] | [CPR-035] [1.1Gbit/s] | [CPR-036] [500Mbit/s] | 10MByte | 10MByte |  | Data Transfer Disturbance in Multi-agent multi-device ML Operations |

*******************End of Change*****************