3GPP SA WG1 Meeting #97e 	S1- 220149

Electronic Meeting, 14 Feb – 24 Feb 2022	(revision of S1-214244)

# Source:	LG Electronics, LG Uplus, OPPO, KRRI, China Unicom, Kyonggi University, Institute for Information Industry (III), Kontron Transportation France, CATT, Orange, SK Telecom, Xiaomi, Sharp, Hyundai Motors, Verizon UK Ltd, Futurewei

# Title:	Feasibility Study on 5G System Support for a Network of Service Robots with Ambient Intelligence

# Document for:	Approval

Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on 5G System Support for a Network of Service Robots with Ambient Intelligence

Acronym: FS_SOBOT

{Propose an acronym. Final acronym to be confirmed at the plenary. The sign "-" is a level separator between (Feature)-(Building Block)-(Work Task). The sign "_" can be freely used. Studies have to start by "FS_". Each acronym level has to be simple and short, 7 characters max recommended}

Unique identifier:	tbd

{A number to be provided by MCC at the plenary}

Potential target Release:	{Rel-19}

{Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan}

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

{Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN}

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

## 2.2	Parent Work Item

{"Parent" Work Item refers to the related, earlier-Stage, Work Item, e.g. the related Stage 1 Work Item shall be indicated here when a Stage 2 normative Work Item or Study Item is presented. "Parent" Work Item can also refer to the related preceding Study Item e.g. the related Study Item and the earlier-stage Work Item shall be indicated here when a normative-work Work Items is started. List here all parent Work Items of which requirements are either fully or partially covered by the proposed Item. }

{This section is mandatory to be filled out by the rapporteur. This section is to be filled with care: it indicates to the companies monitoring the parent Work Item that it will be addressed in this study/work item.}

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 800049 | 5G_HYPOS | Normative work outcome summarized in 22.261 |
| 840041 | eCAV | Normative work outcome summarized in 22.261, 22.104 |
| 840039 | EAV | Normative work outcome on UAS in 22.125 |
| 840031 | VIAPA | Normative work outcome on CMED and A/V Service Production 22.263 |
| 860009 | FS_AMMT | Ongoing study on AI/ML model transfer |
|  |  | {optional free text} |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency}

# 3	Justification

The advancement of robotics application technology would bring more business opportunity in telecommunication market segments through interdisciplinary and cross-industry collaborations. Some critical communication aspect of industrial robots in the context of cyber-physical control systems has been studied so that important use cases, including those with human-machine interface (HMI), can properly be supported with a higher level of communication availability, reliability, clock synchronization and so on. As a result, the related requirements have been identified in three typical traffic classes or communication patterns in industrial environments (refer to 3GPP TS 22.104).

On the other hand, there is a growing demand in consumer electronics segments that expects a great deal of roles that service-oriented robots (or service robots) should play in order to improve the level/quality of a human user’s daily behaviours for, such as shopping, traveling and more to come upon us resulting from smart-living innovations. Some examples of service-oriented robots potentially include:

- serving robot that is designed to deliver food and beverage to residents of Continuing Care Retirement Community (CCRC), guests of hotels and visitors to airport lounges quickly and efficiently;

- porter robot that is designed to help minimize inconvenience for travellers by reducing slow service and long wait times. This robot can also facilitate express check-in and check-out service by handling payment and delivering luggage to a waiting vehicle in a fraction of the time;

- shopping cart robot that is designed to help customers get necessary information and get “hands free” while shopping

The characteristics and required roles of service robots to play are, in general, different from those of industrial robots:

- Application area: service robots are intended to assist humans in various settings (as described in ISO 8373:2012) whereas industrial robots are to replacing human workforce in structured settings, such as manufacturing

- Interacting points: service robots have interactions with human, capable of understanding natural forms of human input (e.g., natural language, gestures, facial expressions) whereas industrial robots have more standardized and structured way of interactions with human workers in job site

- Business opportunity: service robots are designed to play roles in such usage scenarios that are not properly assisted by the use of industrial robots, such as shopping assistance, care-giving, and indoor and local outdoor delivery

- Technology readiness: apart from some basic roles that service robots can play, there exist challenging and promising areas that technology will need to catch up, in order to improve the service quality that service robots can provide, such as AI-native operation, zero-touch operation so that ideally human customers do not need to anything even when disruption happens in service robot operations

The operational models of service robot(s) include: (1) in an individual operation model, a single service robot is used for certain task (2) in a group operation model, a group/family of service robots work together for certain task, often referred to as a multi-agent scenario/model. The group operation model (or multi-agent model) is more advanced than the former model and thus has challenging requirements, which is the main focus of this study. Within the group operation model where a family of service robots work together, there are two modes of collaboration: (1) when the communication channel/link condition is not good enough and, as a result, some service robots out of the family/group cannot share the necessary information in time, they should make decisions with limited information, which is labelled as competitive mode. Each service robot in the group/family should go for a game-theoretic decision making process (typically, zero-sum game). (2) When the communication channel/link condition is good enough (not only at a certain epoch but continuously during their operation), the group/family of service robots can share necessary information fully and they could make a better coordination in strategy planning and can be more productive or efficient than in the previous case. Each service robot should go for a game-theoretic decision making process but, different from the previous case, the group/family can achieve better performance (typically, positive-sum game in this cooperative mode, namely, via the communication, their own individual decisions get better). Therefore, it is critically important to provide sufficient level of communication availability in the group operation model; if not (temporarily or not), it is critically important to provide a predictive means from communications layer suitable for the group operational model of service robots so that the group/family of service robots can maintain high level of performance/productivity/efficiency by avoiding or minimizing disruption of service robot operations. .

Example use case scenarios for cloud-based/cloud-assisted operations of service robots include, but is not limited to, indoor delivery robots, local outdoor delivery robots and disinfection robots via using edge clouds. Various scenarios and economic needs exist. Due to the nature of their roles of assisting human in some use case scenarios (e.g., delivery robots), some event-related information (e.g., accident, robot/sensor breakdown that have potential to contribute to or have contributed to service disruption, and information that has to rather be sent out of such a problematic situation that is predicted to happen or has already happened) needs to be share with the robot operator/server and/or with participating robots within certain time interval whenever such an event is predicted or any precursory indication becomes available (time-bounded communication).

Given the interesting scenarios and requirements involved, it is proposed to study potential new service requirements for the 3GPP 5G system to support service robots communication.

NOTE: Some definitions and/or terminology, which are considered necessary to properly describe the pre-condition, service flow and post-condition, will be aligned with ISO 8373:2012.

# 4	Objective

The scenarios that will be reviewed and studied to derive potential service and performance requirements include group operation models (or multi-agent models) where multiple service robots work together for a single task.

NOTE 1: The definition of service robot follows the definition that has been used in ISO. However, an enhanced definition can also be considered if appropriate for 3GPP study and work.

NOTE 2: Two modes of collaboration are considered to analyse the necessary enhancement of communications feature and performance: no or limited sharing of information among service robots due to limited network resources or bad network conditions (competitive mode) and maximum sharing of information (fully collaborative mode) that can differently affect the need of communication functional and performance requirements.

NOTE 3: This study does not intend to work on analysing scenarios where multiple service robots work together for multiple distinct tasks (although not explicitly precluded).

The objective of this study is to identify use cases and the related potential service and performance requirements for 5G system enhancement that can provide highly reliable, effective and efficient communications service for a group/family of service robots that act as a UE, Relay UE, or both.

  - Enhanced support of on-demand priority communications that are necessary to help avoid or minimize disruptions of service robot operation

    - Exposure to application-layer information that becomes available to communications layer (e.g.,)

NOTE 4: Application-layer information includes: static information on sensor failure, validity of multi-sensory information (e.g., instead of deterministic trend information on failure, stochastic trend information on failure or validity, e.g., x% of failure is estimated).

    - Capability of communications layer to make necessary changes with high priority to help avoid or minimize disruptions that may be caused in the application layer

NOTE 5: 5GS communication path needs to be re-configured very quickly (e.g., Uu to PC5, change PC5 link)..

  - Enhanced support of time bounded communication to help an event-triggered delivery of information/data between multiple service robots (including KPIs related to access delay)

NOTE 6:  3GPP system is expected to be enhanced to support such an event-triggered delivery of information/data in an ultra-timely manner when predictive information on critical communication interruption becomes available to communications layer. Information/data may include robot task record, forensic data, other collected metadata and so on (that are just payload for 3GPP and is out of the scope of 3GPP). 5GS enhancement towards such a capability is aimed at enabling so-called zero-blackbox operation for group operation models of service robots even when critical and extreme situations happen, e.g., loss of physical entity of a robot in search and rescue, hazardous control, and online hijacking scenarios.

NOTE 7: For example, 5GS can be enhanced so that the communications layer can take ultra-responsive preparation to help delivery of information/data that service robots had collected which was not sent to others. Through exposure capability, 5GS can use the causes of some predicted events, such as severe crash, failure, breakdown and deadlock in the group operation model (which multi-sensory operation at the application layer can provide for the communications layer), getting out of synchronization (due to various factors), stochastic trend of synchronization failures, and so on.

NOTE 8: In the group operation model where multiple service robots are collaboratively building up a single set of online fine-grained metadata (e.g., collaborative online high-definition 3D map building), it is necessary that the communications layer should provide in-time service (absolute difference) and a very high level of synchronization accuracy among those service robots working together.

  - Support of service operations aspects, including avoiding or minimizing service disruption due to employing new technology that serves multiple purposes using a common frequency band (e.g., usage scenarios of integrated communication and sensing service that can affect service disruption of cloud-based robot services, trade-off between temporary congestion for traffic and temporary degradation of surface sensing performance for service robots).

NOTE 9: Due to the inherent nature of service robot operations and their related scenarios, the use of sensing and impact on communication disruptions/congestion may be described in service flow and descriptions during the study but if sensing specific requirements are identified, they will be consolidated into Sensing Study outcome (in one place).

  - This study will consider high level service scenarios that have security and privacy implications for resilient operations of a network of service robots.

NOTE 7: Potential threat cause factors in the presence of advanced multi-sensory robot systems (e.g., social engineering attacks, misuse of overheard acoustic signals) will be considered for possible derivation of security/privacy guidelines and/or high-level security and privacy related requirements.

Among various domains of service robots, this study will focus on the support for usage scenarios, such as how to improve the quality of ordinary living (e.g., care-giving) and quality of critical roles (e.g., smart local delivery, highly interactive controls under hazardous conditions) , that can be used against challenges that arise from aging population and/or pandemic situations.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| "Internal TR" | 22.XXX | Study on 5G System Support for a Network of Service Robots with Ambient Intelligence | TSG#96 Dec. 2022 | TSG#97 Mar. 2023 | LEE, Ki-Dong (kidong.lee@lge.com), LG Electronics |

{Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {e.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {e.g. "TSG#89"} | {Free text} |

# 6	Work item Rapporteur(s)

LEE, Ki-Dong, kidong.lee@lge.com, LG Electronics

# 7	Work item leadership

SA1

# 8	Aspects that involve other WGs

# 9	Supporting Individual Members

Supporting IM name

LG Electronics

LG Uplus

OPPO

Korea Railroad Research Institute (KRRI)

China Unicom

Kyonggi University

Institute for Information Industry (III)

Kontron Transportation France

CATT

Orange

SK Telecom

Xiaomi

Sharp

Hyundai Motors

Verizon UK Ltd

Futurewei

KT [?]

Hansung University [?]

Tencent [?]

Spreadtrum [?]