3GPP TSG-SA1 Meeting #97e	S1-220169

Electronic Meeting, 14 – 24 February 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0639 | Rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Usage of Non-3GPP DTT Broadcast Networks for Multicast/Broadcast Services in 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Saankhya Labs, IIT Bombay, Hewlett-Packard Enterprise, Ligado Networks, One Media 3.0, Fraunhofer IIS, CEWiT, Tejas Networks, IIT Kanpur, IIT Madras, IIT Hyderabad, IIT Kharagpur |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-02-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The proliferation of mobile broadband networks is increasingly making the devices like smartphones as the preferred choice for content consumption. However, with the growth in content consumption, especially video consumption via cellular networks, the load on the available cellular spectrum has also increased. This leads to congestion in cellular networks. The increased usage of video consumption along with a host of new services such as software delivery over wireless networks, group communications, and broadcast/multicast IoT applications has created a need for their delivery via multicast/broadcast services in the 5G system. In order to support these requirements and reduce the congestion in cellular networks, the 5GS plans to support multicast broadcast services, as specified in TS 22.261. Though TS 22.261 identifies a set of technologies to be used for multicast broadcast services within 5GS, non-3GPP Digital Terrestrial Broadcasting Technology has not been identified currently as a technology for delivery of multicast broadcast services within 5GS. However, as highlighted below, the usage of non-3GPP Digital Terrestrial Broadcast Technology networks (non-3GPP DTT networks) for multicast broadcast service within 5GS may bring in many advantages: - The non-3GPP digital terrestrial broadcast technologies are purpose-built for optimum broadcast performance and provide support for large coverage areas, which would be helpful for service delivery in rural and sparsely populated areas. - Many non-3GPP DTTs provide high spectral efficiency supporting higher-order modulation & coding schemes, e.g., Advanced Television Systems Committee 3.0 (ATSC 3) standard supports 4096-QAM. - They support improved mobility management by virtue of having a time-interleaver, support for SFNs, and the ability to cater to large coverage areas. - These technologies are already deployed in many countries globally, e.g., ATSC3 in the USA, Korea, DVB-T2 in 144 countries, ISDB-T in 24 countries, and therefore their (non-3GPP DTTs) usage in conjunction with 5GS may unlock new business opportunities. Therefore, it is proposed to allow the usage of non-3GPP DTT networks (legacy as well as new) for multicast broadcast services within 5GS through interworking of non-3GPP DTT with 5GS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Stage 1 specification to be updated to allow usage of non-3GPP DTT networks for multicast/broadcast services in 5GS through interworking of non-3GPP DTT with 5GS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may not be possible to leverage non-3GPP DTT networks for multicast broadcast service delivery within the 5G system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 and 6.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.3.2.5	Non-3GPP Digital Terrestrial Broadcast access

The 5G system shall be able to efficiently support delivery of multicast broadcast services using non-3GPP digital terrestrial broadcast access network.

NOTE:	The specification of non-3GPP digital terrestrial broadcast access network is outside the scope of 3GPP.

### 6.13.2	Requirements

The following set of requirements complement the requirements listed in 3GPP TS 22.146 [7], TS 22.246 [8] and TS 22.101 [6], clause 32.

The 5G system shall support operation of downlink only broadcast/multicast over a specific geographic area (e.g. a cell sector, a cell or a group of cells).

The 5G system shall support operation of a downlink only broadcast/multicast system over a wide geographic area in a spectrally efficient manner for stationary and mobile UEs.

The 5G system shall enable the operator to reserve 0% to 100% of radio resources of one or more radio carriers for the delivery of broadcast/multicast content.

The 5G network shall allow the UE to receive content via a broadcast/multicast radio carrier while a concurrent data session is ongoing over another radio carrier.

The 5G system shall be able to support broadcast/multicast of UHD streaming video (e.g. 4K/8K UHD).

NOTE 1:	Taking into account the bandwidth needs for different streaming video resolution.

The 5G network shall allow the operator to configure and broadcast multiple quality levels (i.e. video resolutions) of broadcast/multicast content for the same user service in a stand-alone 3GPP based broadcast/multicast system.

The 5G network shall support parallel transfer of multiple quality levels (i.e. video resolutions) of broadcast/multicast content for the same user service to the same UE taking into account e.g. UE capability, radio characteristics, application information.

The 5G system shall support parallel transfer of multiple multicast/broadcast user services to a UE.

The 5G system shall support a stand-alone multicast/broadcast network comprising of multiple cells with inter-site distances of up to 200 km.

The 5G system shall support multicast/broadcast via a 5G satellite access network, or via a combination of a 5G satellite access network and other 5G access networks.

The 5G system shall support multicast/broadcast via a non-3GPP digital terrestrial broadcast network, or via a combination of a non-3GPP digital terrestrial broadcast network and other 5G access networks.

The 5G system shall be able to setup or modify a broadcast/multicast service area within [1s].

NOTE 2: For MCPTT related KPIs see 3GPP TS 22.179 [30], clause 6.15.

The 5G system shall be able to apply QoS, priority and pre-emption to a broadcast/multicast service area.

The 5G system shall support downlink parallel transfer of the same content, via broadcast/multicast and/or unicast, such that all receiver group members in a given area receive the media at the same time according to user perception.

NOTE 3: In this context user perception refers to a difference in delay of typically less than 20 ms.

The 5G system shall support a mechanism to inform a media source of relevant changes in conditions in the system (e.g. capacity, failures).

The 5G system shall provide means for a media source to provide QoS requirement requests to the broadcast/multicast service.

The 5G system shall provide means for the broadcast/multicast service to inform the media source of the available QoS, including modification of available QoS characteristics and availability of the broadcast/multicast service.

The 5G system shall be able to support broadcast/multicast of voice, data and video group communication, allowing at least 800 concurrently operating groups per geographic area.

NOTE 4: In this context "concurrently operating groups" means that the associated media streams are delivered concurrently.

The 5G system shall support delivery of the same UE-originated data in a resource-efficient manner in terms of service bit rate to UEs distributed over a large geographical area.

The 5G system shall allow a UE to request a communication service to simultaneously send data to different groups of UEs at the same time.

The 5G system shall allow different QoS policy for each group the UE communicates with.