3GPP SA WG1 Meeting #97e 	S1-220031

Electronic Meeting, 14 - 24 Feb 2022	(revision of xx-yyxxxx)

# Source:	vivo

# Title:	New SID on Personal IoT Networks in roaming scenarios

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Personal IoT Networks in roaming

Acronym:	FS_PINroaming

Unique identifier:

Potential target Release:	Rel19

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 880041 | Study on Personal IoT Networks | Previous Rel-18 work, some basic requirements related to PIN have been specified. |

3	Justification

SA1 has studied Personal IoT Networks(PIN) in release 18, which mainly focused on the smart home/office scenarios and non-roaming scenarios. Supporting PIN in the roaming scenario has not been thoroughly studied.

In brief, there are mainly two major categories of roaming use cases for PIN:

1	all PIN elements are in roaming scenarios, for example:

Use case 1): My parents and I travel abroad. Only I have turned on roaming service, and my parents have no roaming service. My parents can obtain data service directly from my roaming PLMN after joining my PIN during the trip with roaming service off. My parents and I are not together all the time during the trip and my parents need to receive data services directly from my roaming PLMN sometimes (for example, when direct link between my parents’ mobile phone and PIN gateway (i.e. my mobile phone) is not available, my parents cannot get data service via my PIN gateway all the time).

Charge is on my phone using flat fee, for example:

-	60 RMB per day while I travel abroad using the above PIN roaming service;

-	30 RMB per day while I travel abroad with the legacy roaming service.

Use case 2): Alice travels abroad, meet some a new friend Bob in the same tour group, Bob is travelling abroad. Bob and Alice may be subscribers of different PLMNs.

Only Alice has turned on roaming service, Bob have no roaming service. Bob can obtain data service directly from Alice’s roaming PLMN with roaming service off after joining Alice’s PIN. Alice and Bob are not together all the time during the trip and Bob needs to receive data services directly from my roaming PLMN sometimes (for example, when direct link between Bob’s mobile phone and PIN gateway (i.e. my mobile phone) is not available, Bob cannot get data service via my PIN gateway all the time). Charge is on Alice’s phone using flat fee.

2	PIN element A is in roaming and PIN element B is not roaming:

Use case 3): Alice’s data roaming service has been turned off. Alice travel abroad and meet a local friend Bob there.

Under the authorization of Alice’s operator and Bob’s operator, with the assistance and Bob’s phone, Alice could obtain data service directly from the Bob’s operator with the data roaming service off when Alice is with Bob and not with Bob, meanwhile Alice could be reached via the home PLMN MSISDN.

Use case 4): Alice travel abroad and rent a car there.

Under the authorization of Alice’s operator and the car’s operator, Alice could obtain data service directly from the car’s operator when Alice is in the car and out of the car (when direct link between Alice’s mobile phone and PIN gateway (i.e. the car) is not available, Alice cannot get data service via the PIN gateway (i.e. the car) all the time) with the data roaming service off, meanwhile Alice could be reached via the home PLMN MSISDN, so that Alice would not miss the call from family or credit card centre.

When they drive into a landscape that lack of coverage, satellite access can be utilized.

-	The car is able to access satellite while Alice’s phone can’t access satellite: how to enhance IAB backhaul link using satellite access.

-	The car and Alice’s phone are both able to access satellite: the car could share the Ephemeris information to facilitate the UE selecting the proper satellite access.

Note: 	Devices in Roaming country (i.e. PIN element B) could be a mobile phone owned by local friends，local vehicles with USIM issued by roaming country, and any other local UE.

Gap analysis:

-	The Rel18 PIN only studies credential provisioning for a device without any preconfigured credential and the PIN element obtain the data service via PIN gateway or directly from PIN element’s home PLMN.

-	when the PIN element is out of coverage of its home PLMN coverage(operator A), the PIN element has the credential issued by operator A. The PIN element can not be reached via MSISDN A associated with the credential issued by operator A when obtaining data service using credential provisioned by operator B.

All in all, it is useful to study the above use cases so that the 5GS could provide better roaming service experience for users to travel with PIN.

4	Objective

This study is aiming at identifying new use cases, providing gap analysis and defining potential requirements regarding 5GS support of Personal IoT networks (PIN) enhanced in roaming scenarios.

The objectives include:

-	Study use cases related to enhance 5GS to support Personal IoT networks (PIN in roaming scenarios, including but not limited to:

- 	PIN element A with USIM issued by operator A obtains services directly from operator B with roaming service off when PIN element A and PIN element B are both roaming in operator B;

- 	PIN element A with USIM issued by operator A obtains services directly from operator B with roaming service off when PIN element A is roaming in operator B and PIN element B is not roaming (i.e. PIN element B has USIM issued by operator B);

-	Study potential requirements related to enhance 5GS to support Personal IoT networks (PIN) in roaming scenarios, including:

-		authentication and authorization of the PIN in roaming scenarios;

-	How the UE can be reached via the home MSISDN while the UE obtains data service directly from the PLMN other than Home PLMN with the data roaming service off; and

-	Facilitate satellite access as backhaul for PIN and share satellite access information between PIN gateway and PIN devices in one PIN; and

-	Other aspects include security, charging and privacy; and

-	Gap analysis between the identified requirements and what is already defined by existing 3GPP requirements.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | TR22.xxx | To study the use cases and potential service requirements for identifying | SA#97 (Sept 2022) | SA#98 (Dec 2022) | Yanchao Kang |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {e.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {e.g. "TSG#89"} | {Free text} |

6	Work item Rapporteur(s)

Yanchao Kang, vivo, kangyanchao@vivo.com

7	Work item leadership

SA1

8	Aspects that involve other WGs

none

9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

vivo

OPPO

Toyota