3GPP SA WG1 Meeting #97e 	S1-220031

Electronic Meeting, 14 - 24 Feb 2022	(revision of xx-yyxxxx)

# Source:	vivo

# Title:	New SID on Personal IoT Networks in roaming scenarios

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Personal IoT Networks in roaming

Acronym:	FS_PINroaming

Unique identifier:

Potential target Release:	Rel19

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 880041 | Study on Personal IoT Networks | Previous Rel-18 work, some basic requirements related to PIN have been specified. |

3	Justification

SA1 has studied Personal IoT Networks(PIN) in release 18, which mainly focused on the smart home/office scenarios and non-roaming scenarios. Supporting PIN in the roaming scenario has not been thoroughly studied.

In brief, there are mainly two major categories of roaming use cases for PIN:

1	all PIN elements are in roaming scenarios, for example:

Use case 1): My parents and I travel abroad. Only I have turned on roaming service, and my parents have no roaming service. My parents can obtain data service directly from my roaming PLMN after joining my PIN during the trip with roaming service off. My parents and I are not together all the time during the trip and my parents need to receive data services directly from my roaming PLMN sometimes (for example, when direct link between my parents’ mobile phone and PIN gateway (i.e. my mobile phone) is not available, my parents cannot get data service via my PIN gateway all the time).

Charge is on my phone using flat fee, for example:

-	60 RMB per day while I travel abroad using the above PIN roaming service;

-	30 RMB per day while I travel abroad with the legacy roaming service.

Use case 2): Alice travels abroad, meet some a new friend Bob in the same tour group, Bob is travelling abroad. Bob and Alice may be subscribers of different PLMNs.

Only Alice has turned on roaming service, Bob has no roaming service. After joining Alice’s PIN, Bob can obtain data service indirectly from Alice’s roaming PLMN using his watch or his tablet via PEGC, i.e. Alice’s phone. Meanwhile, Bob could be reached via his MSISDN on the watch or tablet.

2	PIN element A is in roaming and PIN element B is not roaming:

Use case 3): Alice’s data roaming service has been turned off. Alice travel abroad and meet a local friend Bob there.

Under the authorization of Alice’s operator and Bob’s operator, with the assistance and Bob’s phone, Alice could obtain data service indirectly from the Bob’s operator using her watch or her tablet via PEGC, i.e. Bob’s phone, meanwhile Alice could be reached via the her MSISDN on the watch or tablet.

Use case 4): Alice travel abroad and rent a car there.

Under the authorization of Alice’s operator and the car’s operator, Alice could obtain data service indirectly from the car’s operator using her watch or her tablet via PEGC, i.e. the car. Meanwhile Alice could be reached via her MSISDN on the watch or tablet, so that Alice would not miss the call from family or credit card centre.

Furthermore, the user could benefit more when PIN elements could facilitate the satellite access. For example, when Alice rent a car for sightseeing tour.

When Alice drives into a landscape that lack of coverage, satellite access can be utilized:

-	The car is able to access satellite while Alice’s phone can’t access satellite: how to enhance IAB backhaul link using satellite access.

-	The car and Alice’s phone are both able to access satellite: the car could share the Ephemeris information to facilitate the UE selecting the proper satellite access.

Gap analysis:

-	The Rel18 PIN only studies credential provisioning for a device without any preconfigured credential and thus the PIN element obtain the data service via PIN gateway.

-	when the PIN element is out of coverage of its home PLMN coverage, the user could get the data service via PEGC in roaming PLMN using his or her watch or tablet. However the user can not be reached via the user’s MSISDN when the user turn off the roaming service.

All in all, it is useful to study the above use cases so that the 5GS could provide better roaming service experience for users to travel with PIN.

4	Objective

This study is aiming at identifying new use cases, providing gap analysis and defining potential requirements regarding 5GS support of Personal IoT networks (PIN) enhanced in roaming scenarios.

The objectives include:

-	Study use cases related to enhance 5GS to support Personal IoT networks (PIN in roaming scenarios, including but not limited to:

- 	PIN element of user A obtains data services indirectly from operator B via UE of user B when user A is roaming in operator B with roaming service off, meanwhile PIN element of user A can be reached by MSISND of user A;

-	user B are roaming in operator B; or

-	user B is not roaming,

-	Study potential requirements related to enhance 5GS to support Personal IoT networks (PIN) in roaming scenarios, including:

-	authentication and authorization of the PIN in roaming scenarios;

-	How the PIN element of user A can be reached via the MSISDN of user A while the PIN element of user A obtains data service indirectly from the PLMN other than Home PLMN when user A turns off roaming service; and

-	Facilitate satellite access as backhaul for PIN and share satellite access information between PIN gateway and PIN devices in one PIN; and

-	Other aspects include security, charging and privacy; and

-	Gap analysis between the identified requirements and what is already defined by existing 3GPP requirements.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | TR22.xxx | To study the use cases and potential service requirements for identifying | SA#97 (Sept 2022) | SA#98 (Dec 2022) | Yanchao Kang |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {e.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {e.g. "TSG#89"} | {Free text} |

6	Work item Rapporteur(s)

Yanchao Kang, vivo, kangyanchao@vivo.com

7	Work item leadership

SA1

8	Aspects that involve other WGs

none

9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

vivo

OPPO

Toyota