3GPP SA WG1 Meeting #97-e 	S1-220039r2

Electronic Meeting, 14 – 24 Feb. 2022

# Source:	OPPO

# Title:	Study on UE-Network coordinated AI/ML Service

# Document for:	Approval

Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Study on UE-Network coordinated AI/ML Service

Acronym: FS_UNAS

Unique identifier:

Potential target Release: Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 920030 | AI/ML model transfer in 5GS | Previous Rel-18 work, some KPIs and basic requirements related to AI/ML model transfer have been specified. |

3	Justification

Following the outcome of the Rel-18 SA1 study on AI/ML model transfer over 5GS, new scenarios and aspects, missing or not fully covered in Rel-18, have been identified and suggested to be further investigated in Rel-19, i.e.:

-	AI/ML model transfer for device-NW coordinated AI/ML operation, e.g., NW-assisted information for predicting model transfer in 5G hotspot coverage, or device route planning

-	AI/ML model transfer over 5G direct device connection, e.g., for distributed learning/inference.

Device-side AI operation is needed in many scenarios, and high data rate connection becomes precondition (e.g. for AI model downloading, AI training with NW) for 5G devices to perform specific AI/ML tasks. On the other hand, high data rate maybe available only in certain hotspots or LOS areas.

Considering, for example, smart-transportation use cases, mobile UEs (Vehicles, Robots, Drones) need to plan the route not only based on the location of destination and transportation map, but also considering the 5G link quality. The UEs could intelligently predict (or move toward) the hotspot/LOS area to access the high data rate for AI services (AI/ML model downloading, Split inference/training), using e,g, AI-based route planning based on the predicted achievable data rate throughout the unvisited area.
To support the above,

some NW-assisted information exposure could be beneficial for the device (or 3rd party transportation company). This may not require to disclose sensitive information about the network or devices. For example, the NW and device can perform split AI route planning, and exchange only intermediate AI/ML inference data.

A second area of further study is

the use of 5G direct device connection to perform distributed learning/inference among devices, achieving the goal of using local data with privacy protection, energy saving, training  efficiency. etc. Potential scenarios are, for example:

- For Distributed Learning, controlled by network, each device uses the localized data while transfer the intermediate data to other nodes in order to train a big model.

- For Distributed Inference, the device, holding valuable input data but with low power/computation resource, may calculate the first few layers of a AI model and offload the intermedia data to another device using D2D link for the final inference result.

- For Federated Learning, Using D2D can involve devices outside 5G hotspot for distributed AI training/inference, partly offload computation to neighboring device, so as to improve the training accuracy/generalization and save device power.

For those D2D scenarios, new 5GS KPIs and  functional requirements should be investigated, for example, different QoS requirement on Uu and slidelink in relay scenario, and taking application, model storage, user consent into consideration for D2D discovery.

It is therefore proposed to study the two areas above, as per detailed objectives decribed in next section.

4	Objective

The objectives are to study new use cases and potential service and performance requirements for identifying UE-NW coordinated functionalities to support more efficient AI/ML operations, including:

-	Device-NW coordinated AI/ML operation, e.g., NW-assisted or NW-guided  UE service traffic (e.g., predicted AI model transfer) in 5G hotspot coverage, or device route planning,

-	Distributed AI training/inference involving direct device connection, e.g. traffic KPIs, different QoS requirement on Uu and slidelink, device discovery (taking application, model storage, user consent into consideration.

  - Note: distributed AI training/inference is assumed to be controlled by the network.

-      Charging and security aspects.

Note: The study should avoid overlaps with stage-2/3 work ongoing in Rel-18.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TR22.xxx | To study the use cases and potential service and performance requirements for identifying coordinated functionalities to support more efficient AI/ML operations. | TSG #98 |  |

6	Work item Rapporteur(s)

Jia Shen, OPPO, sj@oppo.com

7	Work item leadership

SA1

8	Aspects that involve other WGs

None

9	Supporting Individual Members

Supporting IM name

OPPO

Qualcomm

China Telecom

Xiaomi

ZTE

China Mobile

vivo

Tencent