3GPP TSG SA WG 1 Meeting 97e		 S1-220083r1

Electronic Meeting, 14 – 24 February 2022

Title:	[Draft] Reply LS on MINT and Higher priority PLMN Selection

Response to:	LS C1-220816

Release:	Release 17

Work Item:	MINT

Source:	SA1

To:	CT1

Cc:

Contact person:	Erik Guttman

erik.guttman@samsung.com

+49 172 916 6662

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA1 thanks CT1 for the question on MINT and Higher priority PLMN Selection.

CT1 asked 3 questions:

Question 1) Is the UE allowed or even mandated to scan for all available PLMNs in order to obtain service on an allowable PLMN when registered for disaster roaming service?

Answer to question 1) Yes, the UE is allowed to scan for all available PLMNs in order to obtain service on an allowable PLMN when registered for disaster roaming service.

Question 2) Is the UE allowed or even mandated to attempt service on such other allowable PLMNs when registered for disaster roaming service or shall the UE continue to use disaster roaming service on the FPLMN?

Answer to question 2) Yes, the UE is allowed to scan for all available PLMNs in order to obtain service on an allowable PLMN when registered for disaster roaming service.

Question 3) The interval of time between searches for a higher priority PLMN is configured by HPLMN. Can SA1 kindly provide the reasoning whether this interval of time needs be different/adjusted for the UE in disaster roaming condition?

Answer to question 3) Further details, and possible changes, on the interval of time needed for PLMN scanning should be better discussed in CT1.

# 2	Actions

To 3GPP CT1

ACTION:  SA1 asks that CT1 take the answer provided above into account.

# 3	Dates of next TSG SA WG 1 meetings

SA1#98e	09-19 May 2022			Electronic Meeting

SA1#99e	22-26 August 2022 or TBD	Goteborg, SE or Electronic Meeting