3GPP TSG-SA WG1 Meeting #97e 	S1-220105r4

Electronic Meeting, 14 – 24 Feb. 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0633 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of KPI in TS22.261 clause 7.10 for AMMT use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Kyonggi University |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMMT |  |  |  |  |  | Date: |  |  | 2022-02-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Editorial correction |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the UL data rate to the correct box |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect place for a KPI value |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** Start of Change ******************

7.10	KPIs for AI/ML model transfer in 5GS

The 5G system shall support split AI/ML inference between UE and Network Server/Application function with performance requirements as given in Table 7.10-1.

Table 7.10-1 KPI Table of split AI/ML inference between UE and Network Server/Application function

| # Uplink KPI |  |  |  |  | # Downlink KPI |  |  |  | # Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Max allowed UL end-to-end latency | # Experienced data rate | # Payload size | # Communication service availability | # Reliability | # Max allowed DL end-to-end latency | # Experienced data rate | # Payload size | # Reliability |  |
| 2 ms | 1.08 Gbit/s | # 0.27 MByte | # 99.999 % | # 99.9 % |  |  |  | # 99.999 % | # Split AI/ML image recognition |
| 100 ms | 1.5 Mbit/s |  |  |  | # 100 ms | # 150 Mbit/s | 1.5 MByte/‌frame |  | # Enhanced media recognition |
|  | 4.7 Mbit/s |  |  |  | 12 ms | 320 Mbit/s | 40 kByte |  | Split control for robotics |
| NOTE 1: Communication service availability relates to the service interfaces, and reliability relates to a given system entity. One or more retransmissions of network layer packets may take place in order to satisfy the reliability requirement. |  |  |  |  |  |  |  |  |  |

*******************End of Change*****************