3GPP SA WG1 Meeting #97e 	S1-220141r2

Electronic Meeting, 14 Feb –24 Feb 2022	(revision of S1-214241)

# Source:	China Mobile

# Title:	New SID on Additional capabilities of mobile networks for drone operations and management

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:  Additional capabilities of mobile networks for drone operations and management

Acronym: FS_DOM

Unique identifier:

Potential target Release:	{Rel-19}

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820011 | Study on supporting Unmanned Aerial Systems Connectivity, Identification, and Tracking | R17 study analysed and concluded on UAV connectivity, identification, and tracking. |
| 900014 | (Stage 2 of) Support of Unmanned Aerial Systems Connectivity, Identification, and Tracking | R17 SA2 WID on the system level support of UAV authentication by UTM and UAV tracking. |
| 820026 | Application layer support for Unmanned Aerial System (UAS); Functional architecture and information flows | R17 SA6 WID on the Application level support for UAS about functional architecture and information flows. |

3	Justification

5G mobile networks are well suited to support low-altitude drone communication and to be integrated with drone traffic management systems, in order to enhance the safety and security of drone operations. In some industrial UAV scenarios, some new functions and capabilities of 5GS can be applied to UAV flight management and application. There have already been some topics focusing on the support of UAV application in 3GPP, for example, TS 23.256: Support of Unmanned Aerial Systems (UAS); connectivity, identification and tracking; TS 23.255: Application layer support for Unmanned Aerial System (UAS); Functional architecture and information flows. However, there has not proposed any requirements related to additional capabilities of mobile networks for drone operations and management as listed below.

-	5G network can provide functionality or capabilities (e.g, flight mission application, flight monitoring) to support UTM functionality and to collect detailed information about UAS. The network can utilize detailed location information resource to optimize the UAV fight path scheduling based on UAV tracking and propose flight path optimization to the UTM. The information may be based on location tracking of the UAV and additional information potentially provided by the UAV, such as the parameter ’flightPathInfoReport’ in TS 36.331 that will be reported by UAV to eNB. Mobile network can utilize these location information resource to optimize the UAV fight path scheduling based on UAV tracking and provide it to the UTM. The analysis for UAV trajectory can be predicted by the 5GS via aggregating and analysing different types of data in 5GS.

-	5G network can provide wide-area, high-quality and secure connectivity that can enable cost-efficient drone operations beyond visual line-of-sight (BVLOS) range. When UAV use cellular network as the access type, the UTM/drone operator may need to acknowledge the network status, A mechanism is needed to assist UTM/drone operator to determine whether the UAV can access the network, or if already accessed, to continue the flight operation, according to the network capacity and load condition on the flight path. ACJA has defined a set of mechanisms in “ACJA Interface for Data Exchange between MNOs and the UTM Ecosystem: NetworkCoverage Service Definition v1.00” (https://www.gsma.com/iot/wp-content/uploads/2021/02/ACJA-NetworkCoverage-Service-Definition-v1.00.pdf) for interfacing the UAV traffic management systems with the MNO network to collect additional data from connectivity providers. However, system level requirements for provisioning of such and additional information have not been defined.

-	During long flight missions, there may be the need to change the flight path of a UAV with respect to the initially planned and authorized path, or several handovers may occur, or the QoS guarantees may change during the flight, UAV service monitoring in mobile network during long flight missions with changing conditions needs further discussion, to fulfill UAV special service quality requirement.

-	Traditional Detect and Avoid (DAA) mechanism are based on vehicle to vehicle communications. However, traditional aerial vehicles are not supported by a wide-area communication network like a 3GPP network. Compared with the traditional mechanism of Detect and Avoid (DAA) between UAVs that are not connected to a mobile network, DAA for networked 3GPP UAVs can also be supported by the mobile network, e.g., collision warning may be notified by the 5GS, and requirements for the support of a network-assisted or ground-based DAA should be analyzed.

-	The aviation industry (e.g. ACJA) is analysing use cases and defining MOPS (Minimum Operating Performance Requirements) for simultaneous connectivity of a UAV to multiple PLMNs, as a way to achieve better resilience and robustness for critical communications in BVLOS, e.g. communication, command and control (C3) or command and control (C2) connectivity. In 3GPP, the support of simultaneous connectivity to multiple PLMNs is being considered under various areas of work. However, specific requirements for the simultaneous connectivity of a UAV to multiple PLMNs to support reliability and redundancy of critical communications have not been defined yet.

-	Mobile networks are being enhanced with edge solutions. An MNO network has a widespread coverage and have cell towers located over the whole network and close to where UAVs will be flying. Enhancing information collection (e.g. local UAV awareness, micro-weather awareness, etc.) by the MNO and distributing such information to UAVs by leveraging edge services would enhance the set of services that the MNO can provide to UAVs, UAV operators, and the UTM infrastructure.

Hence, 3GPP SA1 should take the responsibility and start the work to explore the further interaction between 5GS and UAV system and specify the requirements on above additional capabilities of mobile networks for drone operations and management.

Note: this study would provide additional capabilities of 5GS to assist UTM, but liability/legal responsibility for drones operation stays with UTM/drone operator.

4	Objective

The objectives of this work item are to identify additional use cases to meet industrial UAV applications in order to derive additional requirements for mobile networks to support drone operations and drone management .

Study use cases and identify potential requirements to:

- Provide additional information to the UAV operator/USS to execute pre-flight preparations and inflight operation (e.g, flight mission application, flight monitoring and control);

- Use the 5G system to enhance the UAV flight management with network capacity and QoS information along the planned the flight path;

- Use of 5G system to enhance the safety and security of drone operations, such as support of network-assisted or ground-based Detect and Avoid (DAA) between networked 3GPP UAVs;

- -

- Support of simultaneous connectivity to multiple PLMNs for redundancy and reliability of critical communications (e.g. C2), e.g. in order to utilize the most suitable PLMN depending on configuration and dynamic conditions;

-  Identify security, charging and regulatory requirements on 5GS when used for UAS operation.

-  A gap analysis will be performed between potential new service requirements and identified KPIs and existing requirements and functionalities, to identify any new needs to support the identified use cases by 3GPP.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| “Internal TR” | 22.XXX | Study on on Additional capabilities of mobile networks for drone operations and managements | (Dec 2022) | (Mar 2023) |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

Pengtai Qin , China Mobile, qinpengtai@chinamobile.com

7	Work item leadership

SA1

8	Aspects that involve other WGs

None

9	Supporting Individual Members

Supporting IM name

China Mobile

Orange

Qualcomm

AT&T

vivo

ZTE

InterDigital

Futurewei