The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP TSG-SA WG1 Meeting #98e 	S1-221078

Electronic Meeting, 9 – 19 May 2022	(revision of S1-21xxxx)

Title:	TR Skeleton of New SID on supporting computing aware network

Agenda Item:	4

Source:	China Mobile

Contact:	Xiaonan Shi, shixiaonan@chinamobile.com

Abstract:  this doc proposes the TR skeleton for FS_CAN.

3GPP TR 22.XXX V0.0.0 (2022-02)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Study on Supporting Computing Aware Network

(Release 19)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

computing aware network

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	6

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview	7

5	Use cases	8

5.1	Use case 1	8

5.1.1	Description	8

5.1.2	Pre-conditions	8

5.1.3	Service Flows	8

5.1.4	Post-conditions	8

5.1.5	Existing features partly or fully covering the use case functionality	8

5.1.6	Potential New Requirements needed to support the use case	8

6 	Consolidated potential requirements	8

7	Conclusion and recommendations	8

Annex <A>: <Annex title>	9

A.1	x	9

Annex <X>: Change history	10

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

Introduction

1	Scope

The present document …

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

4	Overview

5	Use cases

5.1	Use case 1

5.1.1	Description

<Describe what the use case intends to achieve.>

5.1.2	Pre-conditions

<List any pre-conditions that need to exist for this use case, preferably as a bulleted list, e.g. UE is registered to the network.>

5.1.3	Service Flows

<Describe the sequence of events that explain what needs to happen, preferably as a numbered list, e.g. 1. User makes a voice call, 2. Called party receives alerting message.>

5.1.4	Post-conditions

<Describe the end result e.g. Called party can decide whether to accept call based on information displayed on UE screen.>

5.1.5	Existing features partly or fully covering the use case functionality

< Highlight existing features in the existing set of normative specifications that partly or fully cover this use case.>

5.1.6	Potential New Requirements needed to support the use case

<Provide draft new requirements that are needed to realise the use case, and that are not yet covered in any normative specification.>

6 	Consolidated potential requirements

7	Conclusion and recommendations

Annex <A>:
<Annex title>

A.1	x

Heading levels within an annex are used as in the main document, but for Heading level selection, the "A.", "B.", etc. are ignored. e.g. A.1.2 is formatted using Heading 2 style.

Annex <X>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |