3GPP TSG SA WG 1 Meeting #99e 	S1-222128r1

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-222128)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | <Spec#> | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of a location-related requirement supporting various positioning accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Kyonggi University |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 6.1 - Rel-18 correction and clarification CRs |  |  |  |  |  | Date: |  |  | 2022-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The MCX service can support a mechanism to provide location information describing positions of MCX UEs with different location accuracy simultaneously, which is usually required in various smart mobility services as part of the MCX service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corresponding requirements are introduces in Clauses 5.11 and 6.12 of TS 22.280. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MCX service cannot support obtaining and conveying location information describing positions of MCX UEs with different location accuracy simultaneously. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.11 and 6.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

============================ First Change ==============================

5.11	Location

[R-5.11-001] The MCX Service shall support obtaining and conveying Location information describing the position of the MCX UE.

[R-5.11-002] The MCX Service should support obtaining and conveying high accuracy Location information describing the position of the MCX UE.

[R-5.11-002a] The MCX Service shall be able to provide a mechanism for obtaining high accuracy Location information by integrating position information from multiple external sources (e.g. magnetometers, orientation sensors, GNSS)

[R-5.11-002b] The MCX service shall support obtaining and conveying location information describing the positions of each MCX UE with different location accuracy simultaneously.

[R-5.11-003] The MCX Service shall provide for the flexibility to convey future formats of Location information.

[R-5.11-004] The MCX Service shall provide a means for MCX Service Administrators to manage the privacy of Location information for MCX Users within their authority.

[R-5.11-005] An authorized MCX User shall be able to control the supplying of Location information by the MCX UE for MCX Service communications.

[R-5.11-006] The conveyed Location information shall be the most recently obtained information about the position of the MCX UE at the time of the Location information conveyance.

[R-5.11-007] The MCX Service shall be capable of configuring and re-configuring one or more Location information update triggers (i.e., identified conditions that, when satisfied, cause the MCX UE to report its current Location information).

[R-5.11-008] The MCX Service shall be able to modify Location information update triggers of an MCX User while the MCX User is on the network.

[R-5.11-009] The MCX Service shall provide a means for an MCX UE to send a Location information update whenever a trigger condition is satisfied (e.g., initial registration, distance travelled, elapsed time, cell change, tracking area change, PLMN change, MCX Service communication initiation).

[R-5.11-010] The MCX Service shall provide a means for an MCX UE to send a Location information update whenever the MCX User initiates an MCX Service Emergency Alert.

[R-5.11-011] The MCX Service shall provide a means for an MCX UE to send a Location information update whenever the MCX User initiates an MCX Service Emergency Group Communication.

[R-5.11-012] The MCX Service shall provide a means for an MCX UE to send a Location information update if the MCX User is in an MCX Service Emergency State and a configured amount of time has passed since the previous location information update.

[R-5.11-013] The MCX Service shall provide a means for an MCX UE to send a Location information update whenever a trigger condition is satisfied while the MCX User is in MCX Service Emergency State (e.g., initial registration, distance travelled, elapsed time, cell change, tracking area change, PLMN change, MCX Service communication initiation).

NOTE 1:	The Location information update triggers for an MCX User in an MCX Service Emergency State might be different than the Location information update triggers used when the MCX User is not in an MCX Service Emergency State.

[R-5.11-014] The MCX Service shall provide a means for an MCX Service Administrator to define geographical areas to be used for Location information update triggers for MCX Users within their authority.

[R-5.11-015] The MCX Service shall provide a means for an MCX UE in a predefined area to send a Location information update whenever a trigger condition configured in an MCX User's active MCX Service User Profile is satisfied (e.g., initial registration, distance travelled, elapsed time, cell change, tracking area change, PLMN change, MCX Service communication initiation).

NOTE 2:	The Location information update triggers for an MCX User in a predefined area might be different than the Location information update triggers used when the MCX User is not in a predefined area.

6.12	Location

[R-6.12-001] The MCX Service shall provide Location information of the transmitting MCX UE to receiving MCX UEs subject to privacy restrictions.

[R-6.12-002] The MCX Service shall support conveyance of Location information provided by 3GPP location services.

[R-6.12-003] The MCX Service shall provide a means for an authorized MCX User to restrict the dissemination of his Location information.

[R-6.12-004] The MCX Service shall provide end-to-end confidentiality of Location information.

[R-6.12-005] The MCX Service shall provide authentication of messages carrying Location information.

[R-6.12-006] The MCX Service shall provide a means for an authorized MCX User to activate a one-time Location information report of an MCX User and periodic Location information update reports of an MCX User or a specific Functional Alias.

[R-6.12-007] The MCX Service shall provide a means for an authorized MCX User to deactivate periodic Location information update report of an MCX User.

[R-6.12-008] The MCX service shall support obtaining and conveying location information describing the positions of each MCX UE with different location accuracy simultaneously.

============================ End of Changes ============================