3GPP TSG-SA WG1 Meeting #99e 	S1-222185r1

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

Source:	Huawei, Hisilicon, China Mobile

pCR Title:	Pseudo-CR on Use case smart livestock farming using Ambient IoT

Draft Spec:	3GPP TR 22.840

Agenda item:	7.3

Document for:	Approval

Contact:	Shuang Zhang (zhang.shuang2@huawei.com)

Abstract: This document provides a TP to TR 22.840 on the new use case of enabling smart livestock farming with Ambient IoT.

1. Introduction

The global sensor market continues to grow. Connected sensors and IoT have been use to contribute to successes. For animal husbandry and precision livestock farming (PLF), new perspectives are learned for 5G technologies to provide communication to ultra-low complex, extremely thin, light and small IoT devices powered by harvesting ambient energy. This will ensure adoption of IoT in smart livestock farming because this new type of Ambient IoT devices are much more comfortable for the animals, maintenance free (no human intervention as far as energy storage is concerned). By doing so, early signs of ailment of livestock is detected for early intervention, hence more efficient production of quality food at lower cost is realized, which help human beings to improve sustainability and respond to the imminent energy crisis and food shortage we face today.

2. Reason for Change

To provide a new use case of smart livestock farming.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	International Energy Agency (IEA), "World Energy Outlook 2020". Available online: https://www.iea.org/reports/world-energy-outlook-2020

[3] 	Motlagh, N.H.; Mohammadrezaei, M.; Hunt, J.; Zakeri, B. "Internet of Things (IoT) and the Energy Sector." Energies 2020, 13, 494.

[4] 	Pereira F., Correia R., Carvalho N.B. "Passive Sensors for Long Duration Internet of Things Networks." Sensors 2017;17:2268. doi: 10.3390/s17102268.

[5] 	https://www.gs1.org/standards/rfid.

[6]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC)".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.011: “Service accessibility”.

[10] 	5G for smart manufacturing: https://www.gsma.com/iot/wp-content/uploads/2020/04/2020-04_GSMA_SmartManufacturing_Insights_On_How_5G_IoT_Can_Transform_Industry.pdf

[11] 	https://forcetechnology.com/en/articles/batteryless-electronics-energy-harvesting

[12] 	R. Ding and B. Xing, "Comparative Research on the Way of Energy Harvesting of the Wireless Sensor Network Nodes," 2013 6th International Conference on Intelligent Networks and Intelligent Systems (ICINIS), 2013

[13] 	P. D. Hung et al., "A Self-Powered Wireless Gas Sensor Node Based on Photovoltaic Energy Harvesting," 2021 Symposium on VLSI Circuits, 2021.

[x1] 	 	    TR 22.866: "enhanced Relays for Energy Efficiency and Extensive Coverage".

[x2]	TR 45.820: “Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT)”

[y1] 		     Global RFID Sensor Market - Global Industry Analysis, Size, Share, Growth, Trends

and Forecast 2021-2031, Transparency Market Research, June 2021

[y2]		     Sensors: Technologies and Global Markets (

https://www.bccresearch.com/market-

research/instrumentation-and-sensors/sensors-technologies-markets-report.html),BCC

Research

[y3]		     Berckmans, D. Precision livestock farming technologies for welfare management in

intensive livestock systems. Rev. Sci. Tech. Off. Int. Epiz. 2014, 33, 189–196.

[y4] 	     Precision Agriculture for Crop and Livestock Farming - Brief Review, Animals 2021,

11, 2345. https://doi.org/10.3390/ani11082345, by António Monteiro, et al.

[y5]	B. F. Wolter, M. Ellis, S. E. Curtis, N. R. Augspurger, et al, Effect of Group Size on Pig Performance in a Wean-to-Finish production system, Journal of Animal Science

[y6] 	Nora O’Donovan, https://www.independent.ie/business/farming/dairy/dairy-advice/putting-a-size-on-paddocks-30447793.html

[y7]	Carol Souza de Silva, Wageningen University, the impact of Pen Size and Stocking Density on behaviour and welfare of growing pigs

[y8]	Xue Li, Xia Xiong, et al, Effects of stocking density on growth performance, blood parameters and immunity of growing pigs, December, 2020

* * * End of the First Change * * * *

* * * Second Change * * * *

## 5.x	Use case of smart livestock farming enabled by Ambient IoT

### 5.x.1	Description

The global sensor market is predicted to grow from $193.9 billion in 2020 to $332.8 billion in 2025 at a CAGR of 11.4% [y1]. Globally, for tracking and monitoring the IoT market size is forecast to grow from US$ 8,575 million in 2021 to US$ 18,525.0 million at a CAGR of 8.01% in 2031 [y2].

It is certainly not new that connected sensors and IoT can play a role in animal husbandry. Precision livestock farming (PLF) as a more trendy term adopts an innovative production system approach [y3] playing a key role in Industry 4.0 [y4]. More efficient production of quality food at lower cost will be an important tool to improve sustainability and respond to the imminent energy crisis and food shortage we are facing today. Physical vitals of livestock such as temperature are monitored for farmers to take early actions before potential diseases cause severe economic loss. The body temperature of livestock is a precise health indicator and changes in body temperature are often the first sign of an acute illness. For these purposes, the target data acquisition process of animal body temperatures is not latency-sensitive, rarely an acquisition interval down to an hour is needed. Also, the needed network coverage is usually local (e.g. an indoor pigsty or outdoor dairy cow paddocks), which is different from existing NB-IoT/(e)MTC targeting at providing long-range communication while achieving long battery life time.

As the adoption of PLF continues, feedback is often received from livestock farmers at industry conferences. In EU the EU-PLF conference was held as early as 2016. Feedback includes that dairy cow farmers in countries like the Netherlands (one of the global top 5 dairy exports) have been considering replacement of active monitoring IoT devices (with battery-powered transponders) with cheaper ear tags. This is partly because of economical drawbacks with increasing herd sizes. More importantly, the thick and heavy neck- or leg-mounted devices can cause discomfort to livestock, so that they are often scraped off against walls (of the pen) or damaged by animals involuntarily.

A more recent GSMA publication [10] explicates disadvantages as battery drain rendering the loss of asset visibility in only a few months. For these technologies the frequency of data transmission would impact the battery life time. For smart livestock farming, it is preferred to have the IoT device serve livestock’s lifespan – implying the energy storage component in the IoT device should operate autonomously over a long period comparable to the lifespan of the IoT device excluding this component. In the same GSMA paper, another drawback of high CAPEX in dense deployment of the “tag readers” is highlighted, primarily due to the poor communication range supported by these alternative technologies.

These above aspects in smart livestock farming can be better addressed by Ambient IoT. Figure 5.x.1-1 and Fig. 5.x.1-2 illustrate a typical pigsty and dairy cow paddocks respectively. A pigsty is usually smaller than 100 square meters. Pig pin dimensions being considered include 5.74m by 3.05m for 25-pig group, 6.10m by 12.20m for 50-pig group [y5], etc.

Figure 5.x.1-1: Pigsty

Figure 5.x.1-2 Dairy cow paddocks

Paddocks is where grazing pasture is offered to dairy cows outdoors. In general the dimension of paddocks is calculated by multiplying the number of cows by their total daily intake by days in the paddock and then divided by the ideal pre-grazing yield (PGY). There is a physical limitation of the paddock dimension. For instance, for 80 cows assuming regular values for parameters previously mentioned, the paddock size comes to 1.54 hectares [y6] (around 124m by 124m).

Attached to each pig or dairy cow is a small, thin and light tag (a type of Ambient IoT device) that includes a basic temperature sensor. These Ambient IoT devices power themselves by harvesting energy from the environment (e.g. solar).

The base stations provide to the tags random access and data transmission over the radio interface. The tags are capable of storing tags’ identifiers and small sized data captured by sensors. The 5G system provides base station capability (e.g., “tag reading” functionality), tag operation and management. The monitored data is collected remotely according to the health-analyzing applications.

### 5.x.2	Pre-conditions

BIO-Duurzaamheid is modernizing their livestock health management process to improve efficiency and productivity of pig and dairy farms. Hence they have attached to their livestock wireless temperature sensors (tags), a form of Ambient IoT devices, for the remote livestock health management application to retrieve livestock’s temperatures to detect early signs of ailment that could take days or weeks to develop. The remote livestock health management application analyses the collected sensor values to identify potential illness of certain animals before symptoms appear.

BIO-Duurzaamheid has a service level agreement with GroenTEL to deploy 5G network to enable the communication of Ambient IoT devices with the network. As part of the service level agreement, GroenTEL provides efficient 5G coverage and energy-efficient communication to pig pins and paddocks managed by BIO-Duurzaamheid. This includes:

- Interfacing with BIO-Duurzaamheid’s remote livestock health management system;

- Providing energy-efficient mechanisms for Ambient IoT devices’ network access

- Providing efficient communication between the network and Ambient IoT with the required communication performance

- Ensuring the long lifespan of Ambient IoT devices without human intervention of any energy storage component possibly used in Ambient IoT devices

### 5.x.3	Service Flows

- The tags (type of Ambient IoT devives with wireless sensors) measure livestock body temperature. The type of temperature sensors are very simple, typical sampling rate is less than 10 Hz with sample size of 32 bits [x1], thus the data generated per tag is less than 320 bit/s.

- Upon the request from the livestock health management application, the 5G core network starts inventory process via the selected gNB(s). By detecting the signals from the gNB, the tags respond to the command.

- The 5G core network, based on the requests issued by the application function, performs operations (i.e. "inventory", "read", etc.) on tags correspondingly. "Inventory" operation is to read the tag identifier. "Read" operation is to read temperature sensor data.

- The 5G core network then sends the results of the operations to the livestock health management application. The application function includes analytics functions that detect the anomaly and notifies the farmers of BIO-Duurzaamheid when necessary.

- In some additional situations, BIO-Duurzaamheid’s livestock management application requests the 5G core network to perform sensor data read-out operation on specific tags attached to particular individual livestock (e.g. pregnant sows, lactating cows). The corresponding tags respond to the operation and report the temperature data.

### 5.x.4	Post-conditions

The 5G network enables efficient communication for Ambient IoT devices, the livestock management application is enabled by the 5G system to retrieve temperature sensor data collected by sensors in the Ambient IoT devices. Depending on the needs, the livestock management application is enabled by 5G system to retrieve the sensor data for either an entire drove of piglets, a passel, a herd of cattle or some individual pigs/dairy cows.

Owing to the thin and light-weight Ambient_IoT devices, livestock are more receptive of wearing them without feeling discomfort. As a result of data analysis done by the livestock management application, early signs of ailment of livestock are identified, timely intervention is carried out. This largely increases production efficiency of BIO-Duurzaamheid’s livestock farms. Also thanks to the efficient Ambient_enabled IoT communication enabled by the 5G system, BIO-Duurzaamheid continue to use Ambient_enabled IoT without worrying about energy drain of energy storage component in an Ambient IoT device or replacement of Ambient_IoT devices throughout the planned production lifespan of their livestock.

### 5.x.5	Existing features partly or fully covering the use case functionality

SA1 has performed various studies on IoT in previous releases, where related normative stage 1 requirements are introduced in TS 22.011 [9], TS 22.278 [7], TS 22.368 [6], and TS 22.261 [8].

TS 22.011 introduces access control for MTC, examples of periodic network selection attempts are:

For UEs only supporting any of the following, or a combination of, NB-IoT, GERAN EC-GSM-IoT [18], and  Category M1[13] of E-UTRAN enhanced-MTC, the UE shall interpret the interval value to be between 2 and 240 hours, with a step size of 2 hours between 2 and 80 hours and a step size of 4 hours between 80 and 240 hours.

In the absence of a permitted value in the SIM/USIM, or the SIM/USIM is phase 1 and therefore does not contain the datafield, then a default value of 60 minutes, shall be used by the UE except for those UEs only supporting any of the following, or a combination of: NB-IoT, GERAN EC-GSM-IoT [18], and Category M1 [17] of E-UTRAN enhanced-MTC. For those UEs a default value of 72 hours shall be used.

NOTE:	Use of values less than 60 minutes may result in excessive UE battery drain.

TS 22.368 addresses features of MTC communication and service requirements related to MTC device triggering, addressing, identifiers, low mobility, small data transmission, infrequent MT communication, security, remote MTC device management, group-based MTC features including policing and addressing, etc. Example requirements are:

The system shall provide mechanisms to lower power consumption of MTC Devices.

The system shall provide mechanisms for the network operator to efficiently manage numbers and identifiers related to MTC Subscribers.

TS 22.261 captures some important service requirements for IoT, e.g.

The 5G system shall support a secure mechanism for a home operator to remotely provision the 3GPP credentials of a uniquely identifiable and verifiably secure IoT device.

The 5G system shall support a secure mechanism for the network operator of an NPN to remotely provision the non-3GPP identities and credentials of a uniquely identifiable and verifiably secure IoT device.

An IoT device which is able to access a 5G PLMN in direct network connection mode using a 3GPP RAT shall have a 3GPP subscription.

The 5G system shall allow the operator to identify a UE as an IoT device based on UE characteristics (e.g. identified by an equipment identifier or a range of equipment identifiers) or subscription or the combination of both.

An IoT device which is able to connect to a UE in direct device connection mode shall have a 3GPP subscription, if the IoT device needs to be identifiable by the core network (e.g. for IoT device management purposes or to use indirect network connection mode).

The 5G system shall support operator-controlled alternative authentication methods (i.e. alternative to AKA) with different types of credentials for network access for IoT devices in isolated deployment scenarios (e.g. for industrial automation).

In these specifications, albeit the service requirements addressing traits for IoT in terms of low device power consumption, small and infrequent data transmissions, long service lifetime, and resource efficiently, the IoT devices considered in 3GPP have been assumed to be powered by at least batteries up till now. To enable extremely small, light-weight, battery-less Ambient IoT devices that engage in basic IoT data transaction and appropriate level of operator management and charging suitable for the target scenarios, new challenges to the 5G system are foreseen and need to be addressed.

### 5.x.6	Potential New Requirements needed to support the use case

### 5.x.6.1 	KPIs for the use of Ambient IoT devices for smart livestock farming

[PR 5.x.6.1-1] The 5G system shall provide the network connection to address the KPIs for the use of Ambient IoT devices in substations in smart grids, see table 5.x.6.1-1.

Table 5.x.6.1-1 – Potential key performance requirements for the use of Ambient IoT devices for smart livestock farming

| Scenario | Max. allowed end-to-end latency | Max. instantaneous device communication power consumption | Service bit rate: user-experienced data rate | Message Size | Communication range | Device density | Service area dimension |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Smart livestock farming (Indoor pigsty) | >10 s (note 1) | [100] µW | <320 bit/s | Typically [< 100 bytes] (note 2) | 20 m indoors | 850 000 devices / km2 (note 3) | 1200 m2 (note 4) |
| Smart livestock farming (Outdoor dairy cow paddocks) | >10 s | [200] µW | < 320 bit/s | Typically [< 100 bytes] (note 2) | 150 m outdoors | 5200 devices / km2 (note 5) | 1 km2 (note 6) |
| NOTE 1: M2M devices may in general support relaxed delay characteristics. Even for certain applications (e.g. alarms) that may require a reasonably strict delay profile, a delay requirement of 10 seconds is appropriate for the uplink when measured from the application 'trigger event' to the packet being ready for transmission from the base station towards the core network [x2]. NOTE 2: Electronic Product Code standard [5], this size is the payload size. NOTE 3: Stocking density of 1.2 m2/pig is considered high and 2.4 m2/pig considered low [y7]. In [y8] the stocking density range from 0.82 m2/pig and 2.46 m2/pig is studied and 1.23 m2/pig proofs suitable stocking density for growing pigs. The device density of 850000 is from a real farm and translates to stocking density of 1.17 m2/pig. NOTE 4: Temporary activity with 1000 animals in surface area of around 1200 m2. NOTE 5: Calculated from 80 dairy cows assuming regular values for parameters (e.g. daily intake, pre-grazing yield) previously mentioned, the paddock size comes to 1.54 hectares [y6] (about 124m by 124m). NOTE 6: For rotational grazing different number of paddocks are used (e.g. 16, 32, 64). A grazing infrastructure of 1 km2 suffices for hosting a 64-paddocks rotational grazing. |  |  |  |  |  |  |  |

Editor’s Note: It is FFS on whether or not max. Instantaneous device communication power consumption is included in this KPI table, which depends on SA1’s final decision.

### 5.x.6.2	Service requirements for use of Ambient IoT devices for smart livestock farming

[PR.5.3.6.2-1] The 5G system shall support energy efficient communication mechanisms (i.e. minimizing the device communication power consumption) for Ambient IoT devices, while meeting the communication performance requirements.

[PR.5.3.6.2-2] The 5G system shall support a mechanism to interface a 3rd party application to manage and operate on the Ambient IoT devices.

[PR.5.3.6.2-3] The 5G system shall be able to collect per charging period the charging information for using Ambient IoT service on a per Ambient IoT device basis (e.g. total number of application messages over the entire population of Ambient IoT devices) or per ownership basis (e.g. total number of Ambient IoT devices associated with the same enterprise customer).

[PR.5.3.6.2-4] The 5G system shall optimize mobility management support for Ambient-enabled IoT devices that are unable to initiate location update, so as to minimize overall signalling overhead when performing operations to specific Ambient-enabled IoT devices.

NOTE: Applicable when 5G network needs to perform sensor data retrieval from particular individual livestock (e.g. pregnant or lactating cows) specifically in outdoor situations.

* * * End of the Second Change * * * *