3GPP TSG-SA WG1 Meeting #99e 	S1-222205r3

Electronic Meeting, 22 Aug - 1 Sep, 2022

Source:	China Telecom, OPPO

pCR Title:	Use case of direct device connection assisted remote control operation for robotics service

Draft Spec:	3GPP TR 22.876

Agenda item:	7.7

Document for:	Approval

Contact:	Heng Wang (wangh26@chinatelecom.cn)

Yang Xu (xuyang@oppo.com)

Abstract: This document proposes a use case and related potential requirements to be included in FS_AIML_Ph2 TR 22.876 version 0.0.0.

********** First Change *********

2	References

[x1]	3GPP TS 22.104, Service requirements for cyber-physical control applications in vertical domains

[x2]	Huaijiang Zhu, Manali Sharma, Kai Pfeiffer, Marco Mezzavilla, Jia Shen, Sundeep Rangan, and Ludovic Righetti, “Enabling Remote Whole-body Control with 5G Edge Computing”, to appear, in Proc. 2020 IEEE/RSJ International Conference on Intelligent Robots and Systems. Available at: https://arxiv.org/pdf/2008.08243.pdf

[x3]	B. Kehoe, S. Patil, P. Abbeel, and K. Goldberg, “A survey of research on cloud robotics and automation,” IEEE Transactions on automation science and engineering, vol. 12, no. 2, pp. 398–409, 2015.

[x4]	3GPP TR 22.874, Study on traffic characteristics and performance requirements for AI/ML model transfer in 5GS (Release 18)

********** Second Change *********

x.y	Direct device connection assisted remote control operation for robotics service

x.y.1	Description

Remote controlled robot is one of the most significant AI/ML application so far. It plays an important role in many scenarios, e.g. warehouse, disaster rescue and smart factories [x1]. Considering the mobile robots shall work in dynamic environment, hence it needs to perform fast and reliable environment sensing and task schedule with real-time and precise control. An quadruped robot carries batteries of 3kg with maximum power of 650Wh [x2], while a high performance GPU consumes more than 250W. Thus, the battery life will be significantly impacted if such computational modules was embedded on the robot.

Although the robot control task can be completed by itself with its available computation and energy resource, according to [x3, x4], the split controlling operation can reduce both the transmission burden and the local computation burden. As shown in Fig. x.y.1-1, the robot needs to send “sensing” data per control cycle and receiving “remote control” data from the cloud/edge control server per control cycle, in order to ensure the local controller can take over in time once an unexpected latency occurs. In that case, a maximum round-trip latency of 25ms can be tolerated for each control cycle, and the downlink latency for downloading “remote control” part can be relaxed to 12ms.

Figure x.y.1-1. Split controlling operation over 3GPP network [x4]

x.y.2	Pre-conditions

A robot is performing outdoor task using split remote control operation. The robot, as a UE, is transmitting the data to the cloud/edge control server over the operator’s 5G network, and there are related indicators as follow [x3, x4]:

-	For each control cycle of 1ms, the UE sends an UL data with size of 592B to the cloud/edge control server.

-	For each control cycle of 1ms, the UE receives a DL data with size of 40kB from the cloud/edge control server.

-	For transmission latency, the overall remote control application mostly cares round-trip latency. And it can be calculated as: The round-trip latency (25ms) = application processing time (12ms) + 5GS UL and DL transmission time (13ms).

Figure x.y.2-1 Time consumption of application processing and data transmission with certain UL and DL data size

The operator’s 5G network has the ability to support data transmission via direct network connection (Uu) or via direct device connection (sidelink).

x.y.3	Service Flows

- 1. The robot connects to the cloud/edge control server via the  operator’s 5G network. How the split remote control operations works on the robot side and the server side are determined by the application.

- 2. The robot is well powered and in good coverage, so it can perform the determined split control operations well. For each control cycle, the robot performs some local computation and sends the sensing data to the server via direct network connection. The server performs the remote control computation, and respond the commands back to the robot.

- 3. Later on, when the robot is moving to a bad coverage area where the robot will not be able to use the direct network connection for the data transmission anymore, the robot discovers a neighboring relay UE for the UL and DL data transmission via direct device connection.

- 4. To satisfy the same round-trip transmission latency, the QoS parameters of data transmission, with invariable UL and DL data size, should be adjusted due to the introduction of direct device connection. For example, the operator’s 5G network allocates 3ms for UL and DL in direct device connection and 10ms for other parts of the network, and the value may be changed based on the monitoring information.

- 5. The robot service with split remote-control operation is sustained without any user experience degradation and interruption.

x.y.4	Post-conditions

Due to the timely direct device connection assisted by a neighboring UE, the robot in split remote-control operation can always complete outdoor tasks no matter in good or bad coverage.

x.y.5	Existing features partly or fully covering the use case functionality

TS 22.261 v18.6.0 clause 7.10

the AI inference KPI is defined as the table below. Although all KPIs is strictly defined per UL and DL, there is no such KPI requirements applied to the total UL and DL as well as direct device connection within the whole communication.

Table 7.10-1 KPI Table of split AI/ML inference between UE and Network Server/Application function [1]

| # Uplink KPI |  |  |  |  | # Downlink KPI |  |  |  | # Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Max allowed UL end-to-end latency | # Experienced data rate | # Payload size | # Communication service availability | # Reliability | # Max allowed DL end-to-end latency | # Experienced data rate | # Payload size | # Reliability |  |
| 2 ms | 1.08 Gbit/s | # 0.27 MByte | # 99.999 % | # 99.9 % |  |  |  | # 99.999 % | # Split AI/ML image recognition |
| 100 ms | 1.5 Mbit/s |  |  |  | # 100 ms | # 150 Mbit/s | 1.5 MByte/‌frame |  | # Enhanced media recognition |
|  | # 4.7 Mbit/s |  |  |  | 12 ms | 320 Mbit/s | 40 kByte |  | Split control for robotics |
| NOTE 1: Communication service availability relates to the service interfaces, and reliability relates to a given system entity. One or more retransmissions of network layer packets can take place in order to satisfy the reliability requirement. |  |  |  |  |  |  |  |  |  |

x.y.6	Potential New Requirements needed to support the use case

x.y.6.1	Potential Functionality Requirements

[P.R.x.y-001] The 5G system shall be able to provide means to determine a UE’s UL and DL QoS parameters on direct device connection, based on the characteristics of transmission (e.g. UL and DL data size, transmission interval, and total UL and DL transmission latency) required by an authorized 3rd party.

Editors Note: Clarification of this requirement in regard to relay functionality when one UE is out of coverage is FFS.

x.y.6.2	Potential KPI Requirements

Based on the the description in subclause x.y.2, the KPI requirements are summarized as below:

Table x.1-1	KPI requirements for remote control operation for robotic service

| Remote control operation | Maximum data size per round | Total UL and DL end-to-end latency | Experienced UL data rate | Experienced DL data rate |
| --- | --- | --- | --- | --- |
| Remote controlled robot | UL: 592Bytes DL: 40kBytes | 13ms | 4.7Mbit/s (NOTE 1) | 320Mbit/s (NOTE 1) |
| NOTE 1: In the use case of split control, even if the total UL and DL end-to-end latency is certain, the UL part and the DL part may be changed, so the UL and DL data rate may be changed respectively. The assumption here is that the UL end-to-end latency is 1ms and DL end-to-end latency is 12ms. |  |  |  |  |

*******************End of Change*****************