3GPP TSG SA WG 1 Meeting #99e 	S1-222254r6

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

Source:	Hansung University, LGUplus, KT, ETRI

pCR Title:	FS_RAILSS consolidated requirements

Draft Spec:	3GPP TR 22.890 v 0.6.2

Agenda item:	7.1

Document for:	Approval

Contact:	Andrew Min-gyu Han, andyhan@hansung.ac.kr

Abstract: This contribution provides contents for the potential consolidated requirements for RAILSS.

1. Introduction

This pCR provides contents for the potential consolidated requirements for RAILSS.

2. Reason for Change

The potential consolidated requirements are important contents for RAILSS.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.890.

* * * First Change * * * *

# 7	Potential Consolidated Requirements

## 7.1 Introduction

The requirements below refer to a "Railway Smart Station Services", which is acting as an application a FRMCS and the outer systems.

The potential consolidated requirements are mainly focusing the 5G Network characteristics and the interfaces between FRMCS/MCX Functions and Railway Smart Station Services as the application of FRMCS and outer system of 3GPP.

Figure 7.1-1 Scope of the Potential Consolidated Requirements

## 7.2	Connectivity

Table 7.2-1 – Connectivity Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.2-1 | The 5G System shall support the proximity connections between the UEs. Note: UEs can be kiosks or Mobile Intelligent Assistant | PR 6.2.6-1 | Requirement already supported in TS 22.278 clause 7A and TS 22.280 clause 7 |

## 7.3	Other aspects

Table 7.3-1 – Other Aspects Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.3-1 | The MCX service shall be able to exchange data (e.g. 3D display or metaverse enabled map generated by the application) through an open interface to a 3rd party application. | PR 6.2.6-2 | Requirement already supported in TS 22.280 clause 5.18 [R-5.18.2-001] |
| CPR 7.3-2 | The MCX service shall support obtaining and conveying MCX UE location information describing the positions of each MCX UE with different location accuracy simultaneously. | PR 6.3.6-1 | intended to be included in clauses 5.11 and 6.12 of TS 22.280 |
| CPR 7.3-3 | The FRMCS shall be able to provide a mechanism to trigger an emergency alert based on a combination of UE location (e.g. the location of the specific platform door / train door) and application-generated trigger (e.g. train door did not close properly due to a blockage) | PR-8.x.6-1 | intended to be included in a new clause of TS 22.280 |

## 7.4 Performance

Table 7.4-1: KPIs for Railway Smart Station Services

| Scenario (Note 5) | End-to-end latency | Reliability (Note 1) | UE speed | UE Relative Speed | User experienced data rate | Payload size (Note 2) | Area traffic density | Overall UE density | Service area dimension (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Multiple trains’ stops at the same platform (Korea, urban railway) | ≤10 ms | 99.9999% | ≤100 km/h | ≤50km/h | ≤1Mb/s | Small to large | ≤ 1 Mb/s/km | ≤ 5 (100m) | ≤ 15 km along rail tracks including bad weather conditions (Note 4) |
|  | NOTE 1: Reliability as defined in TS 22.289 sub-clause 3.1. NOTE 2: Small: payload ≤ 256 octets, Medium: payload ≤512 octets; Large: payload 513 -1500 octets. NOTE 3: Estimates of maximum dimensions. NOTE 4: Non-Line-of-Sight (NLOS) between UEs shall be supported NOTE 5: Off-network traffic characteristics are not addressed in this table since it can be covered by TR22.990. |  |  |  |  |  |  |  |  |

Note: This table is intended to be included in clause 6.2 of TS 22.289 [9].

* * * 2nd Change * * * *

# 2	References

The following documents contain provisions, which through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	UIC MG-7900v2.0.0, Future Railway Mobile Communication System – Use cases, Feb. 2020.

[3]	UIC FU-7100v5.0.0, Future Railway Mobile Communication System – User Requirements Specification, Feb. 2020.

[4]		TTA TTAK.KO-06.0507/R1, Requirements for Smart Railway Device - Information Model, Dec. 2020.

[5]	TTA TTAK.KO-06.0508/R1, Requirements for Smart Railway Platform - Information Model, Dec. 2020.

[6]	3GPP TR 22.990. “Study on off-network for rail”.

[7]                        3GPP TS 22.280: " Mission Critical Services Common Requirements (MCCoRe)"

[8]	3GPP TS 22.282: “Mission Critical (MC) data”.

[9]	3GPP TS 22.289: “Mobile communication system for railways”.

[10]	3GPP TS 22.281: “Mission Critical (MC) Video”.

* * * End of Change * * * *