SA WG2 Meeting #105	S2-143663

13 - 17 October 2014, Sapporo, Japan	(revision of S2-143242)

Source:	Alcatel-lucent

Title:	Cell ID announcement in ProSe UE-to-Network relays

Document for:	approval

Agenda Item:	7.13

Work Item / Release:	eProSe_Ext / Rel-13

Abstract of the contribution: This contribution adds the capability to announce on demand the ECGI of the cell serving the ProSe UE-to-Network relay.

# Introduction

When a UE is behind a ProSe UE-to-Network relay it cannot obtain the ECGI of the serving cell without the cooperation of the ProSe UE-to-network relay. The procedure defined in this P-CR allows UEs behind a ProSe UE-to-Network relay to request the announcement of the ECGI value of the cell currently serving the ProSe UE-to-Network relay on an ongoing basis, until needed.

# Proposal

It is proposed to discuss the procedure here below and if it was found valuable to add it as agreement to the TR 23.713

FIRST CHANGE

#### 7.2.1.x 	Support of EGGI announcement

Some applications require the knowledge of the ECGI of the cell serving the UE. When a UE is behind a ProSe UE to Network relay, the serving cell is the cell serving the ProSe UE-to-Network relay. So, the Announcement of the ECGI allows remote UEs to receive the value of the ECGI of the cell serving the ProSe UE-to-Network relay.

An example of an application requirement may be the dynamic activation of eMBMS in the cell where the ProSe UE-to-Network relay is camping, if there is a sufficient number of devices behind the ProSe UE-to-Network  relay, in order to avoid e.g. unicast distribution of multiple application streams of the same content in unicast mode to different devices under the same ProSe UE-to-Network  relay (note that aggregation of unicast streams is not possible when the relay is behaving as Layer 3 device).  Once eMBMS is activated in the cell, the ProSe UE-to-Network relay would start behaving as documented in 7.2.1.2 "eMBMS relay support" to deliver eMBMS content to the UEs that requested it.

SECOND CHANGE

### 7.2.4 	Topics for further study for ProSe UE-Network Relays

Editor’s note:	Topics for FFS will be collected for this particular functionality.

The following issues need to be resolved:

-	It is FFS whether the TMGI advertisement by the UE-to-Network relay happens by using Direct Communications one to many using a shared ProSe Layer 2 ID to all UEs the UE-to-Network relay serves; or using a ProSe Layer 2 ID dedicated for a specific application layer group related to the TMGI; or whether the TMGI advertisement happens using discovery signalling.

-	It is FFS whether the messages used by the remote UE to request a UE-to-Network relay to monitor a certain TMGI are at the access stratum or non access stratum layer.

-	 It is FFS whether a security association between the UE and the UE-to Network relay is per UE or per ProSe Application Group.

-	It is FFS whether the ECGI is continuously announced by the ProSe UE-to-Network relay or whether a dedicated procedure is required to activate the broadcasting of ECGI.

- 	If a dedicated procedure is used to activate ECGI broadcasting, it is FFS whether a soft state keeping track of the existence of at least one UE interested in receiving is kept by refreshing the activation request by interested UEs.