SA WG2 Meeting #112	S2-153860

16 - 20 November 2015, Anaheim, California, USA	(revision of S2-15xxxx)

Source:	Deutsche Telekom

Title:	S8HR solution#2 with ENUM

Document for:	Approval

Agenda Item:	6.6

Work Item / Release:	FS_V8/Rel-14

Abstract of the contribution: Number Translation for local numbers of the VPLMN can be realized via ENUM. A Note describing this possibility should be added.

# Proposal

It is proposed to modify the description for solution #2 in TR 23.749 as follows::

***************** Start of changes **********************

#### 6.2.1	Description

This is a solution for:

-	Key Issue 4 - Local Number Translation and Routing

The P-CSCF will follow the procedure as described in 23.228 and forward the request to the S-CSCF, which will forward the request further to an AS to resolve the local number and translates it into a global format in the AS.

UE

PGW

PCRF

P-CSCF

S-CSCF

VPLMN

HPLMN

1. IMS Registration

2. INVITE (VPLMN short code)

3. INVITE

AS

4. INVITE

6. Local Nr. Translation

5. Retrieve Location

# Figure 6.2.1-1: Detection of Local Numbers

1. The UE registers to IMS.

2. The UE sends a SIP INVITE with a local short number of the VPLMN.

3. The P-CSCF detects the local number in the Request URI and forwards it to the S-CSCF.

4. According to the procedure in TS 23.228 [8], clause 4.2.2, the S-CSCF routes the request to the AS

5. The AS may need to identify the visited access network, e.g. from information in SIP signalling or via the Sh interface. If more detailed location information is required, the AS may subscribe to location information reporting to the PCRF/PGW, i.e. using the existing Netloc procedures.

NOTE: 	Based on the required granularity, Netloc should provide the appropriate level of information.

6. The AS translates the local number into a globally routable format.

NOTE: The translation of the local number is implementation specific. A possible solution is that the AS performs an ENUM query in the VPLMN (AS in HPLMN acts as ENUM client and sends the query to ENUM server in VPLMN; e.g. <VPLMN MCC MNC> + < infix for short code translation>+<dialled short code>; the response should contain a routable number).