SA WG2 Meeting #113	S2-160458

25 - 29 January 2016, St. Kitts & Nevis, KN	(revision of S2-16xxxx)

Source:	NTT DOCOMO

Title:	Evaluation of Key Issue #3: PLMN-ID determination at IMS Entities

Document for:	Approval

Agenda Item:	6.6

Work Item / Release:	FS_V8, REL-14

Abstract of the contribution: Provides evaluation of the different solutions for obtaining VPLMN-ID at IMS entities.

# Proposal

The evaluation of Solution #1 is based on updates provided by NEC to this meeting in S2-160124

The following change on TS.23.749 is proposed.

************* START OF CHANGE *************

# 7	Overall Evaluation

## 7.x Key Issue #3: Determination of the ID of the visited PLMN at IMS Entities in HPLMN

The following solutions have been captured in the TR for this key issue:

-	Solution #1 PCRF based PLMN ID.

NOTE: Here we do not consider the aspects of Solution #1 which are not related to determination of PLMN ID.

-	Solution #3 Determination of PLMN ID via HSS interaction

-	Solution #6 P-CSCF query PLMN ID in IMS registration

-	Solution #7 P-CSCF query PLMN ID in IMS registration and session establishment

The following table captures the key evaluation criteria and evaluations of these four solutions.

| Evaluation Criteria | Solution #1 (Rx Query for PLMN-ID) | Solution #3 (HSS based) | Solution #6 (New Rx query for PLMN-ID change) | Solution #7 (ULI based reporting) |
| --- | --- | --- | --- | --- |
| Impact on Nodes and Interfaces | PCRF, P-CSCF: Needs to support new Rx procedure. Rx: New procedure to request for PLMN-Id | HSS: (i) On Cx interaction with I-CSCF needs to differentiate between S8HR and LBO. (ii) Needs to provide VPLMN-ID to S-CSCF on IMS registration. S-CSCF: Needs to forward VPLMN-Id to I-CSCF and then onto the P-CSCF P-CSCF: Needs to process VPLMN-ID from the 4xx SIP authentication request message Cx and SIP 4xx authorization required | PCRF, P-CSCF: Needs to support new Rx procedure. Rx: New procedure to request for PLMN-Id Change | None (Standard Netloc functionality) VPLMN needs to support Netloc |
| Amount of additional signalling | None | None | Subscription to PLMN-ID change in access network | ULI information sent on every bearer establishment, modification, deletion. |
| Support for SA3-LI | Supported | Since PLMN-Id is not available when SIP Register Request arrives at the P-CSCF but only when 4xx Authorization Challenge message is received at the P-CSCF, the decision for LI is delayed till receipt of 4xx Authentication Request. The SIP Register message will need to be buffered at the P-CSCF till this point. | Supported. | Supported |
| Limitations of the Solution | None | The solution cannot work if the UE access EPS simultaneously via more than one access network which use different PLMN-IDs | None | None |

Out of the four solution, three of them are PCC based (Solution #1, #6 and #7).

-	Solution #7 is very signalling heavy and requires Netloc (which is typically used for finer granularity of location reporting) to be deployed in the VPLMN. Hence, Solution #7 is not an appropriate solution for determining just the VPLMN-ID at IMS.

-	Solution #1 has the least amount of signalling. It is a one time query on Rx by the P-CSCF of the PLMN-ID from the PCRF. The PCRF gets the PLMN-ID from the PGW in IP CAN session establishment and reports this to the P-CSCF. For S8HR, one will typically PLMN-ID will not change during an IMS registration. Having the PLMN-ID change during IMS registration will require both the support of inter-PLMN HO at the VPLMN and the HPLMN having roaming agreements with both the old VPLMN and new VPLMN, which has low probability. Hence a single query of PLMN-ID at IMS Registration is sufficient.

-	Solution #6 is a more general PCC solution for providing PLMN-ID to any AF and can be applied to other features than S8HR. Here the AF (P-CSCF) subscribes to PLMN-ID change on Rx interface. The P-CSCF uses existing events and Gx procedure to subscribe to PLMN-ID change from PCEF (PGW). Here the P-CSCF can get informed even when PLMN-ID changes, which though is not a requirement for S8HR.

Solution #3 is an HSS based solution. It has significant impacts to HSS, impacts to SIP header and significant deployment limitations.

Based on the above evaluation, it is proposed to select Solution #6 or Solution #1 for obtaining PLMN-ID information at the P-CSCF and IMS entities.

************* NEXT CHANGE *************

# 8	Conclusions

For Key Issue #3: Determination of the ID of the visited PLMN at IMS Entities in HPLMN:

-	Solution #6/#1 as captured in Section 6.6/6.1 is the selected solution.

************* END OF CHANGE *************