SA WG2 Meeting #113	S2-160732

25 - 29 January 2016, St. Kitts & Nevis, KN	Merge of 0688 and 0137

Source:	NTT DOCOMO, LG Electronics, ZTE, Nokia Networks?, Alcatel-Lucent?, NEC, Ericsson?

Title:	Evaluation of Key Issue #3: PLMN-ID determination at IMS Entities

Document for:	Approval

Agenda Item:	6.6

Work Item / Release:	FS_V8, REL-14

Abstract of the contribution: Provides evaluation of the different solutions for obtaining VPLMN-ID at IMS entities.

Proposal

The evaluation of Solution #1 is based on updates provided by NEC to this meeting in S2-160124

The following change on TS.23.749 is proposed.

************* START OF CHANGE *************

7	Overall Evaluation

7.x Key Issue #3: Determination of the ID of the visited PLMN at IMS Entities in HPLMN

Assumption made for this evaluation:

- VPLMN ID retrieval is NOT done as IMS re-registration

- VPLMN changes is noticed at the IMS level only due to UE performs IMS registration as a result of IP address changes.

The following solutions have been captured in the TR for this key issue:

-	Solution #3 Determination of PLMN ID via HSS interaction

HSS performs roaming restriction check based on PLMN-ID from access network. HSS provides PLMN-ID to S-CSCF in Cx interface at IMS registration. PLMN-ID added to SIP message from S-CSCF to P-CSCF.

-	Solution #6 P-CSCF query PLMN ID in IMS registration

P-CSCF subscribes to PLMN-ID change on Rx interface. The PCRF subscribes to PLMN change in access network. PCRF provides PLMN-ID on Rx to P-CSCF at PLMN-ID change.

-	Solution #7 P-CSCF query PLMN ID in IMS registration and session establishment

P-CSCF uses Netloc and subscribes to access-network information on Rx. Netloc supported in VPLMN and detailed location (eg. ECGI) provided to PCRF via PGW. PCRF provides PLMN-ID to P-CSCF.

The following table captures the key evaluation criteria and evaluations of these four solutions.

| Evaluation Criteria | Solution #3 (HSS based) | Solution #6 (New Rx query for PLMN-ID change) | Solution #7 (ULI based reporting) |
| --- | --- | --- | --- |
| Impact on Nodes and Interfaces | HSS: (i) On Cx interaction with I-CSCF needs to differentiate between S8HR and LBO. (ii) Needs to provide VPLMN-ID to S-CSCF on IMS registration. S-CSCF: Needs to forward VPLMN-Id to I-CSCF and then onto the P-CSCF P-CSCF: Needs to process VPLMN-ID from SIP message. Cx and SIP protocol | PCRF, P-CSCF: Needs to support new Rx procedure. Rx: New procedure to request for PLMN-Id Change | None (Standard Netloc functionality) VPLMN needs to support Netloc |
| Amount of additional signalling | None. (VPLMN ID is carried via existing SIP signalling interaction) | P-CSCF requests for VPLMN at initial IMS registration, and hence doubling the Rx interaction at registration with PCRF. Subscription to PLMN-ID change in access network | P-CSCF requests for VPLMN at every IMS registration, and hence doubling the Rx interaction at registration with PCRF. ULI information sent on IMS bearer establishment, modification, deletion. |
| Support for Lawful Interception | Since PLMN-Id is not available when SIP Register Request arrives at the P-CSCF but only when SIP response message from S-CSCF is received at the P-CSCF, the decision for LI is delayed till receipt of SIP response message from S-CSCF. The SIP Register message will need to be buffered at the P-CSCF till this point | Supported. | Supported |
| Limitations of the Solution | The solution cannot work if the UE access EPS simultaneously via more than one access network which use different PLMN-IDs | None | None |

************* END OF CHANGE *************