SA WG2 Meeting #114	S2- 161446

11 - 15 April 2016, Sophia Antipolis, France

Source:	Sprint

Title:	Solution – Common CP Functions for Multiple Network Slices

Document for:	Discussion and approval

Agenda Item:	NextGen/6.10.1

Work Item / Release:	NextGen/ Rel-14

Abstract of the contribution: This contribution proposes categorizing CP Network Functions into two sets, one set of categories of NFs that are common to all network slices and another set of categories of NFs that are specific to each network slice.

# 1 Introduction

The contribution builds on the scenarios depicted in Annex D for supporting multiple simultaneous network slices. Group B assumes that some Control Plane Network Functions are common between the network slices, while other CP Network Functions reside in its individual network slices. The common CP Network Functions are those related only to the state of UE when associated with a NextGen PLMN. All other Network Functions that are related to any specific sessions or specific services are associated to the CP of a particular network slice.

# 2. Proposal

It is proposed to add the following Solution to TR 23.799 “Study on Architecture for Next Generation System”.

* * * Start of changes * * * *

# 6	Solutions

Editor's Note: This section describes the solutions to the key issues and solutions to architecture design.

## 6.1	Solutions for Key Issue 1: Support of network slicing

### 6.1.X	Solution 1.X: Common CP Functions for Multiple Network Slices

This solution is to solve the key issue#1 on support of network slicing.

#### 6.1.X.1	Architecture description

In order to support the architecture principle of enabling a UE to simultaneously obtain services from one or more specific network slice instances of one operator, one set of CP Functions is common to all network slices while another set of CP Functions is allocated on a per network slice basis as shown in Figure 6.1.X.1-1. UP functions are always allocated on a per network slice basis. The UP functions that belong to a network slice may be influenced by CP functions in that slice as well as CP functions in the common set.

Editor’s Note: Any coordination between common CP functions and per slice CP functions to control the same UP function is FFS.

Shared AN

Per Slice CP NFs

Per Slice UP NFs

Per Slice CP NFs

Per Slice UP NFs

Common CP NFs

Slice 1

Slice 2

Figure 6.1.X.1-1 Multiple Networks Slices with Common CP NFs

CP functions that fall into a number of categories are common to all network slices. This set of common CP functions fall into the following categories:

-	Required for associating the UE device service subscription to a NextGen PLMN.

-	Network discovery and selection across multiple ANs and multiple RATs.

-	Tracking UE location and paging.

-	Required for coordination of services across network slices, e.g., subscription policies that cross multiple services.

The CP functions in a particular network slice fall the following categories:

-	Related to a PDU session management. This includes the network where the session is directed and address allocation/management.

-	Related to management of UP functions. This includes establishment/release of UP path, PDU forwarding, PDU screening and managing all UP charging and QoS policies.

-	Related to mobility management. This includes establishment or change of the mobility anchor, steps to preserve the address, and changes to UP functions to maintain efficiency.

The split between common and per slice categories for CP functions are detailed in the following table.

Table 6.1.X.1: Common vs. Slice Specific CP Function Categories

| CP Function Categories | Common | Per Network Slice |
| --- | --- | --- |
| UE Registration Manage Identities Do Authentication/Authorization Manage Service Profiles | X |  |
| Reachability Knowledge Do area tracking Control paging | X |  |
| Global Policy Management (across services for UE) For Multi-AN, Multi-RAT discovery & selection polices Service Authorization and Policies | X |  |
| PDU Session Management Determine Data Network Type/Name Allocate/Release UP functions Manage address allocation/release |  | X |
| UP Functions Management Control PDU forwarding Control PDU screening Manage Charging rules & policy enforcement |  | X |
| Mobility Management Adjust Mobility Anchor location Initiate address preservation upon HO Make UP changes to maintain efficiency upon HO |  | X |

Editor’s Note: Further additions to CP Function categories in table 6.1.X.1 is FFS.

#### 6.1.X.2	Function description

Editor's Note: This clause will contain function descriptions and the interactions among the network functions.

#### 6.1.X.3	Solution evaluation

Editor's Note: This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.

* * * End of Changes * * *