SA WG2 Meeting #S2-114	                                                                          S2-161720

11 - 15 April 2016, Sophia Antipolis, France

Source:	Huawei, HiSilicon, China Telecom, China Unicom

Title:	Discussion on MT SMS over SGs in eDRX

Document for:	Discussion and Decision

Agenda item:	5.1

Work Item / Release:	eDRX/Rel-13

Abstract of the contribution: This discussion paper attempts to provide detailed analysis on the necessity of updating the legacy CS infrastructure end-to-end for MT SMS over SGs just due to deploying the eDRX.

1. Introduction

This discussion paper attempts to provide detailed analysis on the necessity of updating the legacy CS infrastructure end-to-end for MT SMS over SGs just due to deploying the eDRX.

2. Discussion

  - 2.1 Original problem found by SA2

The original problem for MT SMS delivery in eDRX found by SA2 is:

- MT SMS retransmissions can occur for UEs being temporary unreachable due to eDRX (e.g. eDRX cycle value used by the MME/UE is longer than the retransmission timers in the SMS infrastructure).

This original problem actually is NOT a new issue introduced due to eDRX but can today occur in the field in many other cases, e.g. the UE is temporarily out of coverage, the UE is in the power saving mode (i.e. PSM).

The severity of this problem depends on how often this can occur in an eDRX deployment.

The eDRX is mainly to reduce the UE power consumption in idle mode. Hence, like for PSM in Rel-12, eDRX is more suitable to be deployed for MTC/CIoT devices (like smart meters) which has infrequent and small data transfer.

For devices that have more frequent data transport (like smart phone), no much benefit is gained by deploying eDRX or PSM due to the long/heavy MT data buffering which does impact the user experience very much.

Observation I: the original problem is NOT a new or even more often problem for MT SMS delivery in eDRX.

  - 2.2 New problems identified by CT1

SA2 newly added an MT SMS over SGs procedure (i.e. TS 23.272 subclause 8.2.4a) to avoid the MT SMS retransmissions (the original problem).

CT1 has identified some new problems due to the SA2 newly added MT SMS over SGs procedure, typically the problem results in additional signalling towards the HSS/HLR (i.e. the steps 3, 8, 12, 13 in the below Figure).

SA2 was not aware of the problems added by the new MT SMS over SGs procedure for eDRX at the very beginning but were identified by CT1 and then reported to SA2 by LS.

Also, for this new MT SMS over SGs procedure, SA2 introduced a new SGs cause for eDRX from the MME but did not describe how the MSC/VLR handles this new SGs cause.

At the same time, SA2 clearly indicated (by the below NOTE) that all CS network entities (i.e. MSC/VLR, HLR, SMS-GMSC and SC) in the new MT SMS over SGs procedure perform the legacy behaviour as today.

“NOTE:	Steps 7-9 are as per legacy behaviour of MT SMS delivery failure as defined in TS 23.040 [14].”

Observation II:  based on the new MT SMS over SGs procedure (TS 23.272), nothing new is actually done for the legacy SMS system for eDRX.

  - 2.3 Same problem exists for the Rel-12 PSM feature

Before eDRX, 3GPP completed the Rel-12 PSM feature which reduces UE power consumption.

PSM and eDRX share some common characteristics, e.g. both need to be negotiated between the UE and the CN nodes but only limited to MME/SGSN (i.e. MSC/VLR is excluded), both of them are intended for UEs that are expected to have only infrequent MO/MT services, including MT SMS.

When PSM is activated, the UE is unreachable and the MT SMS retransmissions (the original problem) can also occur when PSM is used.

However, for PSM, no new procedure for MT SMS over SGs delivery was introduced.

The current network behaviour for MT SMS delivery in PSM is up to network implementation (see below text in stage 3 CT1 TS 24.301):

“The network behaviour upon expiry of the active timer is network dependent, but typically the network stops sending paging messages to the UE on the first expiry, and may take other appropriate actions.”

Observation III:  eDRx should adopt similar handling as PSM for the MT SMS over SGs delivery.

  - 2.4 Legacy impacts of solutions tabled in (S2-160301, S2-160302, and S2-160474)

Note that all three set of proposals from CRs (S2-160301, S2-160302, and S2-160474, respectively) have to be deployed together to make sure all identified problems are resolved.

The proposals require upgrade to the legacy CS infrastructure end-to-end (i.e., MSC/VLR, HLR, SMS-GSMC, SGs interface and the D/E interfaces. Note that this would not be the case for similar feature; PSM).

The solutions can also create unnecessary signaling load due to the SMS-GMSC will always include the new proposed parameters to the MSC/VLR and MME regardless whether eDRX is used at the MME or not (the SMS-GMSC does not know whether eDRX was used at the MME so far. Even the SMS-GMSC knows the MME has deployed the eDRX feature, however the SMS-GMSC cannot know whether the MME has enabled the use of eDRX for the specific UE).

During the discussion in CT1#96 meeting, though the optimizations are optional, it was commented that the HPLMN which has deployed this optional enhancement cannot get the benefit if the VPLMN has deployed the eDRX feature but not deployed this optional enhancement. Hence, the optionality is questioned by some operators (who do not want to upgrade legacy infrastructure just for eDRX).

3. Proposed alternative solutions

The more details of this alternative solution can be seen in the Annex which proposed the changes to the existing text in TS 23.272.

The MME can immediately provide feedback to the MSC/VLR for the SGs paging for MT SMS to enable the MT SMS delivered to the MME (steps 6a and 6b in the below Figure).

The MME knows when the UE is reachable based on the negotiated eDRX cycle value and hence can buffer the received MT SMS until that time.

When the UE becomes reachable (i.e. responds to paging), the MME immediately sends the buffered MT SMS to the UE. The subsequent procedures are performed as legacy.

This solution needs NOT so much additional standard work in 3GPP due to it can be fully achieved by MME internal implementation. Hence, the legacy CS SMS infrastructure impacts are then avoided.

So far, the delivery time for MT SM at the SMS-GMSC is not specified in the standard and can be configured with different values per operator policies. For the MT SM delivery via IP-SM-GW, a max. delivery time for MT SM used at the SMS-GMSC can be 600s. Hence for an operator interested to deploy the eDRX, it is reasonable to configure a longer delivery time for MT SM at the SMS-GMSC, e.g. >10m or even 30m or 1h.

Now if we see the defined eDRX cycle values used in S1 mode in the below table (see Extended DRX parameters IE definition given in stage 3 CT1 TS 24.008), for the 10 values, there are 7 values are shorter than 600s while only 3 values are longer than 600s. In case of an operator interested to deploy the eDRX configured a longer delivery time (e.g. 1h), the SM retransmission can be totally avoided then.

| S1 mode The field contains the eDRX value for S1 mode. The E-UTRAN eDRX cycle length duration valueand the eDRX cycle parameter 'TeDRX' as defined in 3GPP TS 36.304 [121] are derived from the eDRX value as follows: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| bit |  |  |  |  |  |
| 8 | 7 | 6 | 5 | E-UTRAN eDRX cycle length duration | eDRX cycle parameter 'TeDRX' |
| 0 | 0 | 0 | 0 | 5,12 seconds | 20 |
| 0 | 0 | 0 | 1 | 10,24 seconds | 21 |
| 0 | 0 | 1 | 0 | 20,48 seconds | 22 |
| 0 | 0 | 1 | 1 | 40,96 seconds | 23 |
| 0 | 1 | 0 | 0 | 81,92 seconds | 24 |
| 0 | 1 | 0 | 1 | 163,84 seconds | 25 |
| 0 | 1 | 1 | 0 | 327,68 seconds | 26 |
| 0 | 1 | 1 | 1 | 655,36 seconds | 27 |
| 1 | 0 | 0 | 0 | 1310,72 seconds | 28 |
| 1 | 0 | 0 | 1 | 2621,44 seconds | 29 |

4. Proposed way forward

Several operators do not want to invest in legacy CS infrastructure just due to deploying eDRX.

The operators who have interest to deploy eDRX in Rel-13 can use the alternative solution proposed in section 3 for MT SMS over SGs. This solution does not impact the egacy CS infrastructure and hence do not require additional big standard work (with some simple clarification is enough).

We propose that the network adopts similar handling as PSM for the MT SMS over SGs in eDRX, i.e. the network can take any appropriate actions (e.g. including MME network implementation, delivery time configuration for MT SM at SMS-GMSC taking eDRX into account).

The whole eDRX work was 100% complete in both stage 2 and stage 3, hence any proposals which require updating the legacy CS network domain should not be allowed as several operators reject investing in CS infrastructure just due to deploying eDRX.

Only if 3GPP needs to specify some behaviour, SA2 is the appropriate group to discuss the problems and decide workable solutions since the problems were introduced in TS 23.272 which is under SA2 responsibility. The CR# 0962 (S2-161432) tabled for TS 23.272 covers the alternative solution proposed in section 3 and in the Annex.

******************************************************************************************************************************

## Annex: Alternative Solution

Changes for TS 23.272:

### 8.2.4a	Mobile Terminated SMS in extended Idle mode DRX

The following sequence flow shows the delivery of mobile terminating SMS when the UE is using extended idle mode DRX (as defined in TS 23.682 [48]) with a DRX cycle value that can cause mobile terminating SMS retransmissions.

Figure 8.2.4a-1: Mobile terminating SMS in extended idle mode DRX

1.	The combined EPS/IMSI attach procedure as described in clause 5.2 has been performed, and extended Idle mode DRX has been negotiated.

2 4.	The SC initiates transfer of mobile terminating SMS. The HLR is requested for routing number for SMS services and the SMS message is forwarded to the MSC/VLR where the MS/UE is CS attached.

5.	The MSC/VLR sends a Paging (IMSI, VLR TMSI, Location Information, SMS indicator) message to the MME.

6a.	If the UE is considered unreachable for paging due to using extended idle mode DRX (as specified in TS 23.682 [48]), the MME sends a Service Request message to the MSC/VLR. If the MME proceeds with step 6a, steps 6b-16 as shown in Figure 8.2.4a-1 and described below are performed.

Otherwise steps 6-16 of clause 8.2.4 are performed.

6b.	The MSC/VLR builds the SMS message to be sent as defined in TS 23.040 [14] (i.e. the SMS message consists of CP-DATA/RP-DATA/TPDU/SMS-DELIVER parts). The MSC/VLR forwards the SMS message to the MME in a Downlink Unitdata message. The MME stores the received SMS message..

7a.	The MME sends the paging request just before the occurrence of next paging occasion.

7b.	The MS/UE is paged by the eNodeBs.

8.	The UE sends a Service Request message to the MME. The UE indicates its S TMSI in the RRC signalling. The MME sends the S1-AP Initial Context Setup Request message to the eNodeB and the eNodeB establishes the Radio Bearers. .

9a.	The MME encapsulates the SMS message in a NAS message and sends the message to the MS/UE.

9b, 9c. The MS/UE acknowledges receipt of the SMS message to the MSC/VLR.

10.	The MS/UE returns a delivery report as defined in TS 23.040 [14]. The delivery report is encapsulated in an NAS message and sent to the MME.

11.	The MME forwards the delivery report to the MSC/VLR in an Uplink Unitdata message.

12-13. These steps are performed as defined in TS 23.040 [14]. The delivery report is forwarded to the SC.

14-15. In parallel to steps 12-13, the MSC/VLR acknowledges receipt of the delivery report to the MS/UE.

16.	The MSC/VLR indicates to the MME that no more NAS messages need to be tunnelled.

The MME should not use the SGs Release Request message as a trigger for the release of S1 resources.

NOTE:	Although the MME could use the RRC establishment cause (set to mt-Access) sent in the S1-AP Initial UE Message in step 8 to deduce that the MS/UE sent the Service Request solely for receiving the SMS), the MME does not know whether the MT-SMS might cause the initiation of user plane traffic or an MO SMS. So, for simplicity of MME implementation, the normal eNodeB procedures should be used to initiate the release of S1 resources.