SA WG2 Meeting #114	S2- 162159

11-15 April 2016, Sophia Antipolis, France	revision of S2-161494

Source:	Qualcomm Incorporated

Title:	Solution X for Key Issue 4 on Session Management

Document for:	Discussion/Approval

Agenda Item:	FS_NexGen /6.10.4

Work Item / Release:	FS_NexGen / Rel-14

Abstract of the contribution: Proposes a new solution for key issue 4, focusing on separation of Session Management and Mobility Management and the support of multiple sessions for each data network connection.

# Introduction

This solution addresses key issue 4 on “session management” in TR 23.799.

It has to be noted that this solution also addresses parts of key issue 1 on network slicing, and specifically how to enable operators to use the network slicing concept to efficiently support multiple 3rd parties (e.g. enterprises, service providers, content providers, etc.) that require similar network characteristics.

Also, the solution addresses some of the issues in Key issue 5 on “Enabling (re)selection of efficient user plane paths”, and specific provides a mechanism for selecting an efficient user plane path for establishing connectivity to a data network.

# Proposal

It is proposed to add the following solution to the TR 23.799 “Study on Architecture for Next Generation System”.

>>>Start of Changes<<<<

## 6.4	Solutions for Key Issue 4: Session management

### 6.4.X	Solution 4.X: Session Management for Data Network Sessions

Editor's Note: This section describes the solutions to the key issues and solutions to architecture design.

This solution applies to key issue 8 on session management, and parts of key issue 1 on network slicing. Specifically, the solution addresses the session management model, the correlation between session management and mobility management functionality, and to key issue 1 on how to enable operators to use the network slicing concept to efficiently support multiple 3rd parties (e.g. enterprises, service providers, content providers, etc.) that require similar network characteristics.

#### 6.4.X.1	Architecture description

Editor's Note: This clause will contain e.g., terminology, overview, architecture description of the solution.

Editor's Note: non-IP session management is FFS.

In this solution, a UE may establish multiple PDU sessions to the same data network. In addition, different UP-GWs may be allocated for the different PDU sessions to the same data network.

The solution is based on the following architecture:

Figure 6.4.X-1 Session Management high level architecture.

The functional entities in the architecture are defined as follows:

- CP-MM: the Control Plane Mobility Management function is in charge of establishing and maintaining the MM context for a device attaching to the NextGen network,including interacting with the CP-AU to authenticate the device. The CP-MM also implicitly interacts with CP-SM to validate the establishment of SM context for a UE that has securely established an MM context.

Editor’s note: whether one or multiple CP-MM instances are applied when the UE connects to multiple network instances/slices is FFS and depends on the definition for the architecture for network slices.

- CP-SM: the Control Plane Session Management function is responsible for establishing, maintaining and terminating PDU sessions on-demand for the UE in the NextGen system architecture,.

- CP-AU: the Control Plane Authentication Management function is responsible for authenticating the UE...

- AAA: profile repository and authentication server, stores the subscriber profile and the subscriber credentials corresponding to a specific set of credentials.

The solution is based on the following model:

- For each network slice, the UE has a single MM context established with a CP-MM function

Editor’s note: this assumption is dependent on the solutions for the key issue on network slicing, and may need to be revisited when such solutions are defined.

- A UE can have a MM context established with the network without requiring the establishment of any PDU sessions, but a PDU session can be established simultaneously to an MM context.

- For each MM context, a UE can associate multiple SM contexts (i.e. PDU sessions)

- The attempt to establish any PDU sessions in relation to an MM context is authorized based on the UE previously having established an MM context

- In this solution, it is assumed that the UE is aware of the connectivity requirements for applications/services (e.g. what data network is required for an application/service, what specific connectivity requirements wrt the transport, e.g. the type of session continuity for the IP flows, etc.). It is furthermore assumed that when applications require connectivity, they either provide specific connectivity requirements (e.g. regarding session continuity; type of connectivity in terms of local breakout, remote; etc.) or the UE is configured with information regarding connectivity requirements of specific applications.

Editor's Note: The exact relationship between session management and session continuity requires further study and depends on the solutions developed for session continuity.

Editor's Note: It is FFS whether and how the UE can be configured with such information, if applications cannot provide such requirements. E.g., in the same way that in EPC it was possible to define per-application ANDSF policies based on application ID, similarly per-application ID connectivity requirements could be provided in the UE.

- UP-GW selection and allocation is performed upon establishment of a PDU session. UP-GW selection for a PDU Session is performed considering the connectivity requirements of the PDU Session to enable optimized establishment of the data path for each PDU Session.

- The UE can establish multiple PDN Sessions to the same data network. Different PDU sessions connecting to the same Data Network may be served by different UP-GWs and, in the case of IP PDU sessions, may have different IP addresses.

- A UE may be served by different CP-SM functions for different DN Sessions (e.g. if specialized features need to be implemented by CP-SM for specific services).

In this solution, the establishment of a PDU session may optionally be authorized/authenticated by the external DN via the session management functionality , similarly to how a PDN connection was authorized by an external data network:

- This enables the external data network to provide service-specific information for the data flows for the data network including QoS requirements, packet filtering information, e.g. for encrypted data, etc., to the CP-SM

- The NextGen CN of the MNO uses such information as input for e.g. QoS establishment and to perform packet filtering. Establishment and authorization of connectivity in an operator network is maintained under the control of the operator in the serving CP-SM, since the UE has a single active operator subscription being used with the operator network.

The following figure describes the functional allocation for this session management solution.

Figure 6.4.X-2 Functional Distribution for Session Management.

#### 6.4.X.2	Function Description

Editor's Note: This clause will contain function descriptions and the interactions among the network functions.

Editor’s note: the specific protocols for DN SM context establishment are FFS. It is expected that a subset of the current NAS functionality will be supported, independently of the protocol supported, depending on the solutions designed for other key issues (e.g. QoS, congestion control, etc.).

#### 6.4.X.2.2	Establishment of a PDU Session

Once a MM context has been established, if specific connectivity requirements arise in the UE for an application or service, the UE establishes a PDU Session

In addition, the UE may request additional PDU sessions for a data network even when the UE already has one or more PDU sessions for the same data network. This may happen as an example when an application has connectivity requirements (e.g. session continuity) that are not satisfied by any of the existing PDU session(s).

Figure 6.4.X-3 PDU Session Establishment.

In this solution the following steps are performed:

- the UE establishes a Mobility Management (MM) context via MM procedures with the CP-MM, including authentication.

Editor's Note: The details of the establishment of the MM context are defined in the solutions for MM key issue.

- The UE sends a PDU Session Establishment Request to the CP-MM, and may include information describing the service and connectivity requirements for the  PDU session establishment. The UE may also authentication informationto be used to authorize the PDU Session Establishment via the external data network.

- The CP-MM selects the CP-SM function to serve the UE, and forwards the request to the selected CP-SM.

- The CP-SM verifies the UE request for the PDU Session based on the information provided by the UE with respect to the UE profile, and whether there are already existing PDU sessions for the same data network. The CP-SM may optionally trigger the authentication of the PDU Session Establishment Request with the external data network based on the authentication information provided by the UE,During the authentication procedure, the data network may provide a connectivity profile to the CP-SM containing service specific requirements (e.g. QoS, connectivity type, etc.). If the UE already has PDU sessions to the same data network, the CP-SM verifies if additional PDU sessions are allowed.

- The CP-SM may interact with the policing function to determine the policies to be applied to the PDU Session.

- The CP-SM establishes the resources required for the PDU Session. For an additional PDU session to the same data network, the CP-SM (e.g. based on the connectivity requirements, the subscription profile, etc.) may select a different UP-GW to serve the new PDU session.

6a.	For the first PDU session, the CP-SM selects a UP-GW. For additional PDU sessions to the same Data Network, the CP-SM verifies if the same UP-GW can be used. The CP-SM interacts with the UP-GW for the establishment of the PDU Session (e.g. IP address allocation for PDU Session for IP traffic)

6b.	If a new UP-GW is required, the CP-SM selects a new UP-GW (e.g. based on the information describing the service, the requirements for the DN session and the PDU Session, the connectivity profile, etc.) and interacts with the UP-GW for the establishment of the PDU Session (e.g. IP address allocation for PDU Session for IP traffic).

6c.	The CP-SM interacts with the AN for the establishment of the PDU Session.

- The CP-SM confirms to the UE the establishment of the PDU Session and includes information related to the PDU Session contexts (e.g. IP addresses, etc.).

# >>>End of Changes<<<<