# 8	Conclusions

Editor's note:	This clause is intended to list conclusions, interim or/and final conclusions, which have been agreed during the course of the work item activities.

## 8.x	Interim Agreements on QoS Key issue #2

The following table documents the current status of agreements on the CN-RAN functional allocation:

- The QoS parameters that will apply for a given flow of PDU are:

- Delay

- Packet priority

- Discard priority

- GBR

- Priority, Pre-emption, Retention

- Within a PDU session, there are neither bearers in NG Core nor in the NG Core to RAN interface. Instead, DL packet marking, from Core to RAN is used:

- From DL traffic sent by CNUP/UPGW to RAN, 2 levels of packet marking may be supported:

    - Packet marking at pure transport level. This level of marking may be modified by intermediate routers/switches between the UPGW and RAN

    - Packet marking that cannot be modified by intermediate routers/switches between the UPGW and RAN.

- Support Reflective QoS over radio under control of the network. The network decides on the QoS to apply, informs the UE (i.e. in-band signalling) and the UE mirrors the QoS.

  - This does not apply to IP flows that simultaneously support applications with different QoS requirements

| Location: Function: | NextGen RAN | NextGen CN | Comments |
| --- | --- | --- | --- |
| Key Issue #1 – Network Slicing |  |  |  |
| CN instance selection when UE attach to a CN network slice | FFS | FFS |  |
| Key Issue #3 – Mobility Management |  |  |  |
| Mobility management control, (Subscription and Policies) |  | X |  |
| Determination of mobility restriction |  | X |  |
| Roaming restrictions execution |  | X |  |
| Mobility restrictions execution, [CN Connected] | X |  |  |
| Mobility restrictions execution, [CN Idle] |  | X | If CN Idle exists. |
| UE registration |  | X |  |
| Area tracking | FFS | FFS | The need for Hierarchical tracking is FFS. Depends also on the existence of a CN Idle state |
| UE unreachability detection |  | X | Assumed to be supported in CN for UEs in CN Idle state. If CN Idle exists. |
| RAN UE unreachability detection | X |  | Assumed to be supported in RAN for UEs in RAN Inactive state. If RAN inactive state exists. |
| NAS state transitions |  | X |  |
| RRC state transitions | X |  |  |
| Paging initiation and control in RAN Inactive state | X |  | RAN Inactive state is RAN state that corresponds to CN connected state. If RAN inactive state exists. |
| Paging initiation in CN Idle state |  | FFS | CN Idle state is FFS. Some companies assume that the CN Idle state is replaced by a RAN "Inactive" State. Some companies think that the CN Idle state remains but is used more seldom due to the new RAN "inactive" state |
| Access Stratum UE Context storage in RAN Inactive state | X |  | If RAN inactive state exists. |
| Control of connected state mobility | X | X |  |
| UP buffer for UE in CN Idle state |  | FFS | Same comment as on Paging initiation in CN Idle state |
| UP buffer for UE in RAN Inactive state | X |  | If RAN inactive state exists. |
| Key Issue #4 - Session Management |  |  |  |
| PDU Session address allocation |  | X | FFS for non-IP PDU Sessions |
| PDU Session Termination Point |  | X | Note that this refers to the ownership of the specification for the function supporting the termination point. In a NW deployment this function may be deployed on or close to a RAN site. |
| Session Management |  | X |  |
| Termination of UP security | FFS | FFS | FFS |
| Subscription Data Handling (incl. default QoS profile) |  | X |  |
| Key issue #12 |  |  |  |
| Authentication and Key Agreement |  | X |  |
| Key Issue #2 QoS |  |  |  |
| Radio Resource Admission Control | X |  |  |
| Radio Resource management (QoS attributes) | X |  | Packet scheduling with regards to resource utilization and availability (RRM) |
| Max rate control | X | X | Maximum bitrate policing in the CN and RAN in UL and DL. |
| QoS Policy Control |  | X |  |
| Transport marking | X | X | Used for prioritization in the transport network. |
| Charging Data Collection |  | X |  |
| Packet classification of DL packets for QoS differentiation on the Radio | FFS | FFS | Some companies think the QoS classification for QoS differentiation of DL packets is performed in RAN. |
| QoS differentiation and verification for UL packets | FFS | FFS | Some companies think the QoS verification for UL packets is performed in RAN and/or CN. |

## 8.x	Interim Agreements on QoS Key issue #2

This section documents the interim agreements for QoS supported in the NextGen System:

- QoS Policy control is supported in the NextGen core network.

- Following QoS policies are supported:

- Guaranteed Flow Bitrate (GFB)

- Maximum Flow Bitrate (MFB)

- Allocation and Retention priority.

- Delay budget