SA WG2 Meeting #115	S2-162902

23 - 27 May 2016, Nanjing, P.R. China	(revision of S2-162341)

Source:	Qualcomm Incorporated

Title:	Solution for key issue 2: Dynamic QoS Differentiation

Document for:	Discussion/Approval

Agenda Item:	FS_NexGen /6.10.2

Work Item / Release:	FS_NexGen / Rel-14

Abstract of the contribution: Proposes a QoS model for the Key issue 2 on QoS framework, with special focus on traffic identification and separation.

# Introduction

The proposal in this paper complements solutions 2.1 and 2.2 for QoS captured in the TR, and builds on both solutions. Only the delta with respect to such solutions is described.

The solution in this paper introduces:

- a set of parameters for QoS signalled when QoS is triggered by an AF

- a set of parameters for QoS used for more granular traffic differentiation than allowed by solutions 2.1 and 2.2

- a mechanism for packet marking to enable traffic differentiation for packets belonging to the same IP flow

- a mechanism to enable differentiated QoS treatment of encrypted multimedia traffic (e.g. video streaming).

## 1.1	Differentiated Packet Treatment

Traditionally, QoS in 3GPP networks has been defined and enforced considering the QoS of IP flows, without differentiating different IP packets in a specific IP flow.

A type of traffic that requires such differentiated traffic is video streaming, though other types of traffic may also have similar requirements. In particular:

- in certain streaming flows, some IP packets are more important than others. In particular, it is essential to guarantee delivery of certain packets, whereas other can be sacrificed, in order to maintain a fluid streaming experience. The difference between these packets is visible only above L4, whereas they have the same L3 and L4 characteristics, and therefore current QoS mechanisms cannot differentiate such packets.

- In other applications (e.g. streaming mechanisms), the UE retrieves packets from a server in blocks. Each block has a specific length and a specific delivery deadline, after which the packets are not useful anymore and the streaming experience is interrupted. The priority of packets in a block may change therefore during the lifetime of the IP flow, requiring a QoS mechanism that allows the QoS treatment of a data flow to be modified dynamically and possible temporarily without requiring a QoS renegotiation with the CN. In particular, if the deadline for a block of packets is approaching, it is desirable to ensure delivery of such packets with a priority higher than normal, if radio conditions allow it; on the contrary, if the deadline is far, normal packet delivery (e.g. scheduling in RAN) can take place. However, neither the CN nor the AN are aware of such deadline or urgency to deliver the packets.

A further complication for traffic differentiation is the trend of content providers to move towards streaming encrypted content (e.g. protected with HTTPS). When the content is protected (either with HTTPS or with other encryption mechanisms) between the UE and the server, the 3GPP network nodes have visibility only of L3 and L4 information about the packets in a flow, and therefore any sort of differentiated packet treatment is not possible.

The following gaps or drawbacks have been identified with solutions 2.1 and 2.2:

- It is not clear how any traffic differentiation, especially in congestion conditions, can be performed with encrypted traffic

- current solutions in the TR do not address how QoS differentiation between different packets within the same IP flow can be done unless DPI is performed. This includes DPI in the AN when AN congestion takes place, or the CN conveying to the AN some additional marking resulting from DPI in the CN. This needs to be performed also for encrypted traffic. As an example, in solution 2.2 the Flow Priority Label (FPL) is defined to help the AN handle the traffic in a data flow in terms of pre-emptions or protection from pre-emption. Such parameter, for traffic differentiation above L4, is ineffective unless DPI is performed in the AN.

# Proposal

It is proposed to add the following solution to the TR 23.799 “Study on Architecture for Next Generation System”.

# >>>Start Changes<<<<

## 6.2	Solutions for Key Issue 2: QoS framework

### 6.2.X	Solution 2.X: QoS model for flexible traffic differentiation

#### 6.2.X.1	Architecture description

Editor's Note: This clause will contain e.g., terminology, overview, architecture description of the solution.

##### 6.2.X.1.1 Functional Architecture

The functional architecture of solution 2.2 is assumed.

##### 6.2.X.1.2 Applications Requirements Input

Solution 2.1 defines a set of application requirement inputs provided by an AF to the CP Function for QoS in section 6.2.1.1.1, and defines in section 6.2.1.2.2 how the QoS authorization based on application requirements takes place.

In addition to what is proposed already, this solution proposes that the AF provides to the CP Functions also an indication of whether different packets within a data flow require differentiated treatment. E.g., for a data flow, the AF may provide two levels of QoS requirements in terms of Service Behavior, Service Requirements, Delay Requirements and Priority for solution 2.1, or in terms of two levels of bitrate for the same IP flow in case of solution 2.2.

##### 6.2.X.1.3 Packet Markings for Traffic Differentiation

A new packet marking that conveys in-band information related to the data flow features (including information describing packet characteristics above layer 4) is introduced in order to enable easier traffic differentiation when traffic cannot be easily detected (e.g. for encrypted traffic) and to enable differentiated treatment of different packets belonging to the same data flow (e.g. packets that have different characteristics at level 5 and above and that require different treatment).

Editor’s Note: if packetization of data traffic in an application server includes e.g. high priority traffic and low priority traffic in the same packet, it is FFS whether it is possible to provide differentiated treatment for such traffic.

Editor’s Note: it is FFS to what media types the solution applies.

Upon definition of the QoS Policy or Authorized QoS in solutions 2.1 and 2.2 in collaboration with the AF (i.e. selection of QoS parameters defining the QoS for a data flow according to the QoS solution and the related parameters), the CP Function also generates a Packet Marking Token associated with the IP traffic.

The solution is based on:

- The presence of an AF that supports this packet marking solution and that interacts with the NextGen CN over NG5 to establish the QoS and packet treatment for the traffic

- The AF marking packets with the proposed Packet Marking Token over the NG6 interface.

Editor’s Note: it is FFS how the Packet Marking Token is carried over NG6. E.g. for IPv6, extension headers can be used.

- The UP-GW that terminates NG6 receives the packet and, based on the Packet Marking Token, identifies the corresponding QoS policy and packet treatment. The UP-GW marks the packet towards the AN with mechanisms defined in other solutions.

Editor’s Note: whether the current markings proposed in other solutions are sufficient to convey the information per in-flow packet differentiation is FFS. E.g. different FPI values could be applied to different packets of the same data flow.

- The marking performed by the UP-GW is carried to the RAN which can enforce QoS as defined in other solutions.

The Packet Marking Token is defined and used as follows:

- Upon definition of the QoS Policy or Authorized QoS in solutions 2.1 and 2.2 in collaboration with the AF, the CP Function also generates downlink Packet Marking Token(s) associated with the data traffic. This may include Packet Marking Tokens for potential future traffic exchanged by the UE and the AF.

- Each Packet Marking Token contains

  - A Token ID, identifying the token and the corresponding QoS policy

  - A Token Parameter Index (TPI) indicating which fields in a data flow header are used to compute the token hash

  - A Token Hash, calculated using the fields in each packet of the data flow described by the TPI

- The Packet Marking Token is generated via a cryptographic hash with a key decided by the CP Function and provided to the entities that need to verify the token

- The Packet Marking Token is appended to each packet of a data flow over NG6. Verification of a Packet Marking Token appended to the packet enables the UP-GW enforcing QoS and performing traffic filtering to identify the QoS policy associated with the packet without the need for TFTs.

Editor’s note: security aspects of this solution, e.g. how to securely generate, allocate, verify, and provision tokens to the UE will need to be studied by SA3.

- In the QoS Policy or Authorized QoS, the CP Function:

  - for each data flow or specific set of packets in a data flow for which a downlink token has been generated, it provides to the UP-GW the Token ID, the information for token verification (e.g. the key used to generate the token), in addition to the remaining QoS information.

  - for each data flow or specific set of packets in a data flow for which a downlink token has been generated, it provides to the AF the downlink Packet Marking Token .

- Tokens are generated as follows:

  - if for a data flow the AF provides a single requirement for packet treatment, e.g. flow descriptor/packet filter and the related QoS requirements, the CP Function generates a single downlink (DL) Packet Marking Token .

  - if for a data flow the AF provides an indication that packets within the data flow require different QoS treatment, and the packets differentiation is at a layer above L3 and L4, and provides requirement for each required level of packet treatment, the CP Function generates a separate downlink Packet Marking Token for each different level of treatment. The CP Function indicates to the AF which Packet Marking Token shall be used for each different level of QoS treatment. When packets marked with the different Packet Marking Tokens corresponding to the different levels of treatment reach a the  UP-GW, the correct QoS policy can be applied.

- For each packet received containing a Packet Marking Token, the UP-GW retrieves the information corresponding to the Token ID, and computes the hash of the fields in the packet identified by the TPI using the token verification key provided by the CP Function. If the token is verified, the UP-GW does not need to perform any further packet inspection and it applies the QoS policy associated with the token.

Editor’s Note: whether and how the solution works in the presence of middleboxes on NG6 is FFS.

#### 6.2.x.2	Function description

Editor's Note: This clause will contain function descriptions and the interactions among the network functions.