SA WG2 Meeting #116	S2-163713

11 - 15 July 2016, Vienna, Austria	(revision of S2-16xxxx)

Source:	Deutsche Telekom AG

Title:	Conclusion for SeDoC roaming to SeDoC

Document for:	 Approval

Agenda Item:	6.11

Work Item / Release:	FS_SeDoc / Release 14

Abstract of the contribution: This contribution considers roaming where both, the HPLMN and the VPLMN, are enhanced for SeDoC.

# Discussion

The TR on SeDoC analysis roaming scenarios where either the HPLMN or the VPLMN are enhanced for SeDoC. In the case where only one network is enhanced for SeDoC an interworking function is needed to map legacy CS network protocols and procedures to IMS profiles and procedures.

When both, the HPLMN and the VPLMN, are enhanced for SeDoC no interworking function is needed between these networks. IMS procedures and protocols can be used the same way as they are used for non-roaming SeDoC scenario. In mixed scenarios where a network operator has roaming to/from enhanced and non-enhanced networks it has to be taken into account which type of roaming network the roaming subscriber belongs to.

For deployments where home and visited networks are enhanced for SeDoC no additional standardization is needed. SeDoC procedures and protocols for non-roaming scenarios can be re-used for roaming between SeDoC enhanced networks.

# Proposal

It is proposed to add following text to clause 8 "Conclusions" in TR 23.719

Start of change

For roaming scenarios where the home and the visited network are enhanced for SeDoC no additional standardization is necessary. Protocols and procedures developed for SeDoC non-roaming cases can be re-used for roaming scenarios when both, the VPLMN and the HPLMN are enhanced for SeDoC.

End of change