SA WG2 Meeting #116	S2-164068

11 - 15 Jul 2016, Vienna, AT	(revision of S2-163678)

Source:	NTT DOCOMO

Title:	Update solution 4.2: Session setup procedures

Document for:	Approval

Agenda Item:	6.10.4

Work Item / Release:	NextGen/R-14

Abstract of the contribution: Addressed the editor’s note in section 6.4.2.2.1.

# Introduction

# Proposal

The contribution proposes to address the section 6.4.2.2.1 editor’s note. It is proposed to add the following changes to TR 23.799.

* * * Start of changes * * * *

### 6.4.2	Solution 4.2: Session setup procedures

This solution addresses the Key Issue 4: Session management, specifically, on the session setup procedures.

#### 6.4.2.1	Architecture description

Session management of next generation system may not be involved during the attach procedure. A UE may request session establishment procedure under the following states:

a)	Unattached, session established with attach procedure (EPS-like session establishment).

b)	Attached but no session established, the UE already attached to the network and would like to get IP/Non-IP connection for data transfer.

c)	Already have session established, the UE already have session established over one or multiple DN(s), and would like to have another session.

Editor's note:	It is FFS whether the same UP function is selected for all sessions to a single DN or whether separate UP functions may be selected is FFS.

Therefore, keeping at least one session is not mandatory but if needed for the desired service (e.g. eMBB service), it should be controlled as always-connected session.

Editor's note:	It is FFS whether always-connected session is requested by UE or based on per subscription.

The session management architecture could be depicted as Figure 6.4.2-1, where the SM is the function in control plane for session management handling.

The procedures below assume that a user plane tunnel is established between AN and the user plane functions in the core network.

Editor's note:	The "User Data" in the figure below is the data repository of information related to session management and user subscription. Whether it is a standalone network function or collocated with some network function is FFS.

Figure 6.4.2.1-1: The non-roaming architecture for session management

The home-routed roaming architecture is shown in Figure 6.4.2-2.

Figure 6.4.2.1-2: The home-routed roaming architecture for session management

Editor's note:	The architecture figures above describe the logical relation between functionality for session management. Reference points used for session management for NextGen architecture and additional description of relation to other functionality in NextGen system such as mobility management functionality and is FFS.

Editor's note:	Whether a "User Data" function needs to be added to the roaming architecture for session management is FFS.

Editor's note:	Whether there is a need for a Policy control function in the VPLMN is FFS and also handled as part of the policy framework key issue.

#### 6.4.2.2	Function description

##### 6.4.2.2.1	Session establishment procedure, non-roaming case

A session can be established, released and modified and this clause shows how a session is established.

The procedures are depicted in the Figure 6.4.2.2.1-1.

Figure 6.4.2.2.1-1: The procedure of session setup

Editor's note:	Further details regarding IP address/prefix assignment is FFS.

Editor's note:	It is FFS whether a procedure with two SM functions and two User Plane Function(s) as in the home routed roaming case applies also in the non-roaming case.

It is assumed for this procedure that the UE is attached to the network. As part of the attach procedure, subscription data is fetched from the Subscriber Data Management function.

1.	The UE initiates a session setup request. The UE provides information indicating the PDU session type (e.g. Non-IP/IP). Indication of the DN network name may also be included. This step may be combined with the attach procedure in case the session is established with the attach procedure (EPS-like session establishment). It is assumed that this message is carried via an access/MM function and that this includes verification that the UE is attached and has an MM context.

Editor's note: It is FFS whether the DNN is identical to the APN.

2.	The SM function fetches user data. Get related information from the UE context and may verify that the UE subscribed the service. If the authorization done as part of step 1, then this step 2 is not needed.

3.	The SM function may interact with the Policy control function which determines the QoS property of the connection based on information such as UE request, and operator policy.

Editor's note:	how the QoS related property is set per operator policy is per the solutions developed for the QoS and Policy Framework Key Issues.

4.	The SM function selects the proper user plane function and, in case of connection type is IP, assigns IPv4 address/IPv6 prefix anchored in the User plane function.

5.	The SM function may request the AN to setup resources for the session.

6.	The SM function triggers the establishment of user-plane. The message includes the related QoS requirement for the connection to be established. Downlink tunnel information is also sent to the User plane functions (e.g. IP address of the AN node).

7.	Session setup is complete. Uplink tunnel information is sent to the AN function (e.g. IP address of the User plane functions). The UE the session is succeed, with IPv4 address/IPv6 prefix in case of session type is IP.

IPv6 stateless address auto-configuration or DHCPv4 may be used for IP address/prefix allocation after PDU session established. In this case, the DHCPv4 or RS/RA messages will be forwarded by the UP function between UE and the SM function.

Editor's note:	In the case of Non-3GPP AN case, step 5 may not be needed. This is FFS.