SA WG2 Meeting #116	S2-164254

11 – 15 July 2016, Vienna, Austria	(was S2-164108)

Source:	Nokia, Alcatel-Shanghai Bell, CATT

Title:	Relationship between Mobility Management and Session Management

Document for:	Approval

Agenda Item:	6.10.4

Work Item / Release:	NextGen / Rel14

Abstract of the contribution: New solution focusing on the Relationship between MM and SM (especially in the context of slicing)

# 1	Discussion

This document addresses  SM_WT_#2 within the Key Issue 4 (Session Management) and parts of key issue 1 on network slicing.

Table 5.4.2-1: Work tasks for Key Issue #4

| Work Task ID | Work Task(s) | Work Task Description |
| --- | --- | --- |
| SM_WT_#1 | SM model | High-level functions definition and allocation (forwarding, address allocation, UP selection…) |
|  |  | - PDU session type: support IP and non-IP connection - Identifier: for DN and PDU session (e.g., reconsider whether to use APN or not) |
|  |  | Basic procedures: on-demand SM setup, roaming/non-roaming support; Session maintenance (release, deactivation…) and related UE state change; UP function selection |
|  |  | UP protocol or model, e.g., - Identify UP functionalities needed to provide IP and non-IP PDU session (e.g. IP anchor, tunnelling, etc.) - Whether to simplify/remove tunnel if no mobility/session continuity support; |
|  |  | Support multi-PDU sessions to the same DN and different DN |
| SM_WT_#2 | Relation between SM and MM | SM and MM interactions, if any. (Note: refer to the KI# 4 for detailed description) |

Some text is inherited from 3417

# 2	Proposal

It is proposed to modify TR 23.799 as follows… (all text is NEW)

### 6.4.x	Solution: Relationship between CCP(MM) and SM

##### 6.4.x.1	Overview

This solution deals with the features that belong to what 3GPP has so far (e.g. in 24.301) called Mobility Management (MM) and Session Management (SM) . It focuses on the impacts of network slicing (especially for UE that would be served by multiple slices) on the relationship between MM and SM

NOTE1: 	This solution considers only per UE features e.g. features like PWS are not considered.

Following architecture is assumed

Editor’s Note:	The figure below will need to be refined per the agreements reached on other Key Issues.

Figure 6.4.x: CCP split from SM

This solution studies an architecture where SM would be split up from a Common Control Plane function (CCP) that supports the MM functionalities together with a NG1(NAS) + NG2 Front End.

- In case the UE is served by multiple slices the CCP belongs to the Common Core Functions (“group B” architecture),

NOTE 2: MM manages the UE as a whole so can hardly be sliced. For example the reachability or the location of an UE is an attribute of the UE, not of a service of the UE.

- No further split between entities within CCP  is considered.

- In case of an UE served by multiple slices there are multiple instances of SM that serve the UE.

NOTE 3: Different SM entities (located in different slices) may manage different services of the UE without conflicting each others.

- The  connection context related with a PDU session in a NGUP (User Plane) is only controlled by one entity (the SM)

- The UE and the Core Network exchange SM signalling via the common NAS Font-End that is located in the CCP. There is an unique NAS Front-End per UE handling both MM and SM related signalling. The NAS Font-End in CCP terminates NAS security and acts as a relay between the UE and the different SM instances

-	The CCP is not expected to maintain session context or be aware of the content of SM NAS message except for NAS SM message routing. SM NAS message are processed by SM. Accordingly Session contexts should only be stored at SM.

-	NAS MM messages and NAS SM messages and the corresponding procedures should be decoupled, so that the NAS Front End inside CCP can easily know if one NAS message should be routed to SM, or locally processed in the CCP.

- NG2 signalling related with UE is terminated in the CCP i.e. there is an unique NG2 termination for a given UE regardless of the number of  PDU sessions (possibly zero) of this UE. To ensure a better decoupling of CCP(MM) and SM, Mobility Management and Session Management related information shall be decoupled within NG2 signalling

NOTE 4: This

-	 ensures that the RAN needs only to consider one single termination in the Core for the NAS signalling that it relays and this independently of the internal structure of the Core (independently of the Core slicing)

-	simplifies the procedures related with NAS security (avoiding that each slice has to manage NAS security and the necessary re-Authentication that may be needed to refresh the NAS security keys)

##### 6.4.x.2	Potential interactions between CCP (MM) and SM

Some Mobility related events or state transitions have an impact on the PDU sessions and may require action on the NGUP (the User Plane entities in the Core) i.e. on NG4. Likewise some Session related actions are required towards the AN (on NG2)

- At the set-up / modification of a PDU session: the SM needs to interact with CCP in order to send  to the AN  Data handling control information (e.g. indication of where to send UL traffic from the UE, rules related with the QoS associated with data flows) and needs to receive from the AN DL Data forwarding address information  (User Plane address of the AN) in order to control DL data forwarding  in the NGUP(s). This corresponds to NG2 impacts of SM events.

- At the release of a PDU session: the SM needs to interact with CCP in order to remove the corresponding  Data handling policies in the AN. This corresponds to NG2 impacts of SM events

- At the transition from IDLE to CONNECTED: the SM needs to interact with CCP  in order to send  to the RAN  Data handling control information (e.g. indication of where to send UL traffic from the UE, rules related with the QoS associated with data flows) and needs to receive from the AN DL Data forwarding address information  (User Plane address of the AN) in order to control DL data forwarding  in the NGUP(s). This corresponds to the handling of a MM event requiring the exchange of session related  information on NG2

- During a Hand-Over : the SM needs to interact with CCP in order to receive from the RAN the DL Data forwarding address information  (User Plane address of the RAN) from the RAN in order to control DL data forwarding  in the NGUP(s). This corresponds to the handling of a MM event requiring the exchange of session related  information on NG4

Note:	The interaction information between CCP(MM) and SM in Handover procedure depends on the detail handover solution discussed in mobility management key issue.

- When the UE becomes IDLE, CCP notifies the SM in order for SM to modify the settings for DL data forwarding in the NGUP(s). This corresponds to the handling of a MM event requiring the exchange of session related  information on NG4

- When it has been made aware by the NGUP that some DL data has arrived for an UE in IDLE mode (and the UE is not known to be in a power saving state), the SM has to be able to interact with CCP in order to trigger UE paging from the RAN. This corresponds to MM impacts (including NG2 impacts) of SM events

- the SM may need to interact with CCP in order to be able to control the NGUP(s) based on the power saving state of the UE

Editor’s Note: The impacts of the various SSC (session and service continuity) modes to the split between CCP and SM are FFS

Management of the relationship between CCP and SM

Some functionalities are needed in the CCP to deal with the multiple SM that serve the UE: These functionalities are aware of the SM that serve the UE and Route NAS SM signaling to SM

- CCP store the identification of serving SM function(s) of UE

-	The relationship between CCP and a SM entity is created at the first PDU session for the UE that is handled by a SM instance and released when the last PDU session for the UE is deactivated in SM.

- SM stores the identification of serving CCP (MM) function of UE.

Further considerations

- The SM needs to receive an indication of the permanent user identity of the UE having sent a NAS SM message. This is needed for Policy, charging and LI interactions.

- When SM needs to send NAS SM signalling to an UE, it provides the permanent user identity of this UE, in order for the NAS Front-End to retrieve the corresponding NAS signalling security context.

##### 6.4.x.4	Subscription data

It is needed to Control whether NAS SM requests from the UE comply with the user subscription. SM enforces such control.

Editor’s Note: Potential CCP-SM interactions related with subscription data are FFS.

#### 6.4.x.2	Function description

Editor's note:	This clause will contain function descriptions and the interactions among the network functions.

#### 6.4.x.3	Solution evaluation

Editor's note:	This clause will contain evaluation on the system impacts, e.g. UE, access network and non-access network.

* * * * Next Change * * * *