SA WG2 Meeting #118-BIS	S2-170076

16 - 20 Jan 2017 Spokane, WA, USA	(revision of S2-17xxxx)

Source:	LG Electronics

Title:	Consideration on determination of roaming mode (TS 23.501 & TS 23.502)

Document for:	Approval

Agenda Item:	6.5.3

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This paper considers the determination of roaming mode in session management.

Discussion

The following is a part of agreements about session management in TR 23.799.

-	In roaming cases, for a new PDU session the AMF selects a SMF in the local PLMN and a SMF in the HPLMN. The AMF provides the identity of the selected SMF in the HPLMN to the selected SMF in the VPLMN together with an indication of whether LBO is allowed for this PDU session. Even if LBO is allowed for a PDU session, the SMF in VPLMN may, when handling a PDU session creation, request decide to apply the HR mode to the PDU session. This may apply e.g. when it detects it is not able to understand information in the UE request that it may need in order to handle the creation of the PDU session.

This paper revisits the case that SMF in VPLMN decide to apply the HR mode to the PDU session.

There are some options as the next step after the SMF in VPLMN decides to apply the HR mode to the PDU session.

Option 1. The SMF in VPLMN directly selects the SMF in HPLMN.

- It is not aligned in the agreement that the AMF selects a SMF in the local PLMN and a SMF in the HPLMN

Option 2. The AMF provides the identity of the selected SMF in the HPLMN in advance, even if in LBO case.

- It is not efficient.

Option 3. The SMF asks the AMF to decide the SMF in HPLMN with the proper cause.

We prefer to agree option 3 among above options.

Moreover, we propose only AMF has the functionality to decide the HR mode or LBO. If the AMF provide the indication that LBO is allowed for this PDU session, but the SMF in VPLMN is not able to understand information from the UE, i.e. the SMF in VPLMN cannot handle LBO, the SMF in VPLMN returns the proper error message (e.g. Not handle LBO) to the AMF. Then, the AMF can decide to transfer HR mode. If AMF decides to apply HR mode, the identity of the selected SMF in the HPLMN will be provided to SMF in VPLMN.

Proposal 1: It is proposed that only AMF has the functionality to decide the HR mode or LBO.

Proposal 2: the SMF in VPLMN may, when handling a PDU session creation request, return to the proper message to the AMF, in order to asking to determine the roaming mode again.

Proposal

It is proposed to add the following changes to TS 23.501 and TS 23.502.

If the following changes are agreed, it can be merged with SM related other papers.

* * * * Start of 1st Change * * * *

TS 23.501 (on top of Nokia’s paper)

5.3	Session Management

5.3.5	Roaming

………….

In roaming cases, for a new PDU session the AMF selects a SMF in the local PLMN and a SMF in the HPLMN. The AMF provides the identity of the selected SMF in the HPLMN to the selected SMF in the VPLMN together with an indication of whether LBO is allowed for this PDU session. Even if LBO is allowed for a PDU session, the SMF in VPLMN may, when handling a PDU session creation request, decide to ask the AMF to apply the HR mode to the PDU session. This may apply e.g. when it detects it is not able to understand information in the UE request that it may need in order to handle the creation of the PDU session.

* * * * Start of 2nd Change * * * *

TS 23.501 (on top of Nokia’s paper)

6.1.1	AMF Functional description

Editor’s notes: This should include various network functions in the architecture, features and functionalities supported.

The Access and Mobility Management function (AMF) includes the following functionality:

-	Termination of RAN CP interface (N2).

-	Termination of NAS (N1), NAS ciphering and integrity protection.

-	Mobility Management.

-	Lawful intercept (for AMF events and interface to LI System).

-	Transparent proxy for routing SM messages.

-	Access Authentication.

-	Access Authorization.

-	Security Anchor Function (SEA). It interacts with the AUSF and the UE, receives the intermediate key that was established as a result of the UE authentication process. In case of USIM based authentication, the AMF retrieves the security material from the AUSF.

-	Security Context Management (SCM). The SCM receives a key from the SEA that it uses to derive access-network specific keys.

NOTE:	Regardless of the number of Network functions, there is only one NAS interface instance per access network between the UE and the CN, terminated at one of the Network functions that implements at least NAS security and mobility management.

-	Determination of roaming mode (HR mode or LBO) for the PDU session.

* * * * Start of 3rd Change * * * *

TS 23.502 (on top of Ericsson’s paper)

4.2.1 	PDU Session Establishment

4.2.1.X 	Non-roaming and Roaming with Local Breakout

Fig X. UE-requested PDU Session Establishment

3. 	From AMF to SMF:  SM Request (Permanent User ID, DNN, SM-NSSAI, PDU Session ID, AMF ID, SM NAS Container, User location information, AT Type, Indication of whether LBO is allowed for this PDU session)

The AMF ID uniquely identifies the AMF serving the UE. The SM Container contains the Activate PDU Session Request received from the UE.

In roaming case, the AMF provide the indication of Indication of whether LBO is allowed for this PDU session. Even if LBO is allowed for a PDU session, the SMF in VPLMN may, when handling a PDU session creation request, decide to ask the AMF to apply the HR mode to the PDU session.

* * * * Start of 4th Change * * * *

TS 23.502 (on top of Ericsson’s paper)

4.2.1 	PDU Session Establishment

4.2.1.X 	Home-routed Roaming

3.	From AMF to SMF: SM Request (Permanent user identity, DNN, SM-NSSAI, PDU Session ID, AMF ID, H-SMF ID, SM NAS Container, User location information, AT Type, Indication of whether LBO is allowed for this PDU session)

The AMF ID uniquely identifies the AMF serving the UE. The H-SMF is provided when the PDU Session is home-routed. The SM Container contains the Activate PDU Session Request received from the UE.

In roaming case, the AMF provide the indication of Indication of whether LBO is allowed for this PDU session. In case of home routed roaming, H-SMF ID is provided together.

* * * * End of Changes * * * *