SA WG2 Meeting #118	S2-166718

14 – 18 November 2016, Reno, Nevada, USA	(revision of S2-16xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluation for Solution 6.5.5 and additional interim agreements for Key Issue #6

Document for:	Approval

Agenda Item:	6.10.6

Work Item / Release:	FS_NexGen/Rel-14

Abstract of the contribution: Propose evaluation for Solution 6.5.5 and additional interim agreements for Key Issue #6.

# Proposal

It is proposed to update TR 23.799 as follows.

* * * Start of 1st Change * * *

#### 6.5.5.3	Solution evaluation

Solution 6.5.5 provides a solution for mobile network operators to establish an efficient end-to-end user plane (UP) connection between UE and external application system (AS), where AS can be operated by the same mobile network operator or a third party service provider. The AS can provide different types of services, including content distribution network (CDN) for video streaming and online shopping, and IoT applications, etc. The UP path selection can be initiated by CP SM function for e.g. UP-GW load balancing or user mobility, taking the AS location and maybe other information received from the AS Controller into account.

-	The solution 6.5.5 allows the external AS controller to provide additional information to CP functions, e.g. application location, UP functions suitable or unsuitable for selection, SSC indicator, etc., to ensure efficient end-to-end UP path.

-	The solution 6.5.5 allows the CP to perform UP path (re)selection with taking the information provided by the external AS controller into account to ensure timely UP path (re)selection.

-	The solution 6.5.5 allows the CP to notify the external AS controller about UP path (re)selection to facilitate AS (re)location and ensure end-to-end UP path efficiency.

* * * End of 1st Change * * *

* * * Start of 2nd Change * * *

## 8.6	Interim Agreements on Key Issue #6: Support for session and service continuity and efficient user plane path

The following bullets are the current status of agreements on key issue 6:

1.	It shall be possible for the operator to provision the UE with SSC Mode Selection Policy (SSCMSP). This policy can be used by the UE to determine the type of SSC mode associated with an application or group of applications. When an application requests data transmission (e.g. opens a network socket) and the application itself does not specify the required SSC mode, the UE determines the SSC mode associated with this application by using the SSCMSP and:

a)	If the UE has already an active PDU session that matches the SSC mode associated with the application, then the UE routes the data of the application within this PDU session unless other conditions in the UE do not permit the use of this PDU session. Otherwise, the UE requests the establishment of a new PDU session with an SSC mode that matches the SSC mode associated with the application.

Interim agreement for Key Issue #6 Support for session and service continuity and efficient user plane path are as follows:

1.	The UP function selection and reselection is performed by the session management function:

2.	The UP Function is selected/reselected based on any combination of the following factors (non-exhaustive):

-	UE location;

-	UE subscription profile in SDN;

-	SSC mode selected for the PDU session ;

-	Data Network Name;

-	UP function load.

3.	The external application system (AS) can provide additional information to PDU sessions, including AS location change or potential AS locations , e.g. for the purpose of selection and reselection of the user plane path, as described in 6.5.5.

4.	The CP can inform the AS controller about the selection and reselection of the user plane path, e.g. for the purpose of AS location and relocation, respectively, as described in 6.5.5.

5.	Method to assign triggers and additional information to PDU sessions between CP functions and external application system in solution 6.5.5 will be defined during the normative phase.

* * * End of 2nd Change * * *