SA WG2 Meeting #118	S2-167156

14 – 18 November 2016, Reno, Nevada, USA	(revision of S2-167134)

Source:	Huawei, Hisilicon

Title:	Interim agreement for SM procedure triggered network slice selection

Document for:	Approval

Agenda Item:	6.10.1

Work Item / Release:	NextGen/Rel-14

Abstract of the contribution: This contribution proposes some additional agreements related to SM procedure triggered network slice selection.

- Discussion

- Proposal

It is proposed to add the agreements for KI_1 to the TR 23.799.

* * * Start of changes * * * *

8.1	Interim Agreements on Key Issue #1: Network slicing

The following bullets are the current status of agreements on the network slicing:

……

8.	The UE need to be able to associate an application with one out of multiple parallel established PDU sessions. Different PDU sessions may belong to different slices.

Editor's note:	it is FFS whether those are identified by DNN or SM-NSSAI+DNN. PDU session identification in a PLMN supporting network slicing may also require alignment with Key issue #4.

x. For roaming scenarios, the slice specific network functions beyond CCNF in VPLMN and HPLMN are selected based on the SM-NSSAI provided by the UE as following.

  - If standard SM-NSSAI is used, then selections of slice specific NF instances are done by each PLMN based on the standard SM-NSSAI.

  - If non-standard SM-NSSAI is used, the VPLMN shall map the SM-NSSAI of HPLMN to a SM-NSSAI of VPLMN based on roaming agreement. The selection of slice specific NF instance in VPLMN are done based on the SM-NSSAI of VPLMN, and the selection of any slice specific NF instance in HPLMN are based on the SM-NSSAI of HPLMN.

Note: whether the SM-NSSAI can be directly used as input to network function instance selection procedure defined in KI 7# should be decided in normative phase.

* * * End of Changes * * *