SA WG2 Meeting #S2-117	S2-167104

14 - 18 November 2016, Reno, Nevada	Revision of S2-166369

Source:	ZTE, Oracle, Telecom Italia, ETRI, KDDI, NEC, Sandvine, China Telecom, Openet

Title:	NextGen additional Policy Control Framework interim agreements

Document for:	Approval

# Agenda Item:	6.10.10

# Work Item / Release:	NextGen/Rel-14

Abstract of the contribution: Proposal for additional interim agreements for Policy Control Framework

# Background

This purpose of this PCR is to propose some additional interim agreements based on the adopted proposal on dual policy control functions in the TR 23.799.

# Proposal

The following changes are proposed to clause 8.10 of TR 23.799.

***** First Change *****

## 8.10	Interim Agreements on the Policy Framework

In the interim agreements below the PCC Framework refers to the Policy and Charging Control defined in EPC. The interim agreements below are subject to updates to cover all the topics listed in the WT list.

Interim agreements on the Policy Framework key issue:

- The PCC framework defined in TS 23.203 is generic and the relevant parts is applicable to NextGen, including:

  - The PCC Framework in NextGen includes the following Functional elements: The NG Policy Function that interfaces to the Session Management, the Application Function, and the Online Charging System.

  - The NG Policy Function evaluates operator policies that are triggered by events received from the Application Function, the Session Management, and the Online Charging System.

  - The NG Policy Function provides Rules for application and service data flow detection, gating, QoS and flow based charging to the Session Management.

NOTE 1: NextGen provides a PDU connectivity service of different PDU types, the existing PCC framework is applicable to PDU sessions of IP Type only.

NOTE 2:	Credit management and reporting are to be defined in SA5 specification.

- The PCC framework for the NextGen shall not preclude the study for dual-level policy control as described in 6.10.8 to provide flexible deployment of PCFs according to their respective realms or administrative domain (e.g. Network Slicing deployment as described in 6.10.8).  Further details on the support for dual-level policy control will be determined during the normative phase.

Editor’s note: additional interim agreements, e.g. for extensions to the existing PCC framework, are FFS.

***** End of Change *****