SA WG2 Meeting #118	S2-167190

14 - 18 November 2016, Reno, Nevada, USA	(revision of S2-167127)

Source:	LG Electronics

Title:	Interim agreements on multiple PDU sessions via different accesses

Document for:	Approval

Agenda Item:	6.10.4, 6.10.8

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: This paper propose to the interim agreements on multiple PDU sessions via different accesses based on the discussion during the session at SA2#118.

Introduction

It is related to SM WT#5 (PDU sessions via different accesses) and 6.8 .

| SM_WT_#5 | PDU sessions via different accesses | UE connected via multiple accesses, including non-3GPP Identify the architecture options for the following cases; - Multiple PDU sessions via different accesses - A single PDU session via different accesses (a.k.a. multi-access PDU session) (NOTE: the way to move IP flows between AN1 and AN2 depends on Key issue#20.) |
| --- | --- | --- |

Based on the discussion durign the session at SA2#118, adding the interim agreements.

Proposal

It is proposed to add the following changes to TR 23.799.

* * * * Start of 1st Change * * * *

8.4	Interim Agreements on Session management and Service Continuity (Key Issue #4, 5 and 6)

…….

Interim agreements for MM and SM interaction are as follows:

1.	A single NG1 NAS connection is used for both MM and SM-related messages and procedures for a UE. The single NG1 termination point is located in MM.

X.	When a UE is simultaneously connected to the same NGC network over 3GPP access and non-3GPP access, the UE is served by a single AMF.

NOTE x: The way to route over multiple accesses to the same AMF will be defined in the normative phase.

NOTE y: The scenario that a UE is roaming and connected over 3GPP access and non-3GPP access through N3IWF located in HPLMN will be considered during the normative phase.

* * * * End of Changes * * * *